3GPP TSG-SA WG6 Meeting #57	S6-232885

Xiamen, China 9th – 13th October 2023	(revision of S6-xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0493 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct description of Expiration time IE in EEC registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Unlike EAS and EES registration, when the EEC registration response does not have Expiration time IE, the deadline for registration is not specified. To make the registration deadline clear in cases when the EEC registration response does not have Expiration time IE, it is necessary to add descriptions like those used for EAS and EES registration when the IE is not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The descriptions to make the registration deadline permanent were added to EEC registration response when Expiration time IE is not included in the response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The registration deadline will not be defined when Expiration time IE is not included in the EEC registration response. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.3.3, 8.4.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.4.2.3.3	EEC registration response

Table 8.4.2.3.3-1 describes information elements in the EEC registration response from the EES to the EEC.

Table 8.4.2.3.3-1: EEC registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful. |
| > Registration ID | M | Identifier of the EEC registration. |
| > Expiration time (NOTE) | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| > EEC context ID | O | Identifier of the EEC Context information available at the EES that performed the registration. |
| > EEC Context Relocation status | O | Indicates whether the EEC context retrieval from the S-EES was successful or not. |
| > Discovered EAS list | O | List of EASs discovered to provide the capabilities required by the AC Profiles. If the request includes the EAS selection request indicator, then Discovered EAS list shall contain only one selected EAS. If the EES selects no EASs, the list may be empty. |
| >> EAS profile | M | Profile of the EAS. Each element is described in clause 8.2.4. |
| > list of unfulfilled AC information | O | List of ACIDs of the AC Profile(s) for which the requirements indicated in the AC profile(s) cannot be fulfilled |
| >> ACID | M | Application Identifier |
| >> reason | O | Reason indicating the cause (e.g. EAS not available, requirements cannot be fulfilled) |
| Failure response | O | Indicates that the registration request failed. |
| > Cause | M | Provides the cause for registration request failure. |
| NOTE: The Expiration time IE shall be included if based on the ECSP policy the EES requires EEC to periodically refresh the EEC registration. |  |  |

* * * Next Change * * * *

8.4.2.3.5	EEC registration update response

Table 8.4.2.3.5-1 describes information elements in the EEC registration update response from the EES to the EEC.

Table 8.4.2.3.5-1: EEC registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration update request was successful. |
| > Expiration time (NOTE) | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| > list of unfulfilled AC information | O | List of ACIDs of the AC Profile(s) for which the requirements indicated in the AC profile(s) cannot be fulfilled |
| >> ACID | M | Application Identifier |
| >> reason | O | Reason indicating the cause (e.g. EAS not available, requirements cannot be fulfilled) |
| Failure response | O | Indicates that the registration update request failed. |
| NOTE: The Expiration time IE shall be included if based on the ECSP policy the EES requires EEC to periodically refresh the EEC registration. |  |  |

* * * END of Change * * * *