3GPP TSG-SA WG6 Meeting #57	S6-232931

Xiamen, China 9th – 13th October 2023	(revision of S6-23xxxx)

Source:	Ericsson

Title:	Update KI#1

Spec:	3GPP TR 23.700-72 v0.1.0

Agenda item:	8.1

Document for:	Approval

Contact:	Wenliang Xu (wenliang.xu@ericsson.com)

1. Introduction

This pCR updates KI#1.

2. Reason for Change

The value-added service in TR 23.700-96 KI#6 includes time information related to an area:

-	Time information of the first entering and the last leaving an area (e.g. working campus)

-	The length of time to stay in an area

-	The times to re-enter and re-leave an area

It is proposed to update KI#1 with those open issues.

Also, if UE under Geofencing monitoring is power-off, the network is unable to track UE location but application doesn’t know UE status.

Dynamic Geofencing also needs attention.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-72.

* * * First Change * * * *

## 5.1	Key Issue #1: Support of Geofencing

Geofencing is a type of location-based marketing and advertising. A mobile app or software uses e.g. the Global Positioning System (GPS), radio frequency identification (RFID), Wi-Fi or cellular data to define a virtual geographical boundary and trigger a specific alert or push notification when a device enters or exits that boundary. This boundary is known as a geofence. Furthermore, Geofencing also has a widespread use in the industrial field.

In Rel-17 and Rel-18, the Location Management architecture and functional model in SEAL have introduced and supported the "Location report trigger with location area criteria", "Monitoring Location Deviation" and "Location area monitoring" functions and procedures related to Geofencing. However, they are still gaps to fulfil the requirements for Geofencing. For example, how to trigger the location alert or push notification to users in case a device enters or exits the boundary which has been established before.

In addition, the time information related to location area are missing in SEAL LM functions, e.g.:

-	Time information of the first entering and the last leaving an area.

-	The length of time to stay in an area.

-	Number of times to (re-)enter and/or (re-)leave an area in a given time period.

None of the existing LM function related to Geofencing supports to report UE status when the network is unable to determine UE location (e.g., UE power off, network connectivity issue). This exposure capability is essential for application to know in Geofencing.

Geofencing may be dynamic (in time and location) and it is needed to support more cases. E.g., student may get off school bus in wrong stop. A person may forget to bring wearable device or wallet when leaving home. Currently, SEAL LM does not support such dynamic Geofencing with target UE(s) under monitoring.

So, to support the Geofencing which is one of value-added location services, the following aspects need to be studied:

-	How to provision/update/revoke the Geofencing services.

-	How to trigger the location alert or push notification to VAL UEs/users.

-	How to support time information in Geofencing service.

-	How to provide more UE status in Geofencing service.

-	How to support dynamic Geofencing.

-	How to support the above identified functions based on the functions and procedures as defined in the SEAL-LM and/or other application enabler architecture.

* * * End of Changes * * * *