3GPP TSG-SA WG6 Meeting #57	S6-2322940

Xiamen, China 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0375 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding priority to MCPTT communication setup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In some MCPTT call setup procedures the requested priority is included within the call setup request information element. However, in other requests, the priority is not included. Therefore, this CR introduces the priority request in the missing call setup requests. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Including the priority information element in MCPTT communication setup where missing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Priority is not included in all MCPTT communication setup requests, which is not consistent among the MCPTT procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2.2.9, 10.6.2.2.27, 10.7.2.1.2a, 10.7.2.1.5a, 10.15.2.1, 10.15.2.2, and 10.19.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.6.2.2.9	Group call request (MCPTT server – MCPTT client)

Table 10.6.2.2.9-1 describes the information flow group call request from the MCPTT server to the MCPTT client.

Table 10.6.2.2.9-1 Group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ID | M | The identity of the MCPTT user towards which the request is sent |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is initiated |
| SDP offer | M | Media parameters of MCPTT server |
| Broadcast indicator | O | Indicates that the group call request is for a broadcast group call |
| Remotely initiated call request indicator | O | Indicates that the MCPTT group call request is a result of receiving of a remotely initiated call request and may be included only for remotely initiated call |
| Requested priority | O | Requested priority for the initiated group call |

# * * * * Next change * * * *

##### 10.6.2.2.27	Group-broadcast group call request (MCPTT server – MCPTT client)

Table 10.6.2.2.27-1 describes the information flow group-broadcast group call request from the MCPTT server to the MCPTT client.

Table 10.6.2.2.27-1 Group-broadcast group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ID | M | The identity of the MCPTT user towards which the request is sent |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is requested |
| SDP offer | M | Media parameters of MCPTT clients |
| Broadcast indicator | M | Indicates that the group call request is for a broadcast group call |
| Requested priority | O | The priority of the requested group call |

# * * * * Next change * * * *

##### 10.7.2.1.2a	MCPTT private call request (MCPTT server to MCPTT client)

Table 10.7.2.1.2a describes the information flow MCPTT private call request from the MCPTT server to the MCPTT client.

Table 10.7.2.1.2a: MCPTT private call request (MCPTT server to MCPTT client) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ID | M | The MCPTT ID of the called party |
| Functional alias | O | The functional alias of the called party |
| Use floor control indication | M | This element indicates whether floor control will be used for the private call. |
| SDP offer | M | Media parameters of MCPTT client. |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit floor request | O | An indication that the user is also requesting the floor. |
| Remotely initiated call request indicator | O | Indicates that the MCPTT private call request is a result of receiving of a remotely initiated call request and may be included only for remotely initiated call |
| Requested priority | O | The priority of the requested private call |

# * * * * Next change * * * *

##### 10.7.2.1.5a	MCPTT emergency private call request (MCPTT server to MCPTT client)

Table 10.7.2.1.5a-1 describes the information flow MCPTT emergency private call request from the MCPTT server to the MCPTT client.

Table 10.7.2.1.5a-1: MCPTT emergency private call request (MCPTT server to MCPTT client) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ID | M | The MCPTT ID of the called party |
| Emergency indicator | M | Indicates that the private call request is an MCPTT emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit floor request | O | An indication that the user is also requesting the floor. |
| SDP offer | M | Media parameters of MCPTT client. |
| Requested priority | O | The priority of the requested call. |

# * * * * Next change * * * *

#### 10.15.2.1	MCPTT first-to-answer call request (MCPTT client to MCPTT server)

Table 10.15.2.1-1 describes the information flow MCPTT first-to-answer call request from the MCPTT client to the MCPTT server.

Table 10.15.2.1-1: MCPTT first-to-answer call request (MCPTT client to MCPTT server) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ID list (see NOTE) | O | The list of MCPTT IDs of the called party |
| Functional alias (see NOTE) | O | The functional alias of the called party |
| Use floor control indication | M | This element indicates whether floor control will be used for the private call. |
| SDP offer | O | Media parameters of MCPTT client. |
| Implicit floor request | O | An indication that the user is also requesting the floor. |
| Location information | O | Location of the calling party |
| Requested priority | O | The priority of the requested call |
| NOTE: At least one identity must be present. |  |  |

# * * * * Next change * * * *

#### 10.15.2.2	MCPTT first-to-answer call request (MCPTT server to MCPTT client)

Table 10.15.2.2-1 describes the information flow MCPTT first-to-answer call request from the MCPTT server to the MCPTT client.

Table 10.15.2.2-1: MCPTT first-to-answer call request (MCPTT server to MCPTT client) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ID | M | The MCPTT ID of the called party |
| Functional alias | O | The functional alias of the called party |
| Use floor control indication | M | This element indicates whether floor control will be used for the private call. |
| SDP offer | M | Media parameters of MCPTT client. |
| Implicit floor request | O | An indication that the user is also requesting the floor. |
| Requested priority | O | The priority of the requested call |

# * * * * Next change * * * *

#### 10.19.2.4	Ad hoc group call request (MCPTT server – MCPTT client)

Table 10.19.2.4-1 describes the information flow ad hoc group call request from the MCPTT server to the MCPTT client.

Table 10.19.2.4-1: Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ID | M | The identity of the MCPTT user towards which the request is sent |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| SDP offer | M | Offered Media parameters of MCPTT server |
| Broadcast indicator (see NOTE) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE) | O | Indicates that the ad hoc group call request is an MCPTT imminent peril ad hoc group call |
| Emergency Indicator (see NOTE) | O | Indicates that the ad hoc group call request is an MCPTT emergency ad hoc group call |
| Preconfigured MCPTT group ID | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| Requested priority | O | The priority of the requested call |
| NOTE: If used, only one of these information elements is present. |  |  |

# * * * * End of changes * * * *