3GPP TSG-SA WG6 Meeting #57	S6-232956

Xiamen, China 9th – 13rd October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0019 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Align the transmission quality analytics with ADAE server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2023-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 9.4.3.2.3, the ADAE server are assumed to provide the performance analytics of load and transmission quality information. The related paper S6-23xxxx in this meeting, are proposed to extend the edge performance analytics (e.g. transmission quality) in TS 23.436. Thefore, the related NOTE in clause 9.4.3.2.3 can be removed, by adding the corresponding refered clause of TS 23.436. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to align the tranmission quality analytics provided by ADAE server, as specified in clause 8.8 of TS 23.436. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The assumption about transmission quality analytics provided by ADAE server exists, and does not be solved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.3.2.3, 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.4.3.2.3	VAL server and SEALDD server registered to EES

Figure 9.4.3.2.3-1: VAL server and SEALDD server registered to EES

1. The VAL server (acting as an EAS) registers to the EES.

2. The SEALDD server (acting as an EAS) registers to the EES.

3. The EEC performs service provisioning procedure as described in clause 8.3.3.2.2 of 3GPP TS 23.558 [10]. The EEC is responded with the EES which supports the SEALDD server and the VAL server. The EEC sends the EAS discovery request for EAS bundle including VAL service and SEALDD service. The EES may collect the performance of VAL server and SEALDD server (e.g. E2E latency between the SEALDD server and client and load information of SEALDD and VAL server) from the ADAE server when receiving the EAS discovery request, as specified in clause 8.8.2 of 3GPP TS 23.436 [x].

4. The EES identifies the VAL server and the associated SEALDD server based on the performance of VAL server and the performance of SEALDD server (e.g. to satisfy the AC service KPI).

5. The EES sends the EAS discovery response to EEC, including the SEALDD server address, the VAL server address.

Upon receiving the SEALDD server address, the VAL server address, the VAL client can initiate SEALDD service via SEALDD client with the SEALDD server's address. The data transmission between the VAL client and the VAL server can be enabled via the SEALDD connection, as specified in clause 9.2.2.

9.4.4	Information flows

The information flow about the SEALDD server discovery for VAL server/SEALDD server can reuse the defined information flow in CAPIF (e.g. service API publish in clause 8.3.2, Service API discover in clause 8.7.2), as specified in 3GPP TS 23.222 [3].

For SEALDD server discovery for VAL client in EDN scenario, the information flows for EAS registration and EAS discovery in caluse 8.4.3.3 and clause 8.5.3 of 3GPP TS 23.558 [10].

The information flow about regular SEALDD data transmission subscription can refer to the SEALDD enabled regular transmission request/response in clauses 9.2.3.1 and 9.2.3.2.

9.4.5	APIs

The APIs about SEALDD server discovery for VAL server/SEALDD server can reuse the defined APIs in CAPIF (e.g. CAPIF_Publish_Service_API in clause 10.3, CAPIF_Discover_Service _API in clause 10.2), as specified in 3GPP TS 23.222 [3].

The APIs about SEALDD server discovery for VAL client in EDN scenario, can reuse the defined APIs in EDGEAPP (e.g. Eees_EASRegistration API in clause 8.4.3.4, Eees_EASDiscovery API in clause 8.5.4), as specified in 3GPP TS 23.558 [10].

The APIs about regular SEALDD data transmission subscription can refer to the Sdd_RegularTransmission API, as defined in clause 9.2.4.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[7]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[8]	3GPP TS 23.548: "5G System Enhancements for Edge Computing ".

[9]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2".

[10]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[11]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[12]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[x]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".