3GPP TSG-SA WG6 Meeting #57	S6-232957

Xiamen, China 9th – 13rd October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0004 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extend the edge performance analytics to support transmission quality analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2023-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As required by the discovery of SEALDD server and VAL server in clause 9.4.3.2.3 of TS 23.433, the EES can identify the VAL server and the associated SEALDD server based on the performance of SEALDD server (e.g. E2E transmission quality between SEALDD server and SEALDD client) and the performance of VAL server (e.g. VAL server load). These performances are assumed to collect from the ADAE server. However, in clause 8.8 of TS 23.436, ADAE server can provide the analytic of edge performance, mainly including edge load analytics. Therefore, how to provide the tranmission quality analytics needs to be specified. In clause 9.7 of TS 23.433, SEALDD server or SEALDD client can generate the transmission quality report between SEALDD server and SEALDD client by encapsulating the DL/UL monitoring packet. Therefore, SEALDD server and/or SEALDD client can act as the data producers, providing the measured transmission quality results to the ADAE server, which can be used for transmission quality analytics by ADAE server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to extend the edge performance analytic in clause 8.8 of TS 23.436, to support the transmission quality analytics between SEALDD server and SEALDD client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current edge performance analytics mainly includes the edge load, how to provide the transmision quality analytics is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.8	Procedure for supporting edge performance analytics

8.8.1	General

This clause describes two procedures (covering both subscribe-notify and request-response models in 8.8.2.1 and 8.8.2.2 respectively) for supporting edge performanceanalytics (e.g. load, transmission quality). where the edge analytics are performed based on data collected from the EDN (EAS and/or EES, SEALDD server or EAS type for SEALDD server) and A-ADRF.

8.8.2	Procedure

8.8.2.1	Subscribe-notify model

Figure 8.8.2.1-1 illustrates the procedure for edge performance analytics enablement solution.

Pre-conditions:

1.	ADAES has discovered the APIs to access the edge services at EDN.

2.	ADAES has subscribed to OAM and NWDAF for receiving management and DN performance analytics respectively.

3.	Data producers (e.g. A-ADRF, EAS, EES, SEALDD server) may be pre-configured with data producer profiles (as in Table 8.2.4.8-1) for the data they can provide. ADAES and ADAEC have discovered available data producers and their data producer profiles.

Figure 8.8.2.1-1: ADAES support for edge analytics

1.	The consumer of the ADAES analytics service sends an edge analytics subscription request to ADAES.

2.	The ADAES sends an edge analytics subscription response as an ACK to the analytics consumer.

3.	The ADAES maps the analytics event ID to a list of data collection event identifiers, and a list of data producer IDs. Such mapping may be preconfigured by OAM or may be determined by ADAES based on the analytics event ID and/or data producer profile (Table 8.2.4.8-1). Such Data Producers can be EASs onboarded to EDN, EESs, SEALDD server, A-ADRF, as well as MEC Platform services.

4.	The ADAES sends a subscription request to the Data Producers (EASs onboarded to EDN, EESs, A-ADRF, ADAEC) or the A-DCCF with the respective Data Collection Event ID and the requirement for data collection.

5.	The Data Producers (e.g., EASs onboarded to EDN, EESs, SEALDD server, A-ADRF, ADAEC) or the A-DCCF send a subscription response as a positive or negative acknowledgement to the ADAES.

6.	The ADAES based on subscription receive offline stats/data on the edge DN load or the transmission quality result measured by SEALDD server based on the analytics/data collection event ID from A-ADRF. Such stats can be about the load in terms of number of EAS or EES connections for a given area or time window, or the average edge computational resource usage or usage ratio based on the EDN total resource availability, EDN overload/high load indication events, probability of EAS/EES unavailability due to high load, etc.

7.	The Data Producers at the edge start collecting data from the data generating entities. Such data can be measurements or analytics based on the data source/producer, as follows:

-	from OAM or EAS/ASP (for EAS load info): Per EAS/EES computational resource load, number of connections per EES/EAS

-	from N6 endpoint: N6 load

-	from 5GC / NWDAF: DN performance analytics

-	from OAM / MDAS: UPF load analytics (per DNAI)

-	from MEC platform services (e.g., RNIS): per cell radio conditions / load for all cells within EDN coverage

-	from SEALDD server: data transmission quality result

NOTE 1: How the ADAES obtains the EAS load information from EAS/ASP is up to implementation.

NOTE 2: Steps 6 and 7 are not necessarily sequential and can be performed in parallel or in different order.

8.	If in step 4 ADAES sent a subscription request to ADAEC as Data Producer, data collection is initiated by ADAEC from UE data generating entities.

NOTE 3:	Data collection at the UE reuses the SA4 mechanism based on EVEX study (TS 26.531 [3]).

9.	The edge Data Producers (targets of the subscription requests in step 4) send the data to the ADAES (based on step 7 measurements or analytics) as a data notification message. Such data can be about the load in terms of number of EAS or EES connections for a given area or time window, or the average edge computational resource usage or usage ratio based on the EDN total resource availability, EDN overload/high load indication events, probability of EAS/EES unavailability due to high load, or the data transmission quality result measured by SEALDD server, etc.

10.	 ADAEC sends data (periodically or if a threshold is reached based on configuration) about the edge load or the transmission quality result measured by SEALDD client as collected at the UE, e.g. in terms of number of AC or EEC connections for a given UE in a given time window, number of edge service sessions, etc.

11.	The ADAES derives edge analytics on EDN / DNAI load or per EES/EAS load, or transmission quality of SEALDD server, based on the analytics ID and type of request. The analytics are derived based on the performance analytics received per DN or load analytics per DNAI/UPF; as well as considering measurements on the computational or RAN resource load or number of connections for the EES/EASs which are active at the EDN. For the transmission quality analytics or prediction, the analytics are derived based on the measured transmission quality result in SEALDD server or SEALDD client.

12.	The ADAES sends the edge analytics to the consumer, based on the request and the derived analytics in step 9. Such analytics indicate a prediction of the EDN load considering inputs from both 5GS as well as from edge platform services. Such prediction can also be in form of a recommendation for triggering an EAS relocation to a different platform. Such analytics also indicate a prediction of the transmission quality between SEALDD server and SEALDD client.

8.8.2.2	Request-response model

Figure 8.8.2.2-1 illustrates the procedure for the analytics consumer to request analytics data of the application server(s) from the ADAE server.

Figure 8.8.2.2-1: ADAES support for edge analytics

1.	The analytics consumer sends a request message to the ADAE server to receive analytics data for one or more application servers. The request message includes the identity of the analytics consumer, security credential(s) for authorization and verification, identity of all the application server for which analytics data is requested, type of analytics data, time duration since when analytics data is required.

2.	Upon receiving the request, the ADAE server authenticates and authorizes the analytics consumer. If the analytics consumer is authorized, the ADAE server may get the analytics data by performing step 3 to 9 of clause 8.8.2.1. The ADAE server sends a response message including the statistical and predictive analytics data of the application servers for the requested duration period (if the time duration is available).

8.8.3	Information flows

8.8.3.1	General

The following information flows are specified for edge load analytics based on 8.8.2.

8.8.3.2	Edge analytics subscription request

Table 8.8.3.2-1 describes information elements for the edge analytics subscription request from the VAL server / Consumer to the ADAE server.

Table 8.8.3.2-1: Edge analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS,..) |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “edge performance analytics”. |
| Analytics filter information | M | Filter information for the analytics event |
| Analytics type | M | Whether analytics event is about prediction or statistics |
| Destination EAS ID | O (NOTE) | This identifier shows the destination EAS for which the analytics subscription applies. |
| Destination EES ID | O (NOTE) | This identifier shows the destination EES for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction. |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the request. |
| NOTE: At least one of these shall be present |  |  |

8.8.3.3	Edge analytics subscription response

Table 8.8.3.3-1 describes information elements for the edge analytics subscription response from the ADAE server to the consumer.

Table 8.8.3.3-1: Edge analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement). |

8.8.3.4	Edge data collection subscription request

Table 8.8.3.4-1 describes information elements for the edge data collection subscription request from the ADAE server to the Data Producer at the EDN or the A-DCCF, or from ADAE server to ADAE client.

Table 8.8.3.4-1: Data collection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server |
| Data Collection Event ID | M | The identifier of the data collection event |
| Data Collection requirements | M | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Analytics ID | O | The identifier of the analytics event, for which the data collection is needed. |
| List of Data Producer IDs | O | In case when this request is performed via A-DCCF, then the list of Data Producer IDs is needed. |
| Destination EAS ID | O (NOTE) | This identifier shows the destination EAS for which the analytics subscription applies. |
| Destination EES ID | O (NOTE) | This identifier shows the target EES for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Area of Interest | O | The geographical or service area for which the requirement request applies. |
| Time validity | O | The time validity of the request. |
| NOTE: At least one of these shall be present. |  |  |

8.8.3.5	Edge data collection subscription response

Table 8.8.3.5-1 describes information elements for the Data collection subscription response from the edge Data Producer at the EDN or the A-DCCF to the ADAE server, or from ADAE server to ADAE client.

Table 8.8.3.5-1: Data collection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the edge data collection subscription request (positive or negative acknowledgement). |

8.8.3.6	Data Notification

Table 8.8.3.6-1 describes information elements for the Data Notification from the Data Producer to the ADAE server.

Table 8.8.3.6-1: Data notification

| Information element | Status | Description |
| --- | --- | --- |
| Data Collection Event ID | M | The identifier of the data collection event. |
| Data Producer ID | M | The identity of Data Producer. |
| Destination EAS ID | O (NOTE) | This identifier shows the destination EAS for which the analytics subscription applies. |
| Destination EES ID | O (NOTE) | This identifier shows the destination EES for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Analytics ID | O | The identifier of the analytics event. |
| Data Type | M | The type of reported data samples which can be network data, application data, edge data, or different granularities / abstraction of data (e.g. real time, non-real time). This also indicates whether data are offline (from A-ADRF or not). |
| Data Output | M | The reported data, which can be inform of measurements or offline/historical data on the requested parameter based on subscription. Such data can be per EDN or per DNAI or per EAS/EES load statistics and edge computational resource utilization stats for a given time and area of interest. |
| NOTE: At least one of these shall be present based on the data collection event. |  |  |

8.8.3.7	Edge analytics Notification

Table 8.8.3.7-1 describes information elements for the Edge analytics Notification from the ADAE server to the VAL server / Consumer.

Table 8.8.3.7-1: Edge analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| Analytics Type | O | The type of analytics based on the event, which can be offline or online analytics, ML-enabled analytics, statistics, or predictive analytics. |
| Analytics Output | M | The predictive or statistical parameter, which can be stats or prediction related to the edge performance/load for the edge platform or EES/EAS for a given area/time and based on the event type. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

8.8.3.8	Get analytics data request

Table 8.8.3.8-1 describes information elements for the Get analytics data request from the analytics consumer to the ADAE server.

Table 8.8.3.8-1: Get analytics data request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS, EES). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “edge performance analytics”. |
| Analytics filter information | M | Filter information for the analytics event |
| Analytics type | M | Whether analytics event is about prediction or statistics. |
| Destination EAS IDs | O (NOTE) | This identifier provides the list of destination EASs for which the analytics request applies. |
| Destination EES IDs | O (NOTE) | This identifier provides the list of destination EESs for which the analytics request applies. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time duration | O | Time duration since when analytics data is required. |
| NOTE: At least one of these shall be present |  |  |

8.8.3.9	Get analytics data response

Table 8.8.3.9-1 describes information elements for the edge analytics subscription response from the ADAE server to the consumer.

Table 8.8.3.9-1: Edge analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics data request (positive or negative acknowledgement). |
| Analytics ID | O | The identifier of the analytics event. |
| Analytics Type | O | The type of analytics - statistics, or predictive analytics. |
| Analytics Output | O | The predictive or statistical parameter, which can be stats or prediction related to the edge performance/load for the edge platform or EES/EAS for a given area/time and based on the event type, or the transmission quality between SEALDD server and SEALDD client. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |