3GPP TSG-SA WG6 Meeting #57	S6-232966

Xiamen, China 9th – 13th October 2023	(revision of S6-23xxxx)

Source:	Huawei, Hisilicon

Title:	Discussion on the preconfigured regroup procedures in 23.280 and 23.379

Agenda Item:	7.1

Contact:	Cuili Ge – gecuili@huawei.com

Abstract: This discussion paper provides the analysis of the preconfigured regroup procedures in TS 23.280 and TS 23.379. In order to keep the whole preconfigured regroup procedures only in 23.280, the differences are highlighted, and change proposals to each specification are provided.

# Part I – The summary of the clause mapping

The mapping table for the procedures in 23.280 and 23.379 about preconfigured regroup.

| No | TS 23.280 | TS 23.379 | Remarks |
| --- | --- | --- | --- |
| 1 | 10.15.3.1 General | 10.6.2.9.1 General 10.6.2.12.1 General | A CR to 23.379 to void the two corresponding clauses. A CR to 23.280 to modify the clause 10.15.1 is needed to make it applicable for all the services. |
| 2 | 10.15.3.2.1 Preconfigured Group Regroup formation in a single MC system | 10.6.2.9.2.1 Regroup formation using preconfigured group in single MCPTT system | A CR to 23.280 is required. A CR to 23.379 is required. |
| 3 | 10.15.3.2.2 Preconfigured User Regroup formation in a single MC system | 10.6.2.12.2.1 User regroup formation in a single MCPTT system | A CR to 23.280 is required. A CR to 23.379 is required. |
| 4 | 10.15.3.2.3 Preconfigured regroup cancellation in a single MC system | 10.6.2.9.2.2 Regroup cancellation in single MCPTT system 10.6.2.12.2.2 User regroup cancellation in single MCPTT system | In 23.280, a single procedure covers both the group regroup and user regroup in 23.379. A CR to 23.379 is needed. |
| 5 | 10.15.3.3.1 Preconfigured Group Regroup formation in multiple MC systems | 10.6.2.9.3.1 Regroup formation using preconfigured group in multiple MCPTT systems | A CR to 23.280 is required. A CR to 23.379 is required. |
| 6 | 10.15.3.3.2 Preconfigured User Regroup formation in multiple MC systems |  | There is no user Regroup formation in multiple MC systems in 23.379. But the MCPTT also needs this feature. Changes to 23.280 clause 10.15.1 can be covered by the CR in No.1 |
| 7 | 10.15.3.3.3 Preconfigured regroup cancellation in multiple MC systems | 10.6.2.9.3.2 Regroup cancellation using preconfigured group in multiple MCPTT systems | There is no user regroup cancellation in multiple MC systems in 23.379. But the MCPTT also needs this feature. A CR to 23.379 is required. Changes to 23.280 clause 10.15.1 can be covered by the CR in No.1 |
| 8 | 10.15.3.3.4 Preconfigured group regroup rejection in multiple MC systems |  | There is no user regroup rejection in multiple MC systems in 23.379. But the MCPTT also needs this feature. Changes to 23.280 clause 10.15.1 can be covered by the CR in No.1 |
| 9 | 10.15.3.4.1 Adding newly affiliated user to preconfigured group regroup | 10.6.2.9.4 Adding newly affiliated user to preconfigured group regroup | A CR to 23.379 is required. |
| 10 | 10.15.3.4.2 Updating the MC service ID list in a preconfigured regroup group based on user regroup | 10.6.2.12.3 User regroup MCPTT ID list update | A CR to 23.379 is required. |
| 11 |  | 10.6.2.9.5.1 MCPTT client PTTs on MCPTT group during an in-progress preconfigured group regroup 10.6.2.9.5.2 MCPTT client PTTs on preconfigured regroup group after group regroup has been cancelled | There is no such procedure in 23.280, 23.281 and 23.282. But the MCVideo also require such feature. A CR to 23.379 is required. A CR to 23.280 is required. A CR to 23.281 is required. |
| 12 | Information flow tables in 23.280 | Information flow tables in 23.379 | As indicated in Part III, A CR to 23.379 is required. A CR to 23.280 is required. |

# Part II – The detailed analysis and change proposals

The difference between the corresponding procedures in 23.280 and 23.379 are listed as below. This analysis uses the procedure description in TS 23.280 as baseline and the highlight part is the essential difference. The term changes e.g., MC service ID and MCPTT ID, MC service and MCPTT are not specially highlighted.

## 1. 10.15.3.1 of 23.280 vs 10.6.2.9.1 and 10.6.2.12.1of 23.379

| Difference | Analysis | CR to 23.280 | CR to 23.379 |
| --- | --- | --- | --- |
| 10.6.2.12.1 General A user regroup may be achieved by regrouping MCPTT users into a new regroup group which uses the configuration of a separate preconfigured MCPTT group. The MCPTT regroup group configuration needs to be provided to the relevant MCPTT users who will be regrouped in advance of the regrouping operation. User regroup with preconfigured group satisfies user regroup requirements in 3GPP TS 22.180 [17], particularly [R-6.6.4.2-003]. NOTE 1: A preconfigured group which is intended only to provide configuration for the preconfigured regroup process is identified by a parameter in the group configuration described in 3GPP TS 23.280 [16]. NOTE 2: The configuration may alternatively be taken from any MCPTT group that has been configured in the user profile of all of the relevant MCPTT users who will be regrouped. Editor's note: These procedures provide a regrouping service for MCPTT only; any issues arising from conflicts with similar regrouping operations for MCVideo and MCData are FFS. The preconfigured MCPTT group that provides the configuration is not used as the MCPTT regroup group itself, it only provides configuration for one or more MCPTT regroup groups. The MCPTT group ID of the regroup group is provided by the authorized user when the preconfigured regrouping is carried out. | For (1), this is due to in 23.280, 10.15.3.1 is the general clause for both the group regroup (cl.10.6.2.9.1) and user regroup (cl. 10.6.2.12.1). Keep this text in 23.280 as it is. For (2), in 23.280, such text from 23.379 is not required. For (3), in 23.280 such EN is not required. The last sentence in 10.6.2.12.1 of 23.379 is no longer required in 23.280. | No changes to the clause 10.15.3.1 | Void the 10.6.2.9.1 and 10.6.2.12.1 |

## 2. 10.15.3.2.1 of 23.280 vs 10.6.2.9.2.1 of 23.379

| Difference | Analysis | CR to 23.280 | CR to 23.379 |
| --- | --- | --- | --- |
|  | In 23.379, the preconfigured regroup is the MC service group in the 4th pre-condition | Add the “reconfigured regroup” to 4th pre-condition | Void 10.6.2.9.2.1 |
|  | A new step 4b was introduced in 23.379 | Update the figure to incorporate the 4b |  |
|  | The highlighted text from 23.379 is about the MCPTT ID is allocated by the MCPTT server. This is also applicable for MCVideo and MCdata. | Update the text in 23.280 to incorporate the yellow highlighted description. |  |

## 3. 10.15.3.2.2 of 23.280 vs 10.6.2.12.2.1 of 23.379

| Difference | Analysis | CR to 23.280 | CR to 23.379 |
| --- | --- | --- | --- |
| 10.6.2.12.2 .1 User regroup formation in a single MCPTT system Figure 10.6.2.12.2.1-1 illustrates the procedure to initiate a user regroup procedure using a preconfigured MCPTT group. The procedure takes place prior to the establishment of a group call to the MCPTT regroup group. Pre-conditions: - MCPTT clients 2 and 3 are registered with the MCPTT service. - An MCPTT group that will be used for configuration of the temporary user regroup group has been preconfigured in MCPTT clients 2 and 3, and MCPTT clients 2 and 3 have received the relevant security related information to allow them to communicate in the temporary user regroup group. - MCPTT client 1 is authorized to initiated a user regroup using the preconfigured regroup procedure. - MCPTT client 1 is aware of a suitable preconfigured group whose configuration has been preconfigured in the MCPTT UEs of the MCPTT users who will be regrouped. | In 23.379, the pre-conditions are more specific to user regroup | Modify the preconditions in 10.15.3.2.2 to incorporate the corresponding pre-conditions from 10.6.2.12.2.1 of 23.379 | Void 10.6.2.12.2.1 |
|  | A new step 4 was introduced in 23.379 | Update the figure to incorporate the 4 |  |
| 1. The authorized user of MCPTT client 1 initiates the user regroup procedure, specifying the list of MCPTT users to be regrouped (MCPTT clients 2 and 3), the MCPTT group ID of the regroup group (if available), and the MCPTT group ID of the group from which configuration information for the regroup group is to be taken. Alternatively, the MCPTT client 1 can provide the criteria which allows the MCPTT server to determine the list of MCPTT users to be regrouped. 2. MCPTT client 1 sends the preconfigured regroup request to the MCPTT server. The request indicates the list of users to be included in the regroup operation. 3. The MCPTT server checks that MCPTT client 1 is authorized to initiate a preconfigured regroup procedure. If the preconfigured regroup request is authorized, the MCPTT server assigns a MCPTT group ID for this regroup group call if: i. one is not provided in the request or ii. the one provided in the request is not accepted. NOTE 1: MCPTT clients can be involved in multiple user and group regroups simultaneously. 4. The MCPTT server shall send the preconfigured regroup request return message to MCPTT client 1 containing the below: i. the MCPTT group ID is generated by the MCPTT server; and ii. result of whether the ad hoc group call is authorized or not If the preconfigured regroup request is not authorized, MCPTT client 1 shall not proceed with the rest of the steps. 5. The MCPTT server sends the preconfigured regroup requests to MCPTT clients 2 and 3 in steps 4a and 4b respectively. NOTE 2: When using multicast, the MCPTT server can periodically rebroadcast the preconfigured regroup request. 6. MCPTT clients 2 and 3 notify their users of the regrouping in steps 5a and 5b respectively. 7. MCPTT clients 2 and 3 may send the preconfigured regroup response to the MCPTT server to acknowledge the regrouping action. These acknowledgements are not sent in response to a multicast transmission of the preconfigured regroup request. 8. The MCPTT server affiliates the regrouped MCPTT clients to the regroup group. 9. The MCPTT server sends a preconfigured regroup response to MCPTT client 1. NOTE 3: After the user regrouping procedure, the regrouping remains in effect until explicitly cancelled by the procedure in 10.6.2.12.2.2. | In step 2, the requester provides the list of users to be regrouped is also applicable for MCVideo and MCData. In step 3 and 4, the highlighted text from 23.379 is about the MCPTT ID is allocated by the MCPTT server. This is also applicable for MCVideo and MCData. | Update the text in 23.280 to incorporate the yellow highlighted description. |  |

## 4. 10.15.3.2.3 of 23.280 vs 10.6.2.9.2.2 and 10.6.2.12.2.2 of 23.379

| Difference | Analysis | CR to 23.280 | CR to 23.379 |
| --- | --- | --- | --- |
| 10.15.3.2.3 of 23.280 vs 10.6.2.9.2.2 10.6.2. 9.2.2 Regroup cancellation in single MCPTT system Figure 10.6.2.9.2.2-1 illustrates the procedure to cancel a regrouping that uses a preconfigured MCPTT group. Pre-conditions: - MCPTT clients 2 and 3 have been regrouped into an MCPTT regroup group. - MCPTT client 1 is authorized to cancel a regrouping that uses a preconfigured MCPTT group. - The GMS has subscribed to group dynamic data as specified in subclause 10.1.5.5.1 from the MCPTT server using the procedures defined in subclause 10.1.5.6 in TS 23.280 [16]. Figure 10.6.2.9.2.2-1: Cancel preconfigured regroup procedure in single MCPTT system 1. The authorized user of MCPTT client 1 initiates the cancellation of the regrouping that uses a preconfigured MCPTT group. 2. MCPTT client 1 sends the preconfigured regroup cancel request to the MCPTT server, specifying the MCPTT group ID of the regroup group. 3. The MCPTT server checks that MCPTT client 1 is authorized to cancel a regrouping that uses a preconfigured group regroup procedure. 4. The MCPTT server sends the preconfigured regroup cancel requests to MCPTT clients 2 and 3 (steps 4a and 4b respectively). 5. MCPTT clients 2 and 3 notify their users of the cancellation of the group regrouping in steps 5a and 5b respectively. 6. MCPTT clients 2 and 3 may send the preconfigured regroup cancel response to the MCPTT server to acknowledge the cancellation of the regrouping function. These acknowledgements are not sent in response to a multicast transmission of the preconfigured regroup cancel request. 7. The MCPTT server de-affiliates MCPTT clients 2 and 3 from the MCPTT regroup group. 8. The MCPTT server sends a preconfigured regroup cancel response to MCPTT client 1. | The 10.15.3.2.3 covers the cancellation for both the group regroup and user regroup, while the 10.6.3.9.2.2 of 23.379 is only for group regroup cancellation, 10.6.2.12.2.2 of 23.379 is only about user regroup cancellation | No need to change the text 10.15.3.2. | Void 10.6.2.9.2.2 and 10.6.2.12.2.2 of 23.379 |
| # 10.15.3.2.3 of 23.280 vs 10.6.2.12.2.2 of 23.379 10.6.2. 12.2.2 User regroup cancellation in single MCPTT system Figure 10.6.2.12.2.2-1 illustrates the procedure to cancel a user regrouping that uses a preconfigured MCPTT group. Pre-conditions: - MCPTT clients 2 and 3 have been regrouped into an MCPTT regroup group. - MCPTT client 1 is authorized to cancel a user regrouping that uses a preconfigured MCPTT group. Figure 10.6.2.12.2.2-1: Cancel preconfigured user regroup procedure in single MCPTT system 1. The authorized user of MCPTT client 1 initiates the cancellation of the user regrouping that uses a preconfigured MCPTT group. 2. MCPTT client 1 sends the preconfigured regroup cancel request to the MCPTT server, specifying the MCPTT group ID of the regroup group. 3. The MCPTT server checks that MCPTT client 1 is authorized to cancel a regrouping that uses a preconfigured user regroup procedure. 4. The MCPTT server sends the preconfigured regroup cancel requests to MCPTT clients 2 and 3 (steps 4a and 4b respectively). 5. MCPTT clients 2 and 3 notify their users of the cancellation of the user regrouping in steps 5a and 5b respectively. 6. MCPTT clients 2 and 3 may send the preconfigured regroup cancel response to the MCPTT server to acknowledge the cancellation of the regrouping function. These acknowledgements are not sent in response to a multicast transmission of the preconfigured regroup cancel request. 7. The MCPTT server de-affiliates MCPTT clients 2 and 3 from the MCPTT regroup group. 8. The MCPTT server sends a preconfigured regroup cancel response to MCPTT client 1. |  |  |  |

## 5. 10.15.3.3.1 of 23.280 vs 10.6.2.9.3.1 of 23.379

| Difference | Analysis | CR to 23.280 | CR to 23.379 |
| --- | --- | --- | --- |
| 10.6.2.9.3.1 Regroup formation using preconfigured group in multiple MCPTT systems Figure 10.6.2.9.3.1-1 illustrates the procedure to initiate a regroup procedure using a preconfigured MCPTT group, where at least one of the groups to be regrouped is configured in a partner MCPTT system. The primary MCPTT system where the preconfigured group regrouping is initiated does not need to be aware of the list of group members belonging to groups whose group home system is the partner MCPTT system. If the group management server in the primary MCPTT of the regroup group shares the necessary security related parameters together with the group configuration of the MCPTT regroup group with the group management server in the partner MCPTT system and the group management server in the partner MCPTT system distributes this configuration including those security parameters to its served MCPTT users according to the procedures in 3GPP TS 23.280 [16] subclause 10.2.7, the primary MCPTT system does not need to be aware of the list of group members of the preconfigured regroup group that are receiving service in the partner MCPTT system. The procedure takes place prior to the establishment of a group call to the regroup group. In this procedure, any gateway MC servers in the primary or partner MCPTT systems are not shown. Pre-conditions: - MCPTT client 1 is authorized to initiated a preconfigured regroup procedure, and is receiving MCPTT service in the primary MCPTT system of MCPTT client 1. - MCPTT client 2 is an affiliated member of MCPTT group 1 where MCPTT group 1 is defined in the partner MCPTT system and MCPTT client 2 is receiving service in the partner MCPTT system of MCPTT client 1. - The MCPTT group identity and group configuration for the regroup MCPTT group have been preconfigured in MCPTT client 2, and MCPTT client 2 has received the relevant security related information to allow communication in the regroup MCPTT group. - In order to be aware whether the group is regrouped, the MCPTT server is subscribed to the group configuration in GMS. - The GMS has subscribed group dynamic data as specified in subclause 10.1.5.5.1 from the MCPTT server within the same MCPTT system using the procedures defined in subclause 10.1.5.6 in TS 23.280 [16]. Figure 10.6.2.9.3.1-1: Regroup procedure using preconfigured group in multiple MCPTT systems 1. The authorized user of MCPTT client 1 initiates the regroup procedure, specifying the list of MCPTT groups to be regrouped including MCPTT group 1, the MCPTT group ID of the regroup group (if available) and the MCPTT group ID of the group from which configuration information for the regroup group is to be taken. 2. MCPTT client 1 sends the preconfigured regroup request to the MCPTT server. 3. The MCPTT server checks that MCPTT client 1 is authorized to initiate a preconfigured regroup procedure, and resolves the group identities of the MCPTT groups requested in step 1. The MCPTT server also checks which group members are affiliated to the requested MCPTT groups that are homed in the primary MCPTT system. The MCPTT server identifies any partner systems which are the group home systems for MCPTT groups identified in the list of groups to be regrouped. The MCPTT server may retrieve the configuration for the regroup group from the GMS if that configuration information is not already known to the MCPTT server. If the preconfigured regroup request is authorized, the MCPTT server assigns a MCPTT group ID for this regroup group call if: i. one is not provided in the request or ii. the one provided in the request is not accepted. NOTE 1: This procedure does not require that the authorized user of MCPTT client 1 is a group member of the MCPTT groups listed in the regroup request, or that the authorized user of MCPTT client 1 is an affiliated group member of any of the listed MCPTT groups. 4a. If the MCPTT server determines that any of the groups requested for regrouping, including the regroup group, have been regrouped by other group regrouping procedures, the MCPTT server then sends a preconfigured regroup reject back to MCPTT client 1 with a reject reason indicating that one of the groups has already been regrouped, and this procedure terminates. 4b. The MCPTT server shall send the preconfigured regroup request return message to MCPTT client 1 containing the below: i. the MCPTT group ID is generated by the MCPTT server; and ii. result of whether the ad hoc group call is authorized or not If the preconfigured regroup request is not authorized, MCPTT client 1 shall not proceed with the rest of the steps. 5. The MCPTT server sends the preconfigured regroup requests to the MCPTT server in the partner MCPTT system. 6. The partner MCPTT server checks the status of any MCPTT groups hosted by that partner MCPTT server, and identifies affiliated group members of any of the identified MCPTT groups (both MCPTT groups that are hosted in the primary MCPTT system and MCPTT groups that are hosted in the partner MCPTT system) that are receiving MCPTT service in the partner MCPTT system, which include MCPTT client 2. 7. The partner MCPTT server sends the preconfigured regroup request to MCPTT client 2. NOTE 2: Only group members that are affiliated to the MCPTT groups that are to be regrouped are sent a preconfigured regroup request. 8. MCPTT client 2 notifies the user of the regrouping. 9. MCPTT client 2 may send the preconfigured regroup response to the partner MCPTT server to acknowledge the regrouping action. This acknowledgement is not sent in response to a multicast transmission of the preconfigured regroup request. 10. The partner MCPTT server affiliates the regrouped MCPTT client 2 to the regroup group. 11. The MCPTT server sends a preconfigured regroup response to the primary MCPTT server. 12. The primary MCPTT server sends the preconfigured regroup response to MCPTT client 1. After the group regrouping procedure, the regrouping remains in effect until explicitly cancelled by the procedure in 10.6.2.9.3.2. MCPTT client participation in the ongoing regroup persists until the MCPTT client is no longer affiliated to any of the regrouped groups (group 1 or 2 in this procedure). MCPTT client affiliation to the regroup group may cease when the UE's MCPTT service ceases, e.g. when the UE is powered down, or by performing a log-off operation. Editor's note: Data persistence in the MCPTT client following a user log-off or power down needs further study. | In the beginning and precondition part, only the “preconfigured” is use differently; In the figure part, a new step 4a and 4b are in 23.379; Accordingly, in step 3 and 4b, the MCPTT ID is generated case is specially described. The last sentence in 23.280 has the same meaning in 4a in 23.379. | improve the general description and the pre-condition; Update the figure to add the new step 4a and 4b from 23.379; Incorporate the changes in 23.379 in step1, step 3 and step 4. | Void 10.6.2.9.3.1 |

## 6. 10.15.3.3.2 of 23.280

There is no user Regroup formation in multiple MC systems in 23.379. However MCPTT also needs this feature. Such feature can be also applicable for MCPTT by modifying the text in 23.280 to remove the restrictions.

“NOTE:	The information flows and procedures for group and user regrouping using preconfigured groups for MCPTT are specified in 3GPP TS 23.379 [16].”

## 7. 10.15.3.3.3 of 23.280 vs 10.6.2.9.3.2 of 23.379

There is no user regroup cancellation in multiple MC systems in 23.379. However MCPTT also needs this feature. Such feature can be also applicable for MCPTT by modifying the text in 23.280 to remove the restrictions.

“NOTE:	The information flows and procedures for group and user regrouping using preconfigured groups for MCPTT are specified in 3GPP TS 23.379 [16].”

| Difference | Analysis | CR to 23.280 | CR to 23.379 |
| --- | --- | --- | --- |
| 10.6.2.9.3. 2 Regroup cancellation using preconfigured group in multiple MCPTT systems Figure 10.6.2.9.3.2-1 illustrates the procedure to cancel a regrouping that uses a preconfigured MCPTT group where multiple MCPTT systems were involved in the regrouping. Pre-conditions: - MCPTT client 2 has been regrouped into an MCPTT regroup group, and is receiving MCPTT service in the partner MCPTT system of the regroup group. - MCPTT client 1 is authorized to cancel a regrouping that uses a preconfigured MCPTT group, and is receiving MCPTT service in the primary MCPTT system of the regroup group. - The GMS has subscribed to group dynamic data as specified in subclause 10.1.5.5.1 from the MCPTT server within the same MCPTT system using the procedures defined in subclause 10.1.5.6 in TS 23.280 [16]. Figure 10.6.2.9.3.2-1: Cancel preconfigured regroup procedure using preconfigured group in multiple MCPTT systems 1. The authorized user of MCPTT client 1 initiates the cancellation of the regrouping that uses a preconfigured MCPTT group. 2. MCPTT client 1 sends the preconfigured regroup cancel request to the MCPTT server, specifying the MCPTT group ID of the regroup group. 3. The MCPTT server checks that MCPTT client 1 is authorized to cancel a regrouping that uses a preconfigured group regroup procedure. 4. The primary MCPTT server sends the regroup cancel request to the partner MCPTT server. 5. The partner MCPTT server sends the preconfigured regroup cancel requests to MCPTT client 2. 6. MCPTT client 2 notifies the user of the cancellation of the group regrouping. 7. MCPTT client 2 may send the preconfigured regroup remove response to the partner MCPTT server to acknowledge the cancellation of the regrouping function. This acknowledgement is not sent in response to a multicast transmission of the preconfigured regroup cancel request. 8. The partner MCPTT server de-affiliates MCPTT client 2 from the MCPTT regroup group. 9. The partner MCPTT server sends the preconfigured regroup cancel response to the primary MCPTT server. 10. The primary MCPTT server sends a preconfigured regroup cancel response to MCPTT client 1. | The two procedures are aligned. and No need to change this clause in 23.280 |  | Void the clause 10.6.2.9.3.2 of 23.379 |

## 8. 10.15.3.3.4 of 23.280

There is no user regroup rejection in multiple MC systems in 23.379. However the MCPTT also needs this feature. Such feature can be also applicable for MCPTT by modifying the text in 23.280 to remove the restrictions,.

“NOTE:	The information flows and procedures for group and user regrouping using preconfigured groups for MCPTT are specified in 3GPP TS 23.379 [16].”

## 9. 10.15.3.4.1 of 23.280 vs 10.6.2.9.4 of 23.379

| Difference | Analysis | CR to 23.280 | CR to 23.379 |
| --- | --- | --- | --- |
| 10.6.2.9.4 Adding newly affiliated user to preconfigured group regroup Figure 10.6.2.9.4-1 illustrates the procedure to add a newly affiliated user to an in-progress preconfigured MCPTT group regroup operation. Pre-conditions: - The MCPTT client is a member of a MCPTT group that is part of an in-progress preconfigured group regroup operation. - The MCPTT group identity and group configuration for the regroup MCPTT group has been preconfigured in the MCPTT client, and the MCPTT client has received the relevant security related information to allow it to communicate in the regroup MCPTT group. Figure 10.6.2.9.4-1: Procedure to add a newly affiliated user to a preconfigured regroup 1. The MCPTT client affiliates to an MCPTT group that is currently part of an in-progress preconfigured group regroup. The affiliation follows the procedure in clause 10.8.3.1 of TS 23.280 [16]. 2. The MCPTT server retrieves the MCPTT group ID of the regroup group and the MCPTT group ID of the group from which configuration information for the regroup group is to be taken. 3. The MCPTT server sends the preconfigured regroup request to the MCPTT client. 4. The MCPTT client notifies the user of the regrouping. 5. The MCPTT client may send the preconfigured regroup response to the MCPTT server to acknowledge the regrouping action. These acknowledgements are not sent in response to a multicast transmission of the preconfigured regroup request. 6. The MCPTT server affiliates the regrouped MCPTT client to the regroup group. Editor's Note: It is FFS whether this procedure can be generalized to apply to both preconfigured and GMS-based group regrouping. Editor's Note: Affiliations of the MCPTT client to the pre-configured regroup group in the MCPTT server and coordination to the MC System is FFS. | The two procedures are aligned. and No need to change this clause in 23.280 |  | Void the clause 10.6.2.9.4 of 23.379 |

## 10. 10.15.3.4.2 of 23.280 vs 10.6.2.12.3 of 23.379

| Difference | Analysis | CR to 23.280 | CR to 23.379 |
| --- | --- | --- | --- |
| 10.6.2.12.3 User regroup MCPTT ID list update Figure 10.6.2.12.3-1 illustrates the procedure to add or remove an MCPTT user based on the MCPTT user criteria known at the MC service server. NOTE 1: This procedure applies to MCPTT users that have been added and are to be added to the preconfigured regroup group based on MCPTT user criteria known at the MCPTT server. If only the criteria are to be applied, then only the criteria are included in the request and both the group and user lists are not included Preconditions: - A preconfigured regroup group has been established. - The MCPTT server has selected MCPTT user 2 to participate in the preconfigured regroup group based on MCPTT user criteria known at the MCPTT server. NOTE 2: Other participants of the preconfigured regroup group are not shown in figure 10.6.2.12.x-1 for simplicity. Figure 10.6.2.12.3-1: Procedure to update the MCPTT ID list 1. The MCPTT server detects that MCPTT client 2, which has been earlier included in the preconfigured regroup group, no longer meets the MCPTT user criteria known at the MCPTT server. 2. The MCPTT server sends MCPTT client 2 a preconfigured user regroup remove request. 3. The MCPTT client 2 notifies the MCPTT user of the preconfigured user regroup removal. 4. The MCPTT client 2 may send the MCPTT server a preconfigured user regroup remove response to acknowledge the preconfigured user regroup removal. 5. The MCPTT server updates the preconfigured regroup group (including the MCPTT user ID list) and potential group communication. 6. The MCPTT server detects that the MCPTT client 3 meets the MCPTT user criteria. 7. The MCPTT server sends the MCPTT client 3 a preconfigured regroup request. 8. The MCPTT client 3 notifies the MCPTT user of the preconfigured regroup request. 9. The MCPTT client 3 may send the MCPTT server a preconfigured regroup response. 10. The MCPTT server updates the preconfigured regroup group (including the MCPTT ID list), and potential group communication. | The two procedures are aligned. |  | Void the clause 10.6.2.12.3 |

## 11. 10.6.2.9.5.1 and 10.6.2.9.5.2 of 23.379

10.6.2.9.5.1	MCPTT client PTTs on MCPTT group during an in-progress preconfigured group regroup is used to update the regroup group members (i.e., adding new member) during an MCPTT session.

This is also required for MCVideo and MCData. So a new CR to 23.280 is needed to move the regroup group steps of this procedure to a generic one to cover all the service. A CR to 23.379 to void this procedure is also needed to update the procedure accordingly.

10.6.2.9.5.2	MCPTT client PTTs on preconfigured regroup group after group regroup has been cancelled is used to update the regroup group members (i.e., remove member) during an MCPTT session.

This is also required for MCVideo and MCData. So a new CR to 23.280 is needed to to move the regroup group steps of this procedure to a generic one to cover all the service. A CR to 23.379 to void this procedure is also needed to update the procedure accordingly.

# Part III – The information flow

In TS 23.280, update table 10.15.2.1-1, 10.15.2.2-1, 10.15.2.3-1with the corresponding IE from 23.379 (as highlighted in the tables).

Introduce a new table in 23.280 about the new Preconfigured regroup request return message.

So a CR is proposed to update the 23.280.

The comparison of the corresponding information flows from TS 23.380 and 23.379.

10.6.2.2.34	Preconfigured regroup request (MCPTT client – MCPTT server)

Table 10.6.2.2.34-1 describes the information flow preconfigured regroup request from the MCPTT client to the MCPTT server.

Table 10.6.2.2.34-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester |
| MCPTT group ID | O | MCPTT group ID of the regroup group |
| MCPTT group ID | M | MCPTT group ID of the MCPTT group from which configuration is to be taken |
| MCPTT group ID list | O (see NOTE 1) | List of MCPTT groups to be regrouped into the group regroup group |
| MCPTT user ID list | O (see NOTE 1) | List of MCPTT users to be regrouped into the user regroup group |
| Requested priority | O | Priority level requested for the call. |
| MCPTT user criteria | O (see NOTE 1, NOTE 2) | Carries the details of criteria which will be used by the MCPTT server for determining the participants. For example, it can be a location based criteria to determine the MCPTT user ID list in a particular area, or it could be tags (e.g. “fire”, “medical”, “police”, etc.), or a combination of tags and location, which is left to implementation. |
| NOTE 1: One and only one of these shall be present. NOTE 2: This information element is used only for the user regroup procedures |  |  |

10.6.2.2.35	Preconfigured regroup request (MCPTT server – MCPTT client)

Table 10.6.2.2.35-1 describes the information flow preconfigured regroup request from the MCPTT server to the MCPTT client.

Table 10.6.2.2.35-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the target MCPTT group member |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| MCPTT group ID | M | MCPTT group ID of the MCPTT group from which configuration is to be taken |
| MCPTT group ID list | O (see NOTE) | List of MCPTT groups to be regrouped into the group regroup group |
| MCPTT user ID list | O (see NOTE) | List of MCPTT users to be regrouped into the user regroup group |
| NOTE: One and only one of these shall be present. |  |  |

10.6.2.2.36	Preconfigured regroup request (MCPTT server – MCPTT server)

Table 10.6.2.2.36-1 describes the information flow preconfigured regroup request from the MCPTT server to the MCPTT server.

Table 10.6.2.2.36-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| MCPTT group ID | M | MCPTT group ID of the MCPTT group from which configuration is to be taken |
| MCPTT group ID list | O (see NOTE) | List of MCPTT groups to be regrouped into the group regroup group |
| MCPTT user ID list | O (see NOTE) | List of MCPTT users to be regrouped into the user regroup group |
| Requested priority | O | Priority level requested for the call. |
| NOTE: One and only one of these shall be present. |  |  |

10.6.2.2.37	Preconfigured regroup response (MCPTT client – MCPTT server)

Table 10.6.2.2.37-1 describes the information flow preconfigured regroup response from the MCPTT client to the MCPTT server.

Table 10.6.2.2.37-1 Preconfigured regroup response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the responding MCPTT client |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| Result | M | Result of the regrouping operation |

10.6.2.2.38	Preconfigured regroup response (MCPTT server – MCPTT client)

Table 10.6.2.2.38-1 describes the information flow preconfigured regroup response from the MCPTT server to the MCPTT client.

Table 10.6.2.2.38-1 Preconfigured regroup response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester of the regrouping operation |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| Result | M | Result of the regrouping operation |

10.6.2.2.39	Preconfigured regroup response (MCPTT server – MCPTT server)

Table 10.6.2.2.39-1 describes the information flow preconfigured regroup response from the MCPTT server to the MCPTT server.

Table 10.6.2.2.39-1 Preconfigured regroup response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the responding MCPTT client |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| Result | M | Result of the regrouping operation |

10.6.2.2.40	Preconfigured regroup cancel request (MCPTT client – MCPTT server)

Table 10.6.2.2.40-1 describes the information flow preconfigured regroup cancel request from the MCPTT client to the MCPTT server.

Table 10.6.2.2.40-1 Preconfigured regroup cancel request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester |
| MCPTT group ID | M | MCPTT group ID of the regroup group |

10.6.2.2.41	Preconfigured regroup cancel request (MCPTT server – MCPTT client)

Table 10.6.2.2.41-1 describes the information flow preconfigured regroup cancel request from the MCPTT server to the MCPTT client.

Table 10.6.2.2.41-1 Preconfigured regroup cancel request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the target MCPTT user or MCPTT group member |
| MCPTT group ID | M | MCPTT group ID of the regroup group |

10.6.2.2.42	Preconfigured regroup cancel request (MCPTT server – MCPTT server)

Table 10.6.2.2.42-1 describes the information flow preconfigured regroup cancel request from the MCPTT server to the MCPTT server.

Table 10.6.2.2.42-1 Preconfigured regroup cancel request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester |
| MCPTT group ID | M | MCPTT group ID of the regroup group |

10.6.2.2.43	Preconfigured regroup cancel response (MCPTT client – MCPTT server)

Table 10.6.2.2.43-1 describes the information flow preconfigured regroup cancel response from the MCPTT client to the MCPTT server.

Table 10.6.2.2.43-1 Preconfigured regroup cancel response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the responding MCPTT client |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| Result | M | Result of the regrouping operation |

10.6.2.2.44	Preconfigured regroup cancel response (MCPTT server – MCPTT client)

Table 10.6.2.2.44-1 describes the information flow preconfigured regroup cancel response from the MCPTT server to the MCPTT client.

Table 10.6.2.2.44-1 Preconfigured regroup cancel response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester of the regroup removal |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| Result | M | Result of the regrouping operation |

10.6.2.2.45	Preconfigured regroup cancel response (MCPTT server – MCPTT server)

Table 10.6.2.2.45-1 describes the information flow preconfigured regroup cancel response from the MCPTT server to the MCPTT server.

Table 10.6.2.2.45-1 Preconfigured regroup cancel response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester of the regroup removal |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| Result | M | Result of the regroup removal operation |

10.6.2.2.46	Preconfigured regroup reject (MCPTT server – MCPTT client)

Table 10.6.2.2.46-1 describes the information flow preconfigured regroup reject from the MCPTT server to the MCPTT client.

Table 10.6.2.2.46-1 Preconfigured regroup reject information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the requester of the regrouping |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| Reject reason | M | Reason for rejecting the regrouping operation |

10.6.2.2.47	Preconfigured regroup reject (MCPTT server – MCPTT server)

Table 10.6.2.2.47-1 describes the information flow preconfigured regroup reject from the MCPTT server to the MCPTT server.

Table 10.6.2.2.47-1 Preconfigured regroup reject information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT group ID | M | MCPTT group ID of the regroup group |
| Reject reason | M | Reason for rejecting the regrouping operation |

10.6.2.2.48	Preconfigured regroup request return (MCPTT server – MCPTT client)

Table 10.6.2.2.48-1 describes the information flow preconfigured regroup request from the MCPTT server to the MCPTT client.

Table 10.6.2.2.48-1 Preconfigured regroup request return information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT group ID | O | The MCPTT group ID to be associated with the preconfigured regroup group call which is either provided by the group call initiator or assigned by the MCPTT server. This information element shall be present if the authorization result is success. |
| Authorization result | M | Indicate if authorization is success or failure |

10.6.2.2.49	Preconfigured user regroup remove request (MCPTT server – MCPTT client)

Table 10.6.2.2.49-1 describes the information flow preconfigured user regroup remove request from the MCPTT server to MCPTT client.

Table 10.6.2.2.49-1 Preconfigured user regroup remove request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the MCPTT user to be removed from the regroup group |
| MCPTT group ID | M | MCPTT group ID of the regroup group from which the MCPTT user is to be removed |

10.6.2.2.50	Preconfigured user regroup remove response (MCPTT client – MCPTT server)

Table 10.6.2.2.50-1 describes the information flow preconfigured user regroup remove response from the MCPTT client to the MCPTT server.

Table 10.6.2.2.50-1 Preconfigured user regroup remove response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the MCPTT user to be removed from the preconfigured regroup |
| Result | M | Result of the preconfigured user regroup removal operation |