3GPP TSG-SA WG6 Meeting #57	S6-232979

Xiamen, China, 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0441 | rev |  | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the regroup group member update during MCX session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 23.379,10.6.2.9.5.1 MCPTT client PTTs on MCPTT group during an in-progress preconfigured group regroup and 10.6.2.9.5.2 MCPTT client PTTs on preconfigured regroup group after group regroup has been cancelled are used to update the regroup group members (i.e., adding new member, remove an existing member) during an MCPTT session. This is also required for MCVideo and MCData. So in 23.280 a similar procedure should introduced to update the group member during the session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the new procedure to update the regroup group member |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency will be caused between the two preconfigured regroups in two individual specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 10.15.3.aa, (new) 10.15.3.aa.1, (new) 10.15.3.aa.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ First change ******************************/

10.15.3.4.aa	Adding a user to regroup group due to comunication request to the group being regrouped

Figure 10.15.3.4.aa-1 illustrates the procedure of adding a user to regroup group due to the user initiatingng a comunication request to the group involved in an in-progress preconfigured MC service group regroup.

Pre-conditions:

-	The MC service client is an affiliated member of MC service group A that is part of an in-progress preconfigured group regroup with MC service groups B and C.

-	MC servicec group D is being used as the preconfigured regroup group.

-	The MC service client has missed the preconfigured regroup request message (e.g. poor signalling conditions, race condition).

MC service client

MC service server

1. Receive a communication request on the group involved in an in-progress preconfigured MC service group regroup.

2.Preconfigured regroup request

3. Notify

user

4. Preconfigured regroup response

5. Affiliate MC service client to regroup group

Figure 10.15.3.4.aa-1: Procedure for Communication request on MCPTT group during an in-progress preconfigured group regroup

1.	The MC service client attempts to start a communication on MC service group A. The MC service server identifies that the MC service group A is currently part of a preconfigured group regroup. In this case the group is part of an active preconfigured group regroup and rejects the communication. This is described in MCPTT as specified in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], 3GPP TS 23.281 [13].

2.	The MC service server sends the preconfigured regroup request to the MC service client containing the group ID of the preconfigured group.

NOTE 1: This message should be sent over unicast.

3.	The MC service client notifies the user of the regrouping procedure.

4.	The MC service client sends the preconfigured regroup response to the MC service server to acknowledge the regrouping action.

5.	The MC service server affiliates the regrouped MC service client into the regroup group.

10.15.3.4.bb	Regroup group cancel notification due to communication request to the cancelled regroup group

Figure 10.15.3.4.bb-1 illustrates the procedure of notifying about the cancellation of the regroup group when a user initicates a communication to a regroup group after the preconfigured MC service group regroup has been cancelled.

Pre-conditions:

-	The MC service client is a member of MC service group A that was part of an in-progress preconfigured group regroup with MC service groups B and C that has been cancelled. MC service group D was used as the MC service regroup group.

-	The MC service client has missed the preconfigured regroup cancel request message (e.g. poor signalling conditions, race condition).

MC service client

MC service server

1. Receive a communication request to the regroup group no longer exist

2.Preconfigured regroup cancel request

3. Notify

userc

4. Preconfigured regroup cancel response

Figure 10.15.3.4bb-1: Procedure for communication request on regroup group after the group regroup is cancelled

1.	The MC service client attempts to start a communication on MC service group D, which is the regroup group. The MC servce server identifies group regroup is no longer active or exist.

NOTE 1: The regroup group D can be a group in the MC service client's profile, and the MC service client can be a member of group D.

2.	The MC service server may send  the preconfigured regroup cancel request to the MC service client.

NOTE 2: This message should be sent over unicast.

3.	The MC service client may notify the user of the regrouping cancellation.

4.	The MC service client may send the preconfigured regroup cancel response to the MC service server to acknowledge the regrouping cancellation.