3GPP TSG-SA WG6 Meeting #57	S6-232982

Xiamen, China, 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0442 | rev |  | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update preconfigured regroup information flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, the MCPTT utilizes the preconfigured regroup procedures in 23.379 while the MCVideo and MCData utilize the preconfigured regroup procedures in 23.280. However, the requirements of the preconfigured regroup for all the three services are identical. Now the two sets of procedures are not fully aligned and make it difficult to maintain the consistency between the 23.280 and 23.379. In order to resolve the issue, keeping a single preconfigured regroup procedures in 23.280 for all the services is a good choice. So the descriptions in table 10.15.2.1-1, 10.15.2.2-1, 10.15.2.3-1 should be updated with the corresponding IE from 23.379 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the table 10.15.2.1-1, 10.15.2.2-1, 10.15.2.3-1 condiering the IEs from 23.379. Introduce a new table in 23.280 about the new Preconfigured regroup request return message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency will be caused between the two preconfigured regroups in two individual specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.15.2.1, 10.15.2.2, 10.15.2.3, (new) 10.15.2.aa |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ First change ******************************/

10.15.2.1	Preconfigured regroup request (MC service client – MC service server)

Table 10.15.2.1-1 describes the information flow preconfigured regroup request from the MC service client to the MC service server.

Table 10.15.2.1-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requester |
| MC service group ID | O | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MC service group ID list | O (see NOTE 1) | List of MC service groups to be regrouped into the group regroup group |
| MC service user ID list | O (see NOTE 1) | List of MC service users to be regrouped into the user regroup group |
| Requested priority | O | Priority level requested for the call. |
| MC service user criteria | O (see NOTE 1, NOTE 2) | Carries the details of criteria which will be used by the MC service server for determining the participants. For example, it can be a location based criteria to determine the MC service user ID list in a particular area, or it could be tags (e.g. "fire", "medical", "police", etc.), or a combination of tags and location, which is left to implementation. |
| NOTE 1: One and only one of these shall be present. NOTE 2: This information element is used only for the user regroup procedures |  |  |

10.15.2.2	Preconfigured regroup request (MC service server – MC service client)

Table 10.15.2.2-1 describes the information flow preconfigured regroup request from the MC service server to the MC service client.

Table 10.15.2.2-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the target MC service group member |
| MC service group ID | M | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MCP service group ID list | O (see NOTE) | List of MC service groups to be regrouped into the group regroup group |
| MC service user ID list | O (see NOTE) | List of MC service users to be regrouped into the user regroup group |
| NOTE: One and only one of these shall be present. |  |  |

10.15.2.3	Preconfigured regroup request (MC service server – MC service server)

Table 10.15.2.3-1 describes the information flow preconfigured regroup request from the MC service server to the MC service server.

Table 10.15.2.3-1 Preconfigured regroup request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requester |
| MC service group ID | M | MC service group ID of the regroup group |
| MC service group ID | M | MC service group ID of the MC service group from which configuration is to be taken |
| MC service group ID list | O | List of MC service groups to be regrouped into the group regroup group |
| MC service user ID list | O (see NOTE) | List of MC service users to be regrouped into the user regroup group |
| Requested priority | O | Priority level requested for the call. |
| NOTE: One and only one of these shall be present. |  |  |

# /************************ Next change ******************************/

10.15.2.aa	Preconfigured regroup request return (MC service server – MC service client)

Table 10.15.2.aa-1 describes the information flow preconfigured regroup request from the MC service server to the MC service client.

Table 10.15.2.aa-1 Preconfigured regroup request return information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the calling party |
| MC service group ID | O | The MC service group ID to be associated with the preconfigured regroup group call which is either provided by the group call initiator or assigned by the MC service server. This information element shall be present if the authorization result is success. |
| Authorization result | M | Indicate if authorization is success or failure |