3GPP TSG-SA WG6 Meeting #57	S6-233019

Xiamen, China 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0392 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of ad hoc group call participants involving multiple MCPTT systems |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current specification, the ad hoc group call is established using the list of participants or the criteria provided by the initiator of the call. Further, the initiator can modify the ongoing ad hoc group call participants by providing one or more users to be added or removed from the ad hoc group call. The MCPTT server can also modify the list of participants based on one or more users meeting the criteria or no longer meeting the criteria. The involved participants can be part of both the primary and partner MCPTT systems. Currently modifying an ongoing ad hoc group call participants by initiator or MCPTT server is supported in case of users belonging to the same MCPTT system and the modifying of an ongoing ad hoc group call participants by MCPTT server is supported in case of users belonging to multiple MCPTT systems. Modifying ad hoc group call participants involving multiple MCPTT systems in which users are added or removed from the partner MCPTT system by the initiator is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A missing procedure is defined to modify the ongoing ad hoc group call participants by providing the list of users to be added or removed from the ad hoc group call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Modifying the ad hoc group call participants involving multiple MCPTT systems in which users are added or removed from the partner MCPTT system is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.3.2.4, 10.19.3.2.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 10.19.3.2.4	Modification of ad hoc group call participants by the MCPTT server

Figure 10.19.3.2.4-1 below illustrates the MCPTT server modifying the ad hoc group call participants procedure involving multiple MCPTT systems.

Pre-conditions:

1.	The MCPTT user at MCPTT client 1 is authorized to initate ad hoc group call.

2.	The MCPTT server 1 of the primary and MCPTT server 2 of the partner MCPTT systems determined the participants for the ad hoc group call based on the criteria specified by the MCPTT client 1 while initiating the ad hoc group call.

3.	The MCPTT server 1 of the primary and MCPTT server 2 of the partner MCPTT systems continuously evaluates the criteria to monitor the list of users who meets or not meets the criteria for participating in the on-going ad hoc group call.

4.	The MCPTT server 1 of the primary MCPTT system detects that the MCPTT client 3 meets the criteria and MCPTT client 2 stops to meet the criteria specified by the MCPTT client 1.

5.	The MCPTT server 2 of the partner MCPTT system detects that the MCPTT client 5 meets the criteria and MCPTT client 4 stops to meet the criteria specified by the MCPTT client 1.

Figure 10.19.3.2.4-1: Modification of ad hoc group call participants by the MCPTT server

1.	The ad hoc group call is established and on-going with the participants MCPTT client 1, MCPTT client 2, and MCPTT client 4. The participants list is determined by both primary and partner systems MCPTT server based on the criteria specified by the MCPTT client 1 while initiating the call.

2.	The MCPTT server 1 detects that MCPTT client 3 now meets criteria and successfully added to call.

3.	The MCPTT server 1 detects that MCPTT client 2 does not meet criteria any more and successfully removed from a call.

4.	The MCPTT server 2 detects that the MCPTT client 5 meets the criteria specified by the MCPTT client 1.

5.	The MCPTT server 2 sends the notification to MCPTT server 1 of the primary MCPTT system to add the MCPTT user at MCPTT client 5 to on-going ad hoc group call.

6.	The MCPTT server 1 sends the ad hoc group call request to the MCPTT client 5.

7.	The MCPTT client 5 notifies the user about the incoming ad hoc group call.

8.	The MCPTT client 5 accepts the ad hoc group call request and sends the ad hoc group call response to the MCPTT server 1.

9.	The on-going ad hoc group call is updated by adding MCPTT client 5 which meets the criteria specified by the MCPTT client 1.

10.	The MCPTT server 2 detects that the MCPTT client 4 is no more satisfying the criteria to be the participant of the ad hoc group call.

11.	The MCPTT server 2 sends the notification to MCPTT server 1 of the primary MCPTT system to remove the MCPTT user at MCPTT client 4 from on-going ad hoc group call.

12.	The MCPTT server 1 sends the ad hoc group call leave request to the MCPTT client 4 and removes it from the on-going ad hoc group call.

13.	The MCPTT client 4 notifies the user of the ad hoc group call leave request.

14.	The MCPTT client 4 sends the ad hoc group call leave response to the MCPTT server 1.

15.	The on-going ad hoc group call is updated by removing MCPTT client 4, which no more meets the criteria specified by the MCPTT client 1.

NOTE:	The ad hoc group call request and response exchanged between MCPTT server 1 of primary MC system and MCPTT client 4/MCPTT client 5 will always traversal through the MCPTT server 2.

* * * Next Change * * * *

##### 10.19.3.2.6	Modification of ad hoc group call participants by the initiator

Figure 10.19.3.2.6-1 below illustrates the MCPTT user modifying the ad hoc group call participants procedure involving multiple MCPTT systems.

Pre-conditions:

1.	An MCPTT ad hoc group call is already in progress and is originated by an authorized MCPTT user at MCPTT client 1 with the participants list provided by the originating MCPTT user while initiating the MCPTT ad hoc group call.

2.	The participants of the MCPTT ad hoc group call belongs to the multiple MCPTT system.

3.	The MCPTT user at MCPTT client 1 determines that the MCPTT client 3 needs to be invited to on-going ad hoc group call and MCPTT client 2 needs to be removed from the on-going ad hoc group call from the primary MCPTT system.

4.	The MCPTT user at MCPTT client 1 determines that the MCPTT client 5 needs to be invited to on-going ad hoc group call and MCPTT client 4 needs to be removed from the on-going ad hoc group call from the partner MCPTT system.

Figure 10.19.3.2.6-1: Modification of ad hoc group call participants by the initiator

1.	The MCPTT client 1 sends the modify ad hoc group call participants request to the MCPTT server 1 to remove both MCPTT client 2 and MCPTT client 4 from the on-going ad hoc group call and to add both MCPTT client 3 and MCPTT client 5 into on-going ad hoc group call.

2.	The MCPTT server 1 verifies whether the MCPTT client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call.

3.	The MCPTT server 1 sends modify ad hoc group call participants response to the MCPTT client 1.

4.	The MCPTT server 1 determines that MCPTT client 1 has requested that MCPTT client 3 from the primary MC system be invited to an on-going ad hoc group call and successfully added to the call according to the procedure defined in clause 10.19.3.1.4.

5.	The MCPTT server 1 determines that MCPTT client 1 has requested that MCPTT client 2 from the primary MC system be removed from the on-going ad hoc group call and successfully removed from call according to the procedure defined in clause 10.19.3.1.4.

6.	The MCPTT server 1 determines that MCPTT client 1 has requested that MCPTT client 5 from the partner MC system be invited to an on-going ad hoc group call. The MCPTT server 1 sends the ad hoc group call request to the MCPTT client 5.

7.	The MCPTT client 5 notifies the user about the incoming ad hoc group call.

8.	The MCPTT client 5 accepts the ad hoc group call request and sends the ad hoc group call response to the MCPTT server 1.

9.	The on-going ad hoc group call is updated by adding MCPTT client 5 which is based on the modify participant list provided by the MCPTT client 1.

10.	The MCPTT server 1 determines that MCPTT client 1 has requested that MCPTT client 4 from the partner MC system be removed from the on-going ad hoc group call. The MCPTT server 1 sends the ad hoc group call leave request to the MCPTT client 4.

11.	The MCPTT client 4 notifies the user of the ad hoc group call leave request.

12.	The MCPTT client 4 sends the ad hoc group call leave response to the MCPTT server 1.

13.	The on-going ad hoc group call is updated by removing MCPTT client 4, which is based on the modify participant list provided by the MCPTT client 1.

NOTE:	The ad hoc group call request and response exchanged between MCPTT server 1 of primary MC system and MCPTT client 4/MCPTT client 5 will always traversal through the MCPTT server 2.

14.	The MCPTT server 1 may notify the initiating MCPTT user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the ad hoc group call.

15.	The MCPTT server may notify the participants about the change in the participants list of on-going ad hoc group call.

* * * End of Changes * * * *