3GPP TSG-SA WG6 Meeting #57	S6-233046

Xiamen, 9th – 13th October 2023	(revision of S6-23xxxx)

Source:	Apple

Title:	AS registration to Edge platform procedure

Spec:	3GPP TR 23.958

Agenda item:	7.9

Document for:	Approval

Contact:	Walter Featherstone (w_featherstone@apple.com)

1. Introduction

This pCR is based on S6-232485 provided by Huawei, Hisilicon that was postponed at SA6#56. It provides a proposal regarding alignment of Application Server registration procedures in 3GPP SA6 and ETSI MEC.

2. Reason for Change

The procedures for Application Registration in clause 5.3.2 of TR 23.958 are indicated as “TBD”.

3. Proposal

To approve the following changes to 3GPP TR 23.958 V1.0.0 and therefore resolve the “TBD” of clause 5.3.2.

# Change 1

### 5.3.2 	Procedures for Application Registration

#### 5.3.2.1	General

Registration of an Edge Application Server to an EES is specified in clause 8.4.3 of 3GPP TS 23.558 [2]. Registration of a MEC application instance, as an Application Server, to a MEC platform is defined in clause 5.2.13 of ETSI GS MEC 011 [5]. Both specifications provide procedures for Application Server registration, registration update and de-registration. Registration allows an Application Server to provide its information to the Edge Platform, e.g., to enable its discovery. If there is a change in Application Server information, the registration update procedure can be used by the Application Server to update its information at the Edge Platform. The update procedure can be used to maintain a time bound registration, i.e., a registration that has an expiry time. To remove Application Server information at the Edge platform, the Application Server can use the deregistration procedure.

#### 5.3.2.2	Procedure for Application Server registration to an Edge Platform

Figure 5.3.2.2-1: Procedure for Application Server registration to an Edge Platform

- The Application Server (as EAS/MEC App Instance) sends (EAS/Application) Registration request message to the Edge platform (as EES/MEC platform). The request message contains the EAS profile/AppInfo which is stored at the Edge platform in the case of a successful registration.

- Upon receiving the request message, the Edge platform sends an (EAS/Application) Registration response message to the Application Server.

Pre-conditions for registration and conditions for successful registration, including authorization requirements, are described in clause 8.4.3.2.2 of 3GPP TS 23.558 [2].

The procedures for registration update and deregistration also follow the request-response model.

# End of changes