3GPP TSG-SA WG6 Meeting #57	S6-233050

Xiamen, China, 09th – 13th October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0008 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Procedure on support for location accuracy analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Errors exist in the procedure on support for location accuracy analytics. This CR proposes to correct the errors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Corrected the errors in clause 8.5.2 for the procedure. - Changed “V2X” to “VAL” in Table 8.5.3.2-1. - Removed the parameter “Analytics filter information” in Table 8.5.3.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistancy in stage 2 specification of ADAES services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2, 8.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

### 8.5.2	Procedure

Figure 8.5.2-1 illustrates the procedure for location accuracy analytics enablement solution.

Pre-conditions:

1.	ADAES is connected to A-ADRF.

2.	ADAES has discovered SEAL LMS or FLS.

Figure 8.5.2-1: Location accuracy analytics procedure

1.	The VAL server makes a subscription request to ADAE server for location accuracy prediction/stats, including an analytics event ID (e.g. "location accuracy prediction" or "location accuracy sustainability"), an analytics request type (if not identified specifically at the event ID) which can be the location accuracy prediction for a given location X and/or for a given UE/app. The request may include also the target area, a target VAL service, or a VAL UE, or group of UEs of the VAL service, time validity, accuracy threshold and requirements. If the VAL UEs are provided by the VAL server, this request may also include the expected route or a set of waypoints for the UEs of the VAL application.

2.	The ADAE server sends a location accuracy analytics subscription response as an ACK to the VAL server.

3.	The ADAE server discovers and maps the Data Sources with the respective analytics event ID for collecting location data for the corresponding VAL UEs or VAL service area.

4.	The ADAE server subscribes for NWDAF UE mobility analytics per VAL UE (for all the VAL UEs) and gets notification on the per UE location/mobility analytics based on TS 23.288 clause 6.7.2. Such analytics may be requested for a list of waypoints per UE route (if indicated at step 1). The ADAE server subscribes also for SEAL LMS location reports for the respective VAL UEs or location reports from all VAL UEs within the requested area.

5.	The ADAE server optionally requests location accuracy historical analytics /data from A-ADRF for the corresponding VAL UEs or VAL service area.

6.	Based on the request, the ADAE server receives location accuracy historical analytics /data from A-ADRF for the corresponding VAL UEs or VAL service area.

7.	The ADAE server abstracts or correlates the data/analytics from steps 4-6 and provides analytics on the location accuracy for the target VAL application. Depending on the event ID in step 1, the ADAE server can indicate whether the location accuracy is sustainable or is predicted to be downgraded or can be upgraded and become more granular (e.g. from meter to decimetre).

8.	The ADAE server sends the location accuracy analytics notification to the consumer.

* * * Next change * * * *

#### 8.5.3.2	Location accuracy analytics subscription request

Table 8.5.3.2-1 describes information elements for the location accuracy analytics subscription request from the VAL server to the ADAE server.

Table 8.5.3.2-1: Location accuracy analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server. |
| Analytics ID | M | The identifier of the location accuracy analytics event. This ID can be for example "location accuracy prediction" or "location accuracy sustainability" depending on the expected outcome. |
| Analytics type | M | Whether analytics event is about prediction or statistics, and whether analytics are offline / online. |
| VAL UE ID(s) or Group ID | M | The identity of the VAL UE(s) or group of UEs for which the analytics subscription applies. |
| VAL service ID | O | The identifier of the VAL service for which location accuracy analytics is requested. |
| Location accuracy requirements | M | The accuracy threshold and VAL requirements. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request. |
| UE mobility / route information | O | Information on the target UE or group UE mobility including the expected route/set of waypoints. |

* * * End of changes * * * *