3GPP TSG-SA WG6 Meeting #57	S6-233059

Xiamen, China 9th – 13th October 2023

Source:	Ericsson

Title:	Solution to KI#1 related to ML client information retrieval

Spec:	3GPP TR 23.700-82 v0.0.0

Agenda item:	8.3

Document for:	Approval

Contact:	Ashish S Sharma (ashish.sanjay.sharma@ericsson.com)

1. Introduction

This pCR proposes a new solution for Key Issue#1 described in S6-233055 for FS_AIMLAPP.

2. Reason for Change

Key Issue use case: In Federated learning, the server needs to select clients or devices to collaboratively train the model. There can be different devices like low power, low computing, etc. The device’s radio link may change because of time-varying radio conditions and UE mobility. The Federated learning process is iterative and requires low latency as the model is exchanged back and forth multiple times between client and server. In Federated learning, each client trains their local model with local data and sends the updated model to the server. The server typically waits to receive model updates from each client participating in the model training. The wait period is impacted or worsened by the weakest client like sending the delayed model updates to the server or taking a relatively long time to train the model. Because of this increased wait period, the server cannot aggregate the received client model updates to train the global model. The server iteratively sends the global model to the clients back so that clients train their local model, this iterative exchange happens until the model convergence is reached. The latencies introduced by different clients because of heterogeneity in the devices like different compute capacities, varying radio conditions, different bandwidth, QoS, usage of different access technologies, and UE mobility impact the convergence of the model and lead to delayed model convergence.

Therefore it is required for the server to choose the clients based on certain parameters to achieve the fast model convergence. Since the model update exchange happens over the 3GPP system, it is important to take network-UE status, compute and communication capability into account. Hence it is necessary to provide the server with some information about the clients and its supported network interaction like supported bitrates, latencies, location, etc.

This paper proposes a solution to filter clients based on filters like Location, QoS, UE availability etc. The VAL server can use the client information to select clients for later use like model training.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82.

[1] S6-233054

* * * First Change * * * *

## X1.Y1	Solution #1: ML client information retrieval

### X1.Y1.1	General

The following clauses specify procedures, information flows and APIs for ML client information retrieval to the Requestor to enable the selection of clients for DML/Federated learning/Split Learning.

### X1.Y1.2	Procedures

Figure X1.Y1.2-1: ML client information retrieval procedure

1.	The VAL server sends the ML client information retrieval request to the AIML enabler server[1]. The request contains Requestor ID, UE Identifier, VAL service ID, application ID, and ML client filter requirements. The filter requirements enable the AIML enabler to filter the ML client based on the requested filter criteria.

2.	Upon receiving the request, the AIML enabler server subscribes to the procedure defined in clause 4.15.13 of TS 23.502. It also fetches client information from the AIML enabler client.

3.	The AIML enabler processes the NEF response and AIML client response and provides the filtered UE(s) satisfying the requested filtering criteria in the response to the Requestor

NOTE: How the AIML enabler server fetches the client information from the AIML enabler client is FFS.

* * * Next Change * * * *

### X1.Y1.3	Information flows

#### X1.Y1.3.1	ML client information retrieval request

Table X1.Y1.3.1-1 describes the information flow from the Requestor to the AIML enabler server to send the ML client information retrieval request.

Table X1.Y1.3.1-1: ML client information retrieval request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the Requestor, e.g. VAL server ID, VAL UE ID or VAL user ID |
| VAL service ID | O | Identity of the VAL service |
| UE identities | M | Identifier of UEs, e.g. UE ID, UE address |
| Application Identity | O | Identifier of application |
| ML client filter requirements | O | Indicates the filter criterias for ML clients as described in Table X1.Y1.3.1-2. |

Table X1.Y1.3.1-2: ML client filter requirements

| Information element | Status |  | Description |  |
| --- | --- | --- | --- | --- |
| Location Information | O |  | Indicates one or more than one location information |  |
| UE QoS | O |  | Indicates the UE QoS. E.g bitrate, jitter, latency, PLR |  |
| AIML compute support | O |  | Indicates the UE compute capacity. Eg CPU; TPU; GPU, |  |
| UE velocity | O |  | Indicates the velocity of the UE. |  |
| UE availabilty | O |  | Indicates the UE availability information e.g. available or unavailable |  |
| ML client capability type | O | Indicates the supported ML client capabilities, e.g DML, FL, ML, Split Learning(SL) |
| ML model convergence communication rounds | O |  | Indicates the number of successful communication rounds required for the UE to reach convergence. |  |
| UE battery level | O |  | Indicates the UE battery level in percent |  |
| UE drop-offs rate | O |  | Indicates the number of times UE drops off during model training. |  |

#### X1.Y1.3.2	ML client information retrieval response

Table X1.Y1.3.2-1 describes the information flow from the AIML Enabler server to the Requestor to send ML client information retrieval response.

Table X1.Y1.3.2-1 ML client information retrieval response

| Information element | Status | Description |
| --- | --- | --- |
| Filtered UE identities | M | Identifier of UEs, e.g. UE ID, UE address that satisfies the filtering criteria |

* * * End of Changes * * * *