3GPP TSG-SA WG6 Meeting #57	S6-233071

Xiamen, China 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0081 | rev | 0 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE-triggered protection zone configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2023-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.21.1; New: 9.21.2.x1; 9.21.2.x2; 9.21.2.x3; 9.21.x4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Notes:

Decribe the “Protection Zone” functionality – maybe ppt?

* * * First Change * * * *

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

The terms and definitions as specified in 3GPP TS 23.286 [6] apply.

VRU high-risk zone: Refers to a geographic area with Vulnerable Road Users (VRUs) recognition mechanisms.

Protection zone: Refers to a geographic area surrounding a UE in which functionality such as ITS V2X, V2P, V2V is employed for the immediate purpose of protection of the UE and of the area VRUs.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BSM	Basic Safety Message

CAM	Cooperative Awareness Message

CPM	Collective Perception Message

DENM	Decentralized Environmental Notification Message

V2P	Vehicle-to-Pedestrian

V2X	Vehicle-to-Everything

VAE	V2X Application Enabler

VAM	VRU awareness message

VASS	V2X Application Specific Server

VRU	Vulnerable Road Users

VRUP	Vulnerable Road User Protection

* * * Next Change * * * *

### 9.21.1	General

This capability supports the configuration and provisioning of the VRU high risk zone at the VAE layer based on a request from the V2X application specific server (VASS). ). UE-initiated triggering of configuration and provisioning of protection zones is also supported at the VAE layer.

NOTE: In this specification, protection zones in the service layer are specific to the protected UE and map to one or more dynamic VRU high-risk zones in the underlying network.

* * * Next Change * * * *

### 9.21.2	Information flows

#### 9.21.2.x1	Protection zone management request

Table 9.21.2.x1-1 describes information elements for the Protection zone management request from the VAE client to the VAE server.

Table 9.21.2.x1-1: Protection zone management request

| Information element | Status | Description |
| --- | --- | --- |
| Protected UE ID | M | The identifier of the VAL UE for which the protection zone is applicable |
| Protected UE type(s) | M | This information identifies which type of UE (V2X UEs, pedestrian UEs) to be protected. A VRU may be mapped to multiple types (based on VRUP scenario) |
| Protection zone area information | M | Provides information about the protection zone area relative to the protected U, as well as the geographical or service area for which the request applies. This element may include e.g. a preset route, a specified area around the protected UE as it moves, etc. |
| VRU application requirements | M | Indicates the types of supported messages and requirements (e.g. requirements for VAM messages), and application QoS requirement dedicated for the VRU high-risk zone (e.g. URLLC like). |
| V2P communication parameters | M | This includes V2P communication parameters such as V2X identifiers, radio communication parameters, supported transmission modes (unicast, groupcast, broadcast) within the protection zone, communication schedule, and QoS parameters. |
| Location reporting interval | O | A time interval for which location reporting is provided to update the protection zone |
| Time validity | M | The time validity of the request |

#### 9.21.2.x2	Protection zone management response

Table 9.21.2.x2-1 describes information elements for the Protection zone management subscription response from the VAE server to VAE client.

Table 9.21.2.x2-1: Protection zone management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the protection zone management subscription request (positive or negative acknowledgement) |
| Protection zone ID | M | An identifier of the Protection zone assigned by the VAE layer. |

#### 9.21.2.x3	Protection zone management notification

Table 9.21.2.x3-1 describes information elements for the Protection zone management notification from the VAE server to the VASS.

Table 9.21.2.x3-1: Protection zone management notification

| Information element | Status | Description |
| --- | --- | --- |
| Protected UE ID | M | The identifier of the VAL UE for which the protection zone is applicable |
| Protection zone ID | M | An identifier of the Protection zone which may be provided by the requestor or assigned by the VAE layer. |
| Protected UE type(s) | M | This information identifies the type of UE (V2X UEs, pedestrian UEs) to be protected. A VRU may be mapped to multiple types (based on VRUP scenario) |
| Protection zone information | M | This information identifies which type of UEs (V2X UEs, pedestrian UEs, etc.) are considered VRUs in relationship to the protected UE. |
| Protection zone configuration parameters | M | These parameters include: - The list of value-add VAE services required within the zone (e.g. location tracking, V2X message bundling), - the supported transmission modes (unicast, groupcast, broadcast) within the applications within the zone, - the allowed interface selection within the zone (Uu, PC5), the use of application relaying or not |
| Area information | M | The geographical or service area for which the request applies |
| V2P communication parameters | M | This includes V2P communication parameters such as V2X identifiers, radio communication parameters, supported transmission modes (unicast, groupcast, broadcast) within the protection zone, communication schedule, and QoS parameters. |
| Protection adaptation information | O | Provides information to allow the network to adapt configurations of dynamic VRU high-risk zones configured to create the broader protection zone. |
| Time validity | M | The time validity of the request |

* * * Next Change * * * *

### 9.21.x4	 Procedure on UE/VRU-triggered protection zone configuration

Protection zones are defined specific to a VRU and are based on UE/VRU-initiated requests and VASS configurations. Based on this information, the VAE layer may configure one or more dynamic VRU high-risk zones which jointy provide the protection zone functionality. The procedure in figure 9.21.x4-1 complements the VASS Server-triggered VRU high-risk zone configuration procedure in clause 9.21.3.

Pre-conditions:

- V2X application specific server or client has subscribed to VAE layer to provide support for V2P communication.

- The VAE server is preconfigured with a policy allowing support of UE protection zone configuration and support of UE/VRU-initiated requests.

Figure 9.21.x4-1: UE-triggered protection zone configuration

- a.  Optionally, an application client on the UE makes a request to the VAE client to create a UE/VRU protection zone.

b.  The VAE client sends a protection zone management request to a VAE server to create a protection zone. The request includes the information elements specified in Table 9.21.2.x3-1.

2.	The VAE server sends a protection zone management notification to a VASS for the creation of the protection zone for the UE and includes the information provided in the request.

3.	The VASS authorizes the request and determines whether a protection zone based on information received from the VAE server is to be created. The VASS provides the VAE server with the protection zone and V2P communication configuration.

4.	The VASS determines one or more dynamic VRU high-risk zone configurations corresponding to the protection zone request and configures the VAE server accordingly. Steps 1-3 of clause 9.21.3 are executed for each of the dynamic VRU high-risk zones determined.

5.	The VAE server configures CN and SEAL LMS subscriptions to obtain UE location from the 5G system for the protected UE and for vehicular UEs around the protected zone. Sources of UE mobility information may come from SEAL LMS servers, EESs, analytics functions (e.g. from ADAES or NWDAF), and the 5G core network. For V2X UE location obtained from the location management server, the procedures specified in 3GPP TS 23.434 (clause 9.3.12) apply.

- a. The VAE Server sends a protection zone management response to the VAE client that the protection zone has been created.

b.  Optionally, the VAE client sends a response to the triggering application client on the protected UE, confirming the creation of the protection zone.

7.	The VAE server uses notifications from the 5G network and SEAL LMS (e.g.updated location information of the protected UE) to update the protection zone configuration, which may result in updates to one or more of the component dynamic VRU high-risk zones.

8.	The VAE server sends notification messages to other VRUs (pedestrians or V2X-UEs at similar risk) within each of the component dynamic VRU high-risk zones. The VAE server also sends vehicular UEs within these zones V2P communication parameters to enable communication between the vehicular UEs and the protected UE.

9. a.The VAE server alerts the VAE client when V2X UEs posing risk to the protected UE are expected to move, in a given time horizon, into the dynamic VRU high-risk zones composing the protection zone.

b. The VAE server alerts VASS when V2X UEs posing risk to the protected UE are expected to move into each of the dynamic VRU high-risk zones composing the protection zone.

* * * End of Changes * * * *