3GPP TSG-SA WG6 Meeting #57	S6-233156

Xiamen, China 9th – 13th October 2023	(revision of S6-233021)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0201 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of ad hoc group call participants involving multiple MCVideo systems |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current specification, the ad hoc group call is established using the list of participants. Further, the authorized user can modify the ongoing ad hoc group call participants by providing one or more users to be added or removed from the ad hoc group call. The MCVideo server can also modify the list of participants based on one or more users meeting the criteria or no longer meeting the criteria. The involved participants can be part of both the primary and partner MCVideo systems. Currently modifying an ongoing ad hoc group call participants by authorized user or MCVideo server is supported in case of users belonging to the same MCVideo system and the modifying of an ongoing ad hoc group call participants by MCVideo server is supported in case of users belonging to multiple MCVideo systems. Modifying ad hoc group call participants involving multiple MCVideo systems in which users are added or removed from the partner MCVideo system by the authorized user is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A missing procedure is defined to modify the ongoing ad hoc group call participants by providing the list of users to be added or removed from the ad hoc group call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Modifying the ad hoc group call participants involving multiple MCVideo systems in which users are added or removed from the partner MCVideo system is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.19.3.2.2, 7.19.3.2.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 7.19.3.2.2	Modification of ad hoc group call participants by the MCVideo server

Figure 7.19.3.2.2-1 below illustrates the MCVideo server modifying the ad hoc group call participants procedure involving multiple MCVideo systems.

Pre-conditions:

1.	The MCVideo user at MCVideo client 1 is authorized to initate ad hoc group call.

2.	The MCVideo server 1 of the primary and MCVideo server 2 of the partner MCVideo systems determined the participants for the ad hoc group call based on the criteria specified by the MCVideo client 1 while initiating the ad hoc group call.

3.	The MCVideo server 1 of the primary and MCVideo server 2 of the partner MCVideo systems continuously evaluates the criteria to monitor the list of users who meets or not meets the criteria for participating in the on-going ad hoc group call.

4.	The MCVideo server 1 of the primary MCVideo system detects that the MCVideo client 3 meets the criteria and MCVideo client 2 stops to meet the criteria specified by the MCVideo client 1.

5.	The MCVideo server 2 of the partner MCVideo system detects that the MCVideo client 5 meets the criteria and MCVideo client 4 stops to meet the criteria specified by the MCVideo client 1.

Figure 7.19.3.2.2-1: Modification of ad hoc group call participants by the MCVideo server

1.	The ad hoc group call is established and on-going with the participants MCVideo client 1, MCVideo client 2, and MCVideo client 4. The participants list is determined by both primary and partner systems MCVideo server based on the criteria specified by the MCVideo client 1 while initiating the call.

2.	The MCVideo server 1 detects that MCVideo client 3 now meets criteria and successfully added to call.

3.	The MCVideo server 1 detects that MCVideo client 2 does not meet criteria any more and successfully removed from a call.

4.	The MCVideo server 2 detects that the MCVideo client 5 meets the criteria specified by the MCVideo client 1.

5.	The MCVideo server 2 sends the notification to MCVideo server 1 of the primary MCVideo system to add the MCVideo user at MCVideo client 5 to on-going ad hoc group call.

6.	The MCVideo server 1 sends the ad hoc group call request to the MCVideo client 5.

7.	The MCVideo client 5 notifies the user about the incoming ad hoc group call.

8.	The MCVideo client 5 accepts the ad hoc group call request and sends the ad hoc group call response to the MCVideo server 1.

9.	The on-going ad hoc group call is updated by adding MCVideo client 5 which meets the criteria specified by the MCVideo client 1.

10.	The MCVideo server 2 detects that the MCVideo client 4 is no more satisfying the criteria to be the participant of the ad hoc group call.

11.	The MCVideo server 2 sends the notification to MCVideo server 1 of the primary MCVideo system to remove the MCVideo user at MCVideo client 4 from on-going ad hoc group call.

12.	The MCVideo server 1 sends the ad hoc group call leave request to the MCVideo client 4 and removes it from the on-going ad hoc group call.

13.	The MCVideo client 4 notifies the user of the ad hoc group call leave request.

14.	The MCVideo client 4 sends the ad hoc group call leave response to the MCVideo server 1.

15.	The on-going ad hoc group call is updated by removing MCVideo client 4, which no more meets the criteria specified by the MCVideo client 1.

NOTE:	The ad hoc group call request and response exchanged between MCVideo server 1 of primary MC system and MCVideo client 4/MCVideo client 5 will always traversal through the MCVideo server 2.

* * * Next Change * * * *

##### 7.19.3.2.6	Modification of ad hoc group call participants by an authorized user

Figure 7.19.3.2.6-1 below illustrates the MCVideo user modifying the ad hoc group call participants procedure involving multiple MCVideo systems.

Pre-conditions:

1.	An MCVideo ad hoc group call is already in progress and the participants list provided by the originating MCVideo user while initiating the MCVideo ad hoc group call.

2.	The participants of the MCVideo ad hoc group call belongs to the multiple MCVideo system.

3.	The MCVideo user at MCVideo client 1 determines that the MCVideo client 3 needs to be invited to on-going ad hoc group call and MCVideo client 2 needs to be removed from the on-going ad hoc group call from the primary MCVideo system.

4.	The MCVideo user at MCVideo client 1 determines that the MCVideo client 5 needs to be invited to on-going ad hoc group call and MCVideo client 4 needs to be removed from the on-going ad hoc group call from the partner MCVideo system.

Figure 7.19.3.2.6-1: Modification of ad hoc group call participants by an authorized user

1.	The MCVideo client 1 sends the modify ad hoc group call participants request to the MCVideo server 1 to remove both MCVideo client 2 and MCVideo client 4 from the on-going ad hoc group call and to add both MCVideo client 3 and MCVideo client 5 into on-going ad hoc group call.

2.	The MCVideo server 1 verifies whether the MCVideo client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call.

3.	The MCVideo server 1 sends modify ad hoc group call participants response to the MCVideo client 1.

4.	The MCVideo server 1 determines that MCVideo client 1 has requested that MCVideo client 3 from the primary MC system be invited to an on-going ad hoc group call and successfully added to the call according to the procedure defined in clause 7.19.3.1.4.

5.	The MCVideo server 1 determines that MCVideo client 1 has requested that MCVideo client 2 from the primary MC system be removed from the on-going ad hoc group call and successfully removed from call according to the procedure defined in clause 7.19.3.1.4.

6.	The MCVideo server 1 determines that MCVideo client 1 has requested that MCVideo client 5 from the partner MC system be invited to an on-going ad hoc group call. The MCVideo server 1 sends the ad hoc group call request to the MCVideo client 5.

7.	The MCVideo client 5 notifies the user about the incoming ad hoc group call.

8.	The MCVideo client 5 accepts the ad hoc group call request and sends the ad hoc group call response to the MCVideo server 1.

9.	The on-going ad hoc group call is updated by adding MCVideo client 5 which is based on the modify participant list provided by the MCVideo client 1.

10.	The MCVideo server 1 determines that MCVideo client 1 has requested that MCVideo client 4 from the partner MC system be removed from the on-going ad hoc group call. The MCVideo server 1 sends the ad hoc group call leave request to the MCVideo client 4.

11.	The MCVideo client 4 notifies the user of the ad hoc group call leave request.

12.	The MCVideo client 4 sends the ad hoc group call leave response to the MCVideo server 1.

13.	The on-going ad hoc group call is updated by removing MCVideo client 4, which is based on the modify participant list provided by the MCVideo client 1.

NOTE:	The ad hoc group call request and response exchanged between MCVideo server 1 of primary MC system and MCVideo client 4/MCVideo client 5 will always traversal through the MCVideo server 2.

14.	The MCVideo server 1 may notify the initiating MCVideo user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCVideo user by the MCVideo server more than once during the call when MCVideo users join or leave the ad hoc group call.

15.	The MCVideo server 1 may notify the authorized participants about the change in the participants list of on-going ad hoc group call.

* * * End of Changes * * * *