3GPP TSG-SA WG6 Meeting #57	S6-233165

Xiamen, China, 9th – 13th October 2023	(revision of S6-232904)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0190 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Changing the criteria during an ongoing MCVideo ad hoc group call (multiple MC systems) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current standard allows to update the participants list of an ongoing adhoc group call by changing the criteria within one MC system. The support when the adhoc group participants are hosted by multiple MC systems is not supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Message flows are extended and the procedure to change the criteria during an ongoing adhoc group call in multiple MC system is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The list of ad hoc group call participants cannot be modified during an ongoing group call by changing the criteria when the participants are in multiple MC systems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 7.19.2.20a, (new) 7.19.2.21a, (new) 10.19.3.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.19.2.20a	Modify ad hoc group call criteria request (MCVideo server – MCVideo server)

Table 7.19.2.20a-1 describes the information flow Modify ad hoc group call criteria request from the MCVideo server to the MCVideo server.

Table 7.19.2.20a-1: Modify ad hoc group call criteria request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call whose participants list needs to be modified |
| Criteria to determine the list of participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCVideo server for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area |

# * * * * Second change * * * *

#### 7.19.2.21a	Modify ad hoc group call criteria response (MCVideo server – MCVideo server)

Table 7.19.2.21a-1 describes the information flow Modify ad hoc group call criteria response from the MCVideo server to the MCVideo server.

Table 7.19.2.21a-1 Modify Ad hoc group call criteria response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group call participants request (success of failure) |

# * * * * Second change * * * *

##### 10.19.3.2.x	Modification of ad hoc group call criteria by an authorized user

Figure 10.19.3.2.x-1 illustrates the modification of ad hoc group call criteria procedure by the initiator of the ad hoc group call between multiple MC systems.

Preconditions:

-	The MCVideo user at the MCVideo client 1 is authorized to modify the criteria.

-	The MCVideo server A and MCVideo server B are aware of the criteria sent by MCVideo client 1 related to the ongoing ad hoc group call.

-	MC system A and MC system B are interconnected.

Figure 10.19.3.2.x-1: Modifying the criteria for determining the participants during an ongoing ad hoc group call between multiple MC systems

1.	An ad hoc group call has been established based on criteria sent by an authorized MCVideo client upon initiating the ad hoc group call.

2.	The MCVideo user at the MCVideo client 1 is authorized and wishes to modify the criteria for determining the list of ad hoc group call participants.

3a/b.	The MCVideo server A receives the modify ad hoc group call criteria request and forwards the request to MCVideo server B at MC system B.

NOTE 1:	If MCVideo server A identifies that the criteria requires to add participants from another MC system then MCVideo server A sends an ad hoc group call request to the MCVideo server in that MC system.

4a/b.	The MCVideo server A receives the ad hoc group call criteria modify response and forwards the response to MCVideo client 1 in MC system A.

5.	MCVideo server B detects that MCVideo client 3 meets the criteria for the ongoing ad hoc group call initiated at MC system A.

5a.	MCVideo server B sends an ad hoc group call add user notification message towards MCVideo server A.

5b.	MCVideo server A sends an ad hoc group call request towards MCVideo client 3.

5c.	The MCVideo user 3 is notified of entering an ongoing ad hoc group call.

5d.	MCVideo client 3 sends an ad hoc group call response towards MCVideo server A.

5e.	MCVideo server B sends an ad hoc group call remove user notification message towards MCVideo server A.

5f.	MCVideo server A sends an ad hoc group call leave request towards MCVideo client 4.

5g.	The MCVideo user 4 is notified of leaving an ongoing ad hoc group call.

5h.	MCVideo client 4 sends an ad hoc group call leave response towards MCVideo server A.

6.	The MCVideo server A adds MCVideo client 3 to the ad hoc group call and removes MCVideo client 4 from the ad hoc group call.

NOTE 2:	If the ad hoc group call is associated with an ad hoc group emergency alert and the change of criteria caused the modification of the ad hoc group call participant list then the ongoing ad hoc group emergency alert is modified accordingly.

# * * * * End of changes * * * *