3GPP TSG-SA WG6 Meeting #57	S6-233172

Xiamen, China 9th – 13th October 2023	(revision of S6-233114)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0193 | rev | 2 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing transmission idle message for MCVideo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In stage 3 in 3GPP TS 24.581, the transmission idle message is described, e.g., in clause 4.1.1.2: “During silence (when no video transmission is ongoing), the transmission control server can send Transmission Idle message to all transmission participants from time to time. The transmission control server sends Transmission Idle message in the beginning of silence.” However, in stage 2 in 3GPP TS 23.281, there is no information about transmission idle message. Therefore, this CR introduces the transmission idle message in stage 2 to make it aligned with stage 3 work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introducing the transmission idle message and its information element - Introducing a procedure describes the usage of transmission idle message - Updating the description of media identifier |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The concept of idle message is not available in MCVideo, such as in MCPTT service, and no alignment with stage 3 work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.1.2.X (new), 7.7.1.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 7.7.1.2.X	Transmission idle

Table 7.7.1.2.X-1 describes the information flow transmission idle from the transmission control server to the transmission control participant(s) during silent period (i.e., when no video transmission is ongoing). This information flow is sent over unicast or broadcast.

Table 7.7.1.2.X-1: Transmission idle

| Information element | Status | Description |
| --- | --- | --- |
| Media identifier | O | Identity of the communication, e.g., by identifying the media flow within a media multiplex, present only if media multiplexing |
| Acknowledgement required | O | Indicates if acknowledgment from the transmission control participant(s) is required |

# * * * * next change * * * *

##### 7.7.1.3.X	Transmission idle during an MCVideo session

During silence, i.e., when no video transmission is ongoing, the transmission control server may send the transmission idle message to all transmission participants from time to time. The transmission control server sends the transmission idle message in the beginning of silence.

Figure 7.7.1.3.X-1 describes the procedure once the transmission idle message is sent from the transmission control server to the transmission control participant(s) during an already established MCVideo session. For simplicity, only three MCVideo UEs are participating in the MCVideo session.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server.

2.	The transmission control is established between the transmission control participants and transmission control server.

3.	The transmission control participant A has been granted the permission to transmit media.

Figure 7.7.1.3.X-1: Transmission idle

1.	The media transmission has ended according to clause 7.7.1.3.6.

2a – 2c.	If no other MCVideo user has permission to transmit the media, then the transmission control server sends the transmission idle message to the transmission control participants.

3.	The transmission control participants may notify the MCVideo users about the transmission idle i.e., silent period.

NOTE:	The transmission idle may be sent multiple times during the MCVideo session.

# * * * * End of changes * * * *