3GPP TSG-SA WG6 Meeting #57	S6-233229

Xiamen, China 9th – 13th October 2023	(revision of S6-232876)

Source:	China Mobile

Title:	Pseudo-CR on key issue of support of providing application layer caller information to callee

Spec:	3GPP TR 23.700-92 v0.1.0

Agenda item:	8.2

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR proposes text for key issue of support of providing application layer caller information to callee. Add some additional information to the caller, e.g. the caller is verified and belonged to a specific organization, verified business card of the organization, may increase the pick-up rate and provide Anti-Fraud Protection to the consumer, i.e. the callees. This KI studies how to support providing application layer caller information to callee in eMMTel Enabler.

2. Reason for Change

Contribution with new text.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.1.0.

* * * First Change * * * *

6.X	Key Issue #X:support of providing application layer caller information to callee

6.X.1	Description

There are several primary business functions that organizations use MMTEL services for, including internal communication in a organization, talking with prospects (sales call), contacting current customers and clients, customer support, and contact centre (or call centre) activities. (Detailed use cases see 3GPP TR 22.873 [10]) In these use cases, the individual caller belongs to a organization may make a call with other party on behalf of his/her organization but uses his/her own caller ID. Add some additional application layer caller information to the caller, e.g. the caller is verified by MNO and belonged to a specific organization, the verified business card of the organization, etc, may increase the pick-up rate and provide Anti-Fraud Protection to the consumer, i.e. the callees. This use case is also highly recommended by GSMA in their white paper GSMA TGY.02[X] addressing the call for business usages.

The service of providing application layer caller information to callee is a value added service on top of the requirement of verification of the caller which is a part of requirement of 3rd party ID and is specified in clause 6.39.3 of 3GPP TS 22.261 [2] as listed below:

-	The 5G network shall provide a means to verify the authenticity of any stored identity information referenced by the call setup to be presented to the called party.

How to support 3rd party ID has been studied by SA WG2 in FS_NG_RTC WID and the result is captured in 3GPP TR 23.700-87 [Y]. There are five solutions were proposed for supporting 3rd party ID, i.e. 3GPP TR 23.700-87 [X]. Out of these, three solutions refer explicitly to STIR/SHAKEN (sol #10, #12, #19) while one solution refers to signing and verification of SIP headers but does not mention STIR/SHAKEN explicitly (sol #11). The conclusion of that KI#3 is that KI#3 will not progress for normative work in Rel-18. In SA WG2 Rel-19 WID FS_NG_RTC_Ph2, there is a WT#3 addressing the 3rd party ID issue:

-	WT-3: Study how to support verified OIP service for 3rd party in IMS session using SA3 work on third party identity authentication as basis.

However, providing application layer caller information to callee can not be supported by Rel-18 work and is not covered by Rel-19 FS_NG_RTC_Ph2 WID. The STIR/SHAKEN works for authenticating and thus proving that a call has not been spoofed, but it does not determine caller intent and affiliation. In the industrial implementation, this use case is usually covered by application layer solutions. Therefore, when enable eMMTel service to application providers/Vertical service provider, providing application layer caller information to callee is needed to be considered as a necessary value added service, i.e. providing application layer caller information to callee is needed to be supported by eMMTel Enabler layer.

This key issue aims to study how to support providing application layer caller information to callee in the eMMTel Enabler layer:

1.	Analyze whether the existing application layer service capabilities can be used by eMMTel Enabler layer to support providing application layer caller information to callee;

NOTE:	Security related aspects should be studied by SA WG3.

2.	How to manage the application layer caller information,  e.g. provide the capability to configure the caller information to the application providers/Vertical service provider, verify the caller information provided by the application providers/Vertical service provider.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[4]	OMA-AD-NGSI-V1_0-20120529-A: "Next Generation Service Interfaces Architecture".

[5]	OMA-TS-REST_NetAPI_ThirdPartyCall-V1_0-20130212-C: "RESTful Network API for Third Party Call".

[6]	OMA-TS-REST_NetAPI_CallNotification-V1_0-20200226-C: "RESTful Network API for Call Notification".

[7]	OMA-TS-REST_NetAPI_AudioCall-V1_0-20200226-C: "RESTful Network API for Audio Call".

[8]	3GPP TS 23.198: "Open Service Access (OSA)".

[9]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[10]	3GPP TR 22.873: "Study on evolution of the IP Multimedia Subsystem (IMS) multimedia telephony service".

[X]	GSMA TGY.02: "Business Voice Calling".

[Y]	3GPP TR 23.700-87: "Study on system architecture enhancement for next generation real time communication".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".