SA WG6 Meeting #57	S6-233238

Xiamen, China, 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0023 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of information flow of PIN status notify |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In PIN status notify, that there are some issues unclear and need to clarification. Firstly, in Table 8.5.9.3.4-1, there are no information under the PIN profiles update. So, it better to add the updated PIN profile under this and refer to the former paragraph. Secondly, for each event ID that in Table 8.5.9.3.4-1, it better to not have the description. Thirdly, due to in some of the procedure, that the PINE (including PEMC or PINE or PEGC) should be notified with the dynamic PIN profile, in order to save the message design, the updated dynamic PIN profile should be added in the PIN status notify message. At last, for PIN modification bullet, only the updated PIN profile is enough, and no need to further describe the PEGC/PEMC information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Updated PIN profile sub-bullet under the PIN profile update in Table 8.5.9.3.4-1. Remove the description of each event ID. Remove the PEGC/PEMC information. Add dynamic PIN profile update to output of PIN status notify. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear of the relationship between updated PIN profile and PEMC/PEGC information. There is no details information under the PIN profiles update. How to update the dynamic PIN profile is FFS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.9.3.4, 8.5.8.2.3, 8.5.9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.5.9.3.4	PIN status notify

Table 8.5.9.3.4-1 describes information elements in the PIN status notify from the PEMC to PEMC/PEGC/PINE/PIN server.

Table 8.5.9.3.4-1: PIN status notify

| Information element | Status | Description |
| --- | --- | --- |
| PIN Status event type | M | Identifies PIN status event types contained in the notification; more than one PIN status event type can be provided. PIN status event types - PIN modification - PINE management (PINE join/leave) - PIN profile update - PIN state (Activated or de-activated) |
| PIN ID | M | Indicate identifier of the PIN that status event related to |
| PINE management | O | Indicates the PINE join/leave the PIN and PINE removed, this IE is present if the PIN status event type is PINE management. |
| PIN modification | O | Indicates the modification of PIN as indicated in section 8.5.10 for example the PEMC/PEGC relocation, this IE is present if the PIN status event type is PIN modification. |
| PIN profiles update | O | Indicates a PIN profile update that indicated in section 8.2.2, this IE is present if the PIN status event type are PIN modification, PINE management, and PIN profile update. |
| PIN state | O | Indicates the state of a PIN. |

* * * Start of Change 2 * * *

8.5.8.2.3	Procedure of PEMC removes the PIN elements from a PIN

The following procedure defines the PIN elements decides to leave the PIN.

Pre-conditions:

1.	The PIN client has already been added into a PIN;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PIN client has been authorized to communicate with the PEMC and already has the application layer connection with PEMC;

Figure 8.5.8.2.3-1: Remove a PIN element from a PIN by PEMC

1.	The PEMC decides to remove the PINE (identified by GPSI, PIN client ID and in certain PIN identified by PIN ID).

2.	The PEMC sends the PIN status notify to the PINE that this PINE has been removed from the PIN that identified by PIN ID.

3-4.	The PEMC sends the PIN status notify to the PEGC and PIN server containing the details of the PIN client that is removed from the PIN. The details of the PINE include PIN client ID, GPSI and etc.

5-7.	The PEMC/PEGC/PIN server updates the dynamic profile information of the PIN to remove the details of the PIN client that is removed from the PIN. The PEGC disables the access control information for this PINE.

* * * Start of Change 3 * * *

8.5.9.2.2	PIN status notify

Figure 8.5.9.2.2-1 illustrates the PIN status notify procedure.

Pre-conditions:

1.	The PINE/PEGC/PIN has subscribed with the PEMC for the PIN status as specified in clause 8.5.9.2.1.

Figure 8.5.9.2.2-1: PIN status notify procedure

1.	When an event occurs at the PEMC that satisfies trigger conditions for notifying (e.g. to provide updated PIN status, for example when a PINE joins into the PIN) a PINE/PEGC/PIN server.

The status information of PIN is updated including the following cases:

-	PIN modification: The PIN modification includes the PEMC/PEGC relocation and the PINE joins into PIN or leaves the PIN;

-	PIN profile updates: The PIN profiles is referred to the section 8.2.2, and the updated PIN profile may be coordinated to other PINEs;

-	PIN state changes: Whenever the PIN state is changed from activated to de-activated or vice-versa;

2.	The PEMC sends an PIN status notify to the PINE/PEGC/PIN server with the updated PIN status information.

The PIN status information notification includes the following parameters:

-	PIN modification: If the PEMC decides the PEMC/PEGC relocation, the PEMC includes the parameters about the newly assigned PEMCs and PEGCs in the PIN status update request, for example, the PEMC/PEGC ID or address, the PEGC information in Table 8.5.2.3.3-1 about access control information and etc;

-	PINE management: If the PEMC decides the PINE to join or leave the PIN, the PEMC includes the parameters about the newly added/removed PINE in the PIN status update request, for example, the PIN client ID or address, the PIN client profile and etc. Also if the PINE is removed from the PIN, the parameters about the removed PINE should be included in the PIN status update request;

-	PIN profile update: If the PIN profile is updated, the PEMC includes the parameters about updated PIN profile in the PIN status update request;

-	PIN state changes: Whenever the PIN state is changed from activated to de-activated or vice-versa;

3.	Upon receiving the notification, the PINE, PEGC or PIN server updates the PIN profile according to the information in the PIN status notify.

* * * End of Change * * *