SA WG6 Meeting #57	S6-233239

Xiamen, China, 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0024 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PIN communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The first change: In Section 8.6.1, there exists the following sentence: For PEGC related procedure, the PIN element sends PIN Communication Request to the PEMC. The PEMC sends Create/Update/Remove Communication Request (PIN ID, Packet filters, requested QoS) to the PEGC. The PEGC configures the local rule accordingly, or according to the Packet filters, the PEGC may initiate PDU Session Modification with the Packet filters and requested QoS towards 5G system in order to make 5GC configure the N4 rules for UPF(s). It describes that the PINE sends the PIN Communication Request to the PEMC. The PEMC sends Create/Update/Remove Communication Request to PEGC. But in the Figure 8.6.2.2-1, in step 1b, the PINE directly sends the PIN Communication Request to the PEGC. So there exists the conflict that whether the PINE can directly send request to PEGC. It is better to allow the PEMC to control whether to allow the PINE to communicate with other PINE via PEGC directly or via 5GS. Also in downlink, that the PEMC controls the traffic whether to deliver to certain PINEs. The second change: In section 8.6.1: For a certain PINE, it has the subscription whether the PINE has the permission to use 5GS to communicate. But the mechanism of subscription is not specified in current specification. So, it is better to allow the PEMC to control whether the PINEs are allowed to communicate with other PINEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove step 2 and step 3, and replace the step 2 as PIN status notify. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear whether the PINE can directly send request to PEGC to trigger the PDU session modification. It is unclear whether the PINE can subscribe the permission of PINE to use 5GS to communicate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.1, 8.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.6.1	General

For a certain PINE in PIN, the PEMC controls whether to allow or forbid the PINEs to communicate with other PINEs via PEGC directly or communicate with other PINEs via PEGC by 5GS. Also, the PEMC controls whether to allow or forbid the traffic from network sides to deliver to certain PINE.

The PIN client within a PIN can communicate with other devices, services and applications within the same PIN. Furthermore, PIN client can connect the 5G Network via a PEGC. Also, as a feature, some of the PIN client in PIN has the permissions that they can communicate with other UE or application outside of PIN with the help of 5GC.

-	There are two methods to enable the PIN with 5GS communication:

-	Establish QoS for PINE with AF support.

-	PEGC triggers PDU session establishment/modification for PINE.

For the AF related procedure, the AF trigger the QoS create/modification procedure with parameters of Packet filters, DN specific ID, to request the 5GS to arrange resource for PIN.

For PEGC related procedure, the PIN element sends PIN Communication Request to the PEMC and the PEMC checks whether to allow or forbid the PINEs to communicate with other PINEs via PEGC directly or communicate with other PINEs via PEGC by 5GS. If the alloewd, the PEMC sends Create/Update/Remove Communication Request (PIN ID, Packet filters, requested QoS) to the PEGC. Or, the PEMC directly send Create/Update/Remove Communication Request (PIN ID, Packet filters, requested QoS) to the PEGC. The PEGC configures the local rule accordingly, or according to the Packet filters, the PEGC may initiate PDU Session Modification with the Packet filters and requested QoS towards 5G system in order to make 5GC configure the N4 rules for UPF(s).

* * * Start of Change 2 * * *

8.6.2.2	Procedures of PIN communication via 5GS triggered by PEGC

Pre-conditions:

1.	The PINE already has application layer connection with PEGC;

2.	The UE Identifier or PIN client Identifier is available for PINE;

3.	The PIN client has been authorized to communicate with PEMC;

4.	The PINE has the subscription that it can communicate via 5GS;

5.	The PINE has already received the IP address of other PINE.

6.	The communication between PEMC and PEGC is available.

7.	The PIN is in the activated state.

Figure 8.6.2.2-1: Procedures of PIN communication via 5GS triggered by PEGC

The PINE may have the traffic to send via 5GS communication.

1a-1b.	The PINE sends PIN Communication Create/Update/Remove Request (PIN ID, MAC address/IP address, Traffic descriptors, Packet filters, requested QoS) to the PEMC in step 1a, and the PEMC checks whether to allow or forbid the PINEs to communicate with other PINEs via PEGC directly or communicate with other PINEs via PEGC by 5GS. If permitted, PEMC sends PIN Communication Create/Update/Remove Request to PEGC in step 1b. If not permitted, PEMC directly performs step 3b to indicate failure in PIN Communication Create/Update/Remove response.

After the PEGC receives the PIN Communication Create/Update/Remove Request, the PEGC configures the local rule accordingly.

2.	[Optional] According to the Packet filters, the PEGC may initiate PDU Session Modification with the Packet filters and requested QoS towards 5G system in order to make 5GC configure the N4 rules for UPF(s).

NOTE:	 How to the PEGC interacts with 5GS to initiate PDU Session Modification is referred to section 4.3.3.2 of TS 23.502[5].

3a.	The PEGC sends PIN Communication Create/Update/Remove Response to the PEMC.

3b.	If the request is triggered by PINE, step 3b is performed. The PEMC sends PIN Communication Create/Update/Remove Response to PINE.

After receiving successful response, the PINE sends data through the PEGC to the 5GS using the PDU session modified in step 2.

* * * End of Change * * *