SA WG6 Meeting #57	S6-233241

Xiamen, China, 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0026 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PINE registration indirectly to PIN server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Section 8.4.2.2.2, it seems that the PINE registration request between PINE and PEMC is the same as between PEMC and PIN server. Due to the PEMC represents the PINE/PEGC to registration, so, step 2 that the PEMC may contain multiple PINEs/PEGCs and the PEMC may replace PINE/PEGC to be the message sender. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new table to clarify the PEMC to be the message sender to distinguish the represent registration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to PEMC to represent PINE to do the registration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.4.2.3.2	PINE Registration request

Table 8.4.2.3.2-1 describes information elements in the PINE Registration request from the PINE (including PEMC, PEGC, PINEs) to the PIN server.

Table 8.4.2.3.2-1: PINE Registration request (PINE/PEGC to PIN server, or PEMC represents PINE/PEGC to PIN server)

| Information element | Status | Description |
| --- | --- | --- |
| UE Identifier | M | The identifier of the hosting UE (i.e. GPSI or identity token) of PINE/ PEMC/PEGC |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| Indication of representation registration | O | If PEMC represents PINE/PEGC to perform registration to PIN server, this indication is included. |
| Lists of PINEs/PEGCs |  |  |
| > MAC address | O | MAC address of the requested PINEs. |
| > Vendor name | O | The vendor name of the PINE |
| > Device description | O | Description of the device. |
| > PINE Address | M | The IP address of PINE, if available. |
| > Port number | M | Port number of PIN client on PINE/PEMC/PEGC to support PIN enabler layer communication. The port number is the port used by a PINE to expose a service within the PIN. |
| > PINE Capabilities | O | Identify whether the PINE is capable of becoming a PEMC, a PEGC or both. |
| > > Maximum number of PINEs (see NOTE) | O | Indicates the maximum number of PINE that can be managed by the PEMC or PEGC |
| NOTE: Only present if PINE Capabilities is present. |  |  |

* * * End of Change * * *