SA WG6 Meeting #57	S6-233242

Xiamen, China, 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0028 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of service continuity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Section 8.9.2.2, there exists some issues that unclear for changing the access to 5GS: It says: This procedure solves the situation, that PINE A has application layer communication with PINE B via PEGC. But when the communication via PEGC is not viable, for example, the PINB moves out of the coverage of PEGC 1, only newly establish the 5GS communication to connect PINE B is the potential way. But the problem is, when the PINE B moves out of the coverage of PEGC 1, and if PINE B is not able to trigger the PDU session modification procedure, that the PINE B may not be discovered or addressed. One possible way to solve the problem is if PINE B moves into the PEGC 2 coverage, and the PEGC 2 triggers the PDU session modification procedure to establish the PDU session. And the PINE A and PINE B can communicate via PDU session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification of how to PINE 2 to establish the PDU session towards PINE 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to the PINE 2 to discover the PINE 1 if PINE 2 is out of the coverage of PEGC 1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.9.2.2	Service continuity in changing access to 5GS

This procedure solves the situation, that PINE 1 has application layer communication with PINE 2 via PEGC 1. But when the communication via PEGC 1 is not viable, for example, the PINE 2 moves out of the coverage of PEGC 1 1, only PINE 2 triggers to establish the 5GS communication to connect to PINE 1 via PEGC 2 is the potential way.

Figure 8.9.2.2-1 illustrates the service continuity to change application layer communication to communication via PEGC + 5GS based on request/response model.

Pre-conditions:

1.	The PEMC in a PIN has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The PIN has already been created and a PIN ID is distributed by PIN server;

3.	The PINE 1 has the subscription that can communicate with PINE 2 via 5GS;

4.	The PINE 2 doesn’t have the 3GPP capability and require support of a PEGC to access the 3GPP network.

Figure 8.9.2.2-1: Change application layer communication to communication via 5GS

1.	The PINE 1 has application layer communication with PINE 2 via PEGC 1.

2.	The PINE 2 moves out of PEGC 1 and moves into the coverage of PEGC 2. The PINE 2 connects to the network that provided by PEGC 2 via access control information.

3.	The PEGC 2 receives the related parameter (PIN ID, MAC address/IP address, Traffic descriptors, Packet filters, requested QoS) as indicated in section 8.6.2.2 from PINE 2 or PEMC.

4.	According to the Packet filters, the PEGC 2 may initiate PDU Session Modification as indicated in section 4.3.3.2 of TS 23.502[5] with the Packet filters and requested QoS towards 5G system in order to make 5GC configure the N4 rules for UPF(s), referred to section 8.6.2.2.

5.	The PINE 2 communicates with PINE 1 via 5GS by PEGC 2.

* * * End of Change * * *