3GPP TSG-SA WG6 Meeting #57	S6-233247

Xiamen, China 9th – 13th October 2023	(revision of S6-233006)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0237 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update subscription service operation for SS_LocationInfoEvent |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, subscriptions for location information events can be created and deleted. The issue with this approach is that when a VAL server wants to update a subscription (e.g. to change the time between consecutive reports or the Location QoS), it has to delete the old subscription and create a new one. Depending on the order these operations are invoked, the VAL server may either receive duplicated notifications (create-new followed by delete-old) or may miss notifications (delete-old followed by create-new). Also, the CRUD (Create-Read-Update-Delete) paradigm used in the RESTful mapping in stage 3 does require to allow an update operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable a location information subscription update procedure triggered by a VAL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 3 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.5, 9.3.2.6, 9.3.2.7a (New), 9.4.1, 9.4.3.1, 9.4.3.5 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: capitalization problems, complete messages names in the figure, correction of the name of the service api operation, reuse of the subscription request and response messages to update a subscription |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 9.3.2.5	Location information subscription request

Table 9.3.2.5-1 describes the information flow from the VAL server or location management client to the location management server for location information subscription request or for updating an existing location information subscription.

Table 9.3.2.5-1: Location information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE (NOTE 3) |
| Identities list | M | List of VAL users or VAL UEs whose location information is requested. |
| VAL service ID | O | Identity of the VAL service for which the location information is subscribed. |
| Time between consecutive reports | M | It indicates the interval time between consecutive reports |
| Supplementary location information indication | O | Indicates that supplementary location information is required. |
| Location QoS | O (NOTE 2). | Definition of the location Quality of Service for which the location information is requested (see NOTE 1). |
| NOTE 1: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [50] and the clause 6.1.6.2.13 of TS 29.572 [51]. NOTE 2: The element is only applicable for the information flow from the VAL server to the location management server. NOTE 3: This information element shall not be updated via location information subscription update procedure in clause 9.3.2.7a. |  |  |

* * * * Next Change * * * *

#### 9.3.2.6	Location information subscription response

Table 9.3.2.6-1 describes the information flow from the location management server to the VAL server or location management client for location information subscription response or location information subscription update response.

Table 9.3.2.6-1: Location information subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE |
| Subscription status | M | It indicates the subscription result |

* * * * Next Change * * * *

#### 9.3.2.7a	Location information subscription update procedure

Figure 9.3.2.7a-1 illustrates the high level procedure for allowing a VAL server update an existing subscription about location information at the location management server.

Pre-conditions:

-	the VAL server has an active location information subscription at the location management server.

Figure 9.3.2.7a-1: Location information subscription update procedure

1.	When the VAL server decides to update an active location information subscription, it sends a location information subscription update request to the location management server including the information specified in table 9.3.2.5-1.

2.	The location management server shall check if the VAL server is authorized to update the subscription.

3.	If the VAL server is authorized to update the subscription, the location management server updates the location information subscription.

4.	The location management server sends a location information subscription update response to the VAL server including the information as specified in table 9.3.2.6-1.

* * * * Next Change * * * *

## 9.4	SEAL APIs for location management

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Update_Location_Info_Subscription | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
|  | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |

* * * * Next Change * * * *

#### 9.4.3.1	General

API description: This API enables the VAL server to subscribe, receive and unsubscribe the UEs location information from the location management server over LM-S, as well as modify an active subscription.

* * * * Next Change * * * *

#### 9.4.3.5	Update_Location_Info_Subscription

API operation name: Update_Location_Info_Subscription

Description: Updating an active subscription about the UEs location information.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.5

Outputs: See subclause 9.3.2.6

See subclause 9.3.2.7a for the details of usage of this API operation.

* * * * End of Changes * * * *