3GPP TSG-SA WG6 Meeting #57	S6-233257

Xiamen, China 9th – 13rd October 2023	(revision of S6-232958)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0020 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on media delivery functionality in SEALDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As a general data delivery enabler, SEALDD should support the data transmission for all traffic types, including media type (e.g. media streaming, real-time media), this discussion paper mainly focuses on these issues: The architecture relationship between SEALDD and SA4 in media communication The specific component of SEALDD compared with media functionality defined in SA4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper adds the architreture mapping and the related description to solve how the SEALDD to support the media type traffic delivery. The functionality/protocol defined in SA4 can be reused and further enchanced by SEALDD, the SEALDD server can utilize both 5GMS/RTC AF and 5GMS RTC AS. The 5G transmission adaptation component is added in SEALDD, to facilitate the specific optimization for XR application provided by 5G network |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relationship between SEALDD and SA4 is unclear regarding of media delivery |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new), 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

Annex X (Informative):
Media data delivery in SEALDD

X.1	General

The SEALDD layer provides a general functionality to support data storage and data delivery by introducing 5G interaction (N5/N33 in control plane, N6 in user plane), including application data storage and management, application signaling and data transmission, E2E URLCC transmission, IoT message transmission (i.e. MSGin5G), etc. As a general data delivery enabler, SEALDD should support the data transmission for all traffic type, including media type (e.g. media streaming, real-time media).

X.2	SEALDD deployment for media delivery

For media delivery functionality in SEALDD, as shown in Figure X.2-1, the functionality/protocol defined in SA4 can be reused and further enhanced by SEALDD for supporting media type traffic, 5GMS/RTC AF and 5GMS RTC AS in 3GPP TS 26.501 [y] and 3GPP TS 26.506 [z] is utilized in SEALDD server to support the general media type delivery. The 5G transmission adaptation component is added in SEALDD, to facilitate the specific optimization for XR application provided by 5G network.

NOTE: The functionality about 5G transmission adaptation component for media type will be further detailed based on the other progresses (e.g. application enabler for XR Services).

Figure x.2-1 SEALDD deployment for media delivery

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[7]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[8]	3GPP TS 23.548: "5G System Enhancements for Edge Computing ".

[9]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2".

[10]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[11]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[12]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[y]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[z]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".