SA WG6 Meeting #57	S6-233279

Xiamen, China, 9th – 13th October 2023	(revision of S6-233242)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0029 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Service Switch |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Section 8.7.2.2, there exists some issues that should be fixed: Firstly, when the PINE B is selected to receive the switching traffic from PINE A, before the service switch, the PINE B should be notified by the PIN server about the service switch. Also, on the application layer perspective, the PINE B may also open the same application or under the same PEGC. At least for current description, no PINE B’s behaviour is not right. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification of PINE B’s behaviour to support service switch. During PINE registration, service info is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to PIN server to know whether the certain PINE supports to provide the service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.2.2, 8.7.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.7.2.2	Service switch in a PIN without PIN server support

Figure 8.7.2.2-1: PIN Service Switch internal PIN

In Figure 8.7.2.2-1, it describes the service switch scenario supported internal PIN.

The PINE A has the application communication with application server. And when the PINE A decides to select other alternative PINE B to apply the traffic flow, the PINE A firstly should discover a PIN and join in. And then, in the PIN, there should exist the PINE that can be hosted with the same service type as PINE A, for example, the video flow, music flow or game flow.

The PINE A can send the request to PEMC to determine the PINE B to host the application traffic. After the determination, the PEMC sends the endpoint information to PINE A and the PINE A can offload the traffic either directly to PINE B or via PEGC.

Pre-conditions:

1.	The UE Identifier or PIN client Identifier of PINE A or PINE B is available;

2.	The PIN client in PINE A or PINE B has been authorized to communicate with the PEMC;

3.	The PIN client in PINE B can provide the same PIN service as PINE A’s traffic flow.

Figure 8.7.2.2-2: Service switch procedure internal PIN

0.	The PINE A has application layer communication with application server. And the PINE A decides to do the service switch to other PINEs. And the PINE A has already been in a PIN.

1.	The PINE A trigger the PIN service discovery request towards PEMC. This request carries the list of services the PINE A wants to consume.

2.	Upon receiving the request, the PEMC performs an authorization check to verify whether the PINE A has authorization to perform the operation.

3.	The PEMC provides the list of PINE endpoint(s), application client endpoint(s) information that are offering the requested services to PINE A in PIN service discovery response. The PIN service can be represented by service type that PINE provides or the application client on PINE. If the request fails, the PEMC should give the failure response to indicates the cause of request failure.

4.	The PINE A selects PINE B from the list of PINEs provided in the PIN service discovery response.

The PINE A sends the PIN Management Service Switch Configure request to the selected PINE B including: PIN ID, requestor ID, service information (e.g. PIN service type, PIN service feature) before performing service switch.

5.	Upon receiving the request from PINE A, PINE B determines whether it can accept the service switch request and sends the PIN Management Service Switch Configure response to PINE A.

If the PIN Management Service Switch Configure response indicates failure, the PINE A may consider other PINEs in the list of PINEs provided in the PIN service discovery response received in step 3 and may perform step 4 again.

If the PIN Management Service Switch Configure response indicates success, the PINE A maintains the service towards AS and proceeds with switching the service traffic to PINE B.

6.	The PINE A switches the traffic flow to PINE B via direct communication or via PEGC.

* * * Start of Change 2 * * *

#### 8.7.3.6 	PIN management service switch configure request

Table 8.7.3.6-1 describes information elements for the PIN management service switch configure request that is sent from the PIN management client to the PIN gateway client and the target PIN client.

Table 8.7.3.6-1: PIN management service switch configure request

| Information element | Status | Description |
| --- | --- | --- |
| PINE identifier | M | Requestor identifier. |
| Security credentials | M | Security credentials of the PIN management client. |
| PIN identifier | M | Identifier of the PIN. |
| Application client identifier | M | Identifier of the application client. |
| Application server identifier | M | Identifier of the application server. |
| PIN gateway client identifier (see NOTE 1) | O | Identifier of the PIN gateway client. |
| Application session identifier | M | Identifier of the application session. |
| Application session descriptor | O | Descriptor of application traffic flows (e.g., IP 4 tuple) |
| Target PIN client identifier (see NOTE 2) | O | Target PIN client identifier. |
| Service information (see NOTE 3) | O | PIN service type and PIN service feature |
| NOTE 1: The IE is present if the request is sent to the target PIN client. NOTE 2: The IE is present if the request is sent to the PIN gateway client. NOTE 3: The IE is present if the request is sent to the PINE. |  |  |

* * * End of Change * * *