3GPP TSG-SA WG6 Meeting #57	S6-233306

Xiamen, China 9th – 13th October 2023	(revision of S6-233060)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0513 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Registrar EES endpoint missing in common EAS procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Observation 1: The EES includes the “EES endpoint where the discovered EAS is registered” in the EAS Discovery response as indicated in clause 8.5.2.2: “If the discovered EAS is registered to another EES, then the EES endpoint of the EES where the discovered EAS is registered is also included in the response message.” Observation 2: The announcing EES endpoint is not included in Common EAS announcement (clause 8.19). Conclusion: The “EES endpoint where the discovered EAS is registered” cannot be provided as described as in Observation 1 when the procedure in Observation 2 is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the “EES endpoint where the discovered EAS is registered” (e.g., announcing EES to Common EAS announcement (clause 8.19) procedure and information flow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EAS Discovery of a common EAS will not include “EES endpoint. where the discovered EAS is registered” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.19.2, 8.19.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.19.2	Procedure

Pre-conditions:

1.	The EES may be pre-configured with the EES information of other EES(s), which belongs to the same EDN; or

2.	The EES may receive the list of EES(s) information (e.g. address information) from the EEC during the EAS information provisioning procedure.

Figure 8.19.2-1: Announce common EAS procedure

1.	The announcing EES sends Announce common EAS request message to the receiving EES. The request message includes the requestor identifier [EESID], the security credentials, the selected common EAS information, the announcing EES endpoint, and the Application group ID.

2.	Upon receiving the request from the announcing EES, the receiving EES checks if the announcing EES is authorized to provide the selected common EAS. The receiving EES stores the received selected EAS information and announcing EES endpoint along with the Application group ID.

3.	If the processing of the request was successful, the receiving EES sends Announce common EAS response to the announcing EES indicating a successful status; otherwise, the receiving EES shall indicate a failure status and include appropriate reasons.

* * * Next Change * * * *

#### 8.19.3.2	Announce common EAS request

Table 8.19.3.2-1 describes the information elements for Announce common EAS request from the announcing EES to the receiving EES.

Table 8.19.3.2-1: Announce common EAS request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the announcing EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Selected common EAS ID | M | The identifier of the Selected Common EAS |
| Selected common EAS endpoint | M | The endpoint of the Selected Common EAS |
| EES endpoint | M | The endpoint address (e.g., URI, IP address) of the announcing EES. |
| Application Group ID | M | Application group identifier as defined in 7.2.11. |

* * * End of Change(s) * * * *