3GPP TSG-SA WG6 Meeting #57	S6-233360

Xiamen, China 9th – 13rd October 2023	(revision of S6-232953)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0502 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on default instantiation behaviour for EAS discovery subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the EAS discovery subscription procedure in clause 8.5.2.3, if the subscribed event is “EAS availability change”, the EES can identify the required EAS based on the provided EAS discovery filter from EEC. And the EES can trigger the ECSP management system to perform the dynamic EAS instantiation if the EAS discovery filter cannot be matched regarding of the current registered or instantiated EAS. Therefore, the dynamic instantiation behavior for EAS discovery subscription behavior is consistent with EAS discovery request. That is, without the EAS instantiation triggering indication, the EES may trigger ECSP management system to instantiate EAS if the EAS discovery filter cannot be matched. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to align the default EAS instantiation behaviour between EAS discovery subscription and EAS discovery request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EAS instantiation behavior in EES is not aligned and conflict between EAS discovery subscription and EAS discovery request. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2.3.2, 8.5.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.5.2.3	Subscribe-notify model

8.5.2.3.1	General

Clause 8.5.2.3.2 and clause 8.5.2.3.3 together illustrate the EAS discovery procedure based on Subscribe/Notify model.

Clause 8.5.2.3.4 illustrates the EAS discovery update procedure.

Clause 8.5.2.3.5 illustrates the EAS discovery unsubscribe procedure.

8.5.2.3.2	Subscribe

Figure 8.5.2.3.2-1 illustrates the EAS discovery subscription procedure between the EEC and the EES. This subscription enables EES to inform EEC of various EAS discovery related events of interest to EEC (e.g. EAS discovery notification and EAS dynamic information).

Pre-conditions:

1.	The EEC has received information (e.g. URI, IP address) related to the EES;

2.	The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.11;

3.	The EES is configured with ECSP's policy for EAS discovery; and

4.	The EEC has optionally acquired a Notification Target Address to be used in its subscriptions to notifications.

NOTE 1:	Details of ECSP's policy are out of scope.

NOTE 2:	How the EEC acquires the notification target address or a notification channel URI to receive the notifications is out of scope of this release. The notification target address can terminate at the EEC (e.g. in an IoT device) if the deployment supports EEC reachability, or it can terminate at a push notification service. Details of the push notification service are out of scope of this release.

Figure 8.5.2.3.2-1: EAS discovery subscription

1.	The EEC sends an EAS discovery subscription request to the EES. The EAS discovery subscription request includes the EECID along with the security credentials, Event ID, and may include EAS discovery filters and EAS dynamic information filters to subscribe to information about particular EAS(s) or a category of EASs (e.g. gaming applications) or dynamic information about EAS(s).

If the application triggering is supported and required by the EEC, the EEC may include the EEC Triggering request information element instead of the Notification Target Address in the request message.

2.	Upon receiving the request from the EEC, the EES checks if the EEC is authorized to subscribe for information of the requested EAS(s). The authorization check may apply to an individual EAS, a category of EASs or to the EDN, i.e. to all the EASs. The EES may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.10.3. If the request is authorized, the EES creates and stores the subscription for EAS discovery.

3.	If the processing of the request was successful, the EES sends an EAS discovery subscription response to the EEC, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the EEC shall send an EAS discovery subscription update request prior to the expiration time. If an EAS discovery subscription update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly unsubscribed.

In the case of subscription to an EAS availability change event, if there is no instantiated EAS that matches the requested EAS discovery filters and such EAS is instantiable based on the information about instantiable EASs, the request is treated as successful. If the EEC indicates EAS Instantiation Triggering in the EAS discovery subscription request and the EAS instantiation is not in progress for the requested EASID, the EES may trigger dynamic instantiation of the EAS as specified in the clause 8.12; otherwise, the EES does not trigger the EAS instantiation. If the EEC indicates instantiable EAS request indication in the EAS discovery subscription request, the EES shall not trigger the EAS instantiation and may determine Instantiable EAS Information for EAS(s) that are instantiable but not yet instantiated, Instantiable EAS Information is provided in the EAS Discovery notification.

NOTE 3:	Without any indication from EEC (either EAS instantiation triggering indication or instantiable EAS request indication), the EES handling is as per R17 procedure.

In the case of subscription to an EAS dynamic information change event, if the EES is unable to determine the instantiated EAS information using the inputs in the EAS discovery subscription request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery subscription request and respond with an appropriate failure cause.

If the EEC is not registered with the EES, and ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall include an appropriate failure cause in the EAS discovery response indicating that EEC registration is required.

If the EAS discovery subscription request fails, the EEC may resend the EAS discovery subscription request again, taking into account the received failure cause. If the failure cause indicated that EEC registration is required, the EEC shall perform an EEC registration before resending the EAS discovery subscription request.

8.5.2.3.3	Notify

Figure 8.5.2.3.3-1 illustrates the EAS discovery notification procedure between the EEC and the EES.

Pre-conditions:

1.	The EEC has subscribed with the EES for the EAS discovery information as specified in clause 8.5.2.3.2.

Figure 8.5.2.3.3-1: EAS discovery notification

1.	An event occurs at the EES that satisfies trigger conditions for notifying (e.g. to provide EAS discovery information or EAS dynamic information) a subscribed EEC. If UE's location information is not already available, the EES obtains the UE location by utilizing the capabilities of the 3GPP core network as specified in clause 8.10.3. If EAS discovery filters were provided by the EEC during subscription creation, the EES identifies the EAS(s) based on the provided EAS discovery filters and the UE location.

For EAS dynamic information change event, the EES, based on local policy, may use received EAS endpoints from EEC or all registered EAS endpoints to get edge load analytics information from ADAES services (e.g. as specified in clause 8.8 of TS 23.436 [27]) to monitor EAS service status like EAS status and EAS schedule.

When the bundle EAS information is provided, then;

-	If EAS bundle identifier was provided, the EES identifies all or part of the EAS(s) associated with the same EAS bundle identifier.

-	If a list of EASIDs is provided, the EES identifies the EASs which are all or part of the EAS bundle.

If the EEC indicates that service continuity support is required, the EES shall take the indication which ACR scenarios are supported by the AC and the EEC and which of these are preferred by the AC into consideration.

If EAS discovery filters were not provided, then:

-	if available, the EES identifies the EAS(s) based on the UE-specific service information at the EES and the UE location;

-	EES identifies the EAS(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 1:	Details of the UE-specific service information and how it is available at the EES is out of scope.

NOTE 2:	Both steps are evaluated prior to sending a response.

If the UE is located outside the Geographical or Topological Service Area of an EAS, then the EES shall not include this EAS in the EAS discovery notification.

If the application triggering is supported and required by the EEC as indicated in EEC Triggering Request IE of the EAS Discovery Subscription Request, then the EES performs the EEC triggering service as described in the clause 8.16.1 and skips the step 2.

2.	The EES sends an EAS discovery notification to the EEC with the EAS information determined in step 1.

Editor's note: The usage of received analytics information by EEC in EAS discovery notification and EAS discovery response is FFS.

Upon receiving the EAS discovery notification, if the EEC selects an EAS which is instantiable but not yet instantiated (i.e. an EAS profile is not provided), the EEC sends the EAS information provisioning request indicating the selected EASID as in clause 8.15.

* * * Next Change * * * *

8.5.3.4	EAS discovery subscription request

Table 8.5.3.4-1 describes the information elements for EAS discovery subscription request from the EEC to the EES.

Table 8.5.3.4-1: EAS discovery subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Event ID | M | Event ID: - EAS availability change - EAS dynamic information change |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Notification Target Address (NOTE 1) | O | The Notification target address (e.g. URL) where the notifications destined for the EEC should be sent to. |
| EAS discovery filters | O | Set of characteristics to determine matching EASs (as detailed in Table 8.5.3.2-2). Applicable for "EAS availability change" event |
| EAS dynamic information filters | O | List of dynamic information changes (as detailed in Table 8.5.3.4-2) about EAS, the EEC is interested in. Applicable for "EAS dynamic information change" event |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| EAS Instantiation Triggering Indication (NOTE 2) | O | Indicates to the EES that EAS instantiation triggering may be performed for the current request if needed. |
| Instantiable EAS request indication (NOTE 2) | O | Indicates to the EES that instantiable EAS information IE is requested to be provided in EAS discovery notification |
| EEC Triggering request (NOTE 1) | O | Indicates that EEC Triggering is requested |
| NOTE 1: One of them may be included in the request message. NOTE 2: One of them may be included in the request message. |  |  |

Table 8.5.3.4-2: EAS dynamic information filters

| Information element | Status | Description |
| --- | --- | --- |
| List of dynamic information filters | M | List of EAS dynamic information required by the EEC per EAS. |
| > EASID | M | Identifier of the EAS |
| > ACIDs | O | Flag to notify change in list of ACIDs served by the EAS |
| > EAS Description | O | Flag to notify change in description of the. EAS. |
| > EAS Endpoint | O | Flag to notify change in EAS endpoint It may also include one or more EAS endpoints to be monitored by the EES. If no EAS endpoint is provided, all EASs registered in the EES will be monitored. |
| > EAS Features | O | Flag to notify any change in features provided by the EAS |
| > EAS Schedule | O | Flag to notify change in availability schedule of the EAS (e.g. time windows) |
| > EAS Service Area | O | Flag to notify change in geographical service area that the EAS serves |
| > EAS Service KPIs | O | Flag to notify change in characteristics of the EAS. |
| > EAS Status | O | Flag to notify change in the status of the EAS (e.g. enabled, disabled, etc.) |
| > Service continuity support | O | Flag to notify change in EAS support for service continuity. |

* * * Next Change * * * *

8.5.3.6	EAS discovery notification

Table 8.5.3.6-1 describes the information elements for EAS discovery notification from the EES to the EEC.

Table 8.5.3.6-1: EAS discovery notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the EES for the request |
| Event ID | M | Either EAS availability change or EAS dynamic information change |
| EAS list | O | List of discovered EAS profiles. Each element includes the information described below. |
| > EAS profile | M | Profile of the EAS. Each element is described in clause 8.2.4 |
| > Lifetime | O | Time interval or duration during which the information elements in the EAS profile is valid and supposed to be cached in the EEC (e.g. time-to-live value for an EAS Endpoint) |
| > Analytics information | O | Analytics information for EAS. |
| >> Statistical data | O | Indicates the statistical analytics data (e.g. number of times the client received expected performance from the EAS). Applicable for "EAS availability change" event. |
| >> Predictive data | O | Indicates predictive analytics data for EAS service status (e.g. EAS schedule, EAS status) change. Applicable for "EAS dynamic information change" event. |
| Instantiable EAS information (NOTE 1) | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet) |
| > Instantiation criteria (NOTE 2) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| NOTE 1: “Instantiable EAS information” can be provided to EEC, if the “instantiable EAS request indication” is provided in EAS discovery subscription request. NOTE 2: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". |  |  |