3GPP TSG-SA WG6 Meeting #58	S6-233446

Chicago, US, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0181 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of EN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The 2nd Editor’s Note asks if steps 2 and 3 need to be kept or corrected. In the present CR steps 2 and 3 are kept. However, no API has been specified for these protocol steps. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The 2nd Editor’s Note is removed an an API definition is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Editor’s Note remains unresolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.4.2, 9.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is alternative 2; alternative 1 is presented in CR 0180 in S6-233445. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * First Change * * *

#### 8.7.4.2	Message delivery from UE to group

Figure 8.7.4.2-1 shows the MSGin5G Group messaging procedure in which MSGin5G Client (both IMS and non-IMS UE) sends a message to a group.

Editor's note:	Solution for the group hosted on the legacy side where MSGin5G Client is one of the group members is FFS.

Pre-conditions:

1.	An MSGin5G Group is created by following group management SEAL service procedures as specified in 3GPP TS 23.434 [5].

2.	All participants in the MSGin5G Group may get the Group information i.e. the Group Service ID.

3.	The MSGin5G Server has a copy of the group profile with all the group members by using Group information query specified in 3GPP TS 23.434 [5].

Figure 8.7.4.2-1: Group messaging in MSGin5G Service

1.	The MSGin5G Client 1 sends a message to a group as specified in clause 8.3.2 with following clarifications:

a)	The MSGin5G message request includes Originating UE Service ID, Recipient Group Service ID and Message ID information elements from Table 8.3.2-1. The MSGin5G message request may include Delivery status required, Application ID, Payload and Priority type information elements from Table 8.3.2-1.

2.	Upon receiving the MSGin5G message request to send the group message, the MSGin5G Server may send the message to the Application Server based on service ID present in the received MSGin5G message request (e.g. to log application specific message or for analytics) using the API request as specified in clause 9.x. Otherwise go to step 4.

a)	Upon receiving the MSGin5G message request, the Application Server validates the message and if the message is not valid, the Application Server sends MSGin5G message response with delivery status set as Reject to the MSGin5G Server per API response message as specified in clause 9.x. Otherwise, go to step 3.

b)	The MSGin5G Server sends the MSGin5G message response with delivery status set as Reject to the MSGin5G Client 1. The information elements defined in Table 8.3.2-3 are included in the response. Following procedures will be skipped.

3.	The Application Server initiates to send message to all group members and sends the MSGin5G message request to the MSGin5G Server.

4	Upon receiving the MSGin5G message request, if the MSGin5G Server determines the MSGin5G Client-1 is authorized to send the group message, the MSGin5G Server resolves the group ID to determine the members of that group, based on the information from the group management server as specified in 3GPP TS 23.434 [5].

NOTE:	If the originating UE is member of the group, the originating UE is not included as recipient of the group message.

5.	The MSGin5G Server sends the message to all participants of the group by their UE Service ID. The MSGin5G message request includes Originating UE Service ID, Recipient Group ID, Recipient UE Service ID, Message ID, Payload information elements from Table 8.3.3-1. The MSGin5G message request may include Delivery status required, Application ID and Priority type information elements from Table 8.3.3-1. The MSGin5G Server routes, using the procedures in clause 8.3.3, the message to:

a)	a MSGin5G UE,

b)	a Legacy 3GPP UE,

c)	a Non-3GPP UE.

NOTE:	Steps 5 a), 5 b) and 5 c) can happen in parallel and in any order.

5.	Upon receiving the group message, if message delivery status report is requested and if supported by target message client, the MSGin5G Client or Legacy 3GPP UE or Non-3GPP message client sends the message delivery status report to originator MSGin5G Client 1 as specified in clause 8.2.4 and 8.3.5.

* * Next Change * * *

## 9.x	APIs provided by the Application Server

### 9.x.1	Mas API

#### 9.x.1.1	General

API description: This API enables the MSGin5G Server to deliver MSGin5G messages to the Application Server.

#### 9.x.1.2	Send Message operation

API operation name: Forward MSGin5G_Message

Description: Forward an MSGin5G message to the Application Server.

Known Consumers: M5S.

Inputs: Refer clause 8.3.3

Outputs: The request to MSGin5G Server for sending MSGin5G message as specified in table 8.3.2-1 in case the request is to be delivered as a group message, or the Message Response as specified in table 8.3.2-3 if the request to deliver the message as a group message is rejected.

NOTE:	The Application Server does not alter the Originating UE Service ID.

See clause 8.3.3 for the details of usage of this API operation.

* * End of Change * * *