SA WG6 Meeting #58	S6-233459

Chicago, USA, 13th – 17th November 2023	(revision of S6-232806)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0021 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of PIN profile and dynamic PIN profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For now, some of the parameters in PIN profile and dynamic PIN profile are the same. And also, it seems unclear that where is the difference between PIN profile and dynamic PIN profile. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification of the details of PIN profile and dynamic PIN profile. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear where is the difference between PIN profile and dynamic PIN profile. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.0(new), 8.2.2.1, 8.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1, all new text * * *

8.2.2.0	PIN Profile in a PIN

PIN profile information include static and semi-static data and default configuration that are needed for configuration of Personal IoT Networks, such as the PEMC, PEGC and PIN server end point addresses. Information in a PIN profile is infrequently changed.

Dynamic PIN profile information includes data that are updated more frequently due to operations of a PIN, e.g. caused by PINE join or PINE leave operations. Example information include PINE lists and the services that PINEs provide.

* * * Start of Change 2 * * *

8.2.2.1	PIN Profile in a PIN

PIN profile includes information about the static data needed for configuration for the Personal IoT networks.

Table 8.2.2.1-1: PIN Profile

| Parameter Name | Parameter Description | PIN Server | PEMC | PEGC |
| --- | --- | --- | --- | --- |
| PIN ID | The identifier of the PIN | Y | Y | Y |
| PIN Description | Human-readable description of the PIN, for example, the company name, location or the type of service. | Y | Y | Y |
| Duration | Specifies the time period of how long the PIN can be active | Y | Y | Y |
| Maximum number of PIN elements | Maximum number of PIN elements allowed to join the PIN | Y | Y | N |
| PIN service | List of service that a PIN can provide, including the PINE service or the service that can provided by application client on PINE: > PIN service Provider Identifier > PIN service type > PIN service Feature | Y | Y | N |
| PEMC list | The list of identifiers of the PIN elements which can be allowed to take the role as PEMC (e.g.: PIN client ID, UE GPSI etc.,) and also it contains whether the role is primary or secondary > PIN Element ID > Role | Y | Y | Y |
| PEGC list | The list of identifiers of the PIN elements which can be allowed to take the role as PEGC (e.g.: PIN client ID, UE GPSI etc.,) > PIN Element ID > Role > KPIs supported by this PEGC (e.g., maximum number of assigned PIN elements) | Y | Y | Y |
| PIN Server ID | The identifier of the PIN server that serves the PIN | N | Y | Y |
| PIN server Endpoint | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the PIN server. | N | Y | Y |
| Allowed PIN elements list | List of PIN elements which can be allowed to join the PIN > PIN element ID | Y | Y | Y |

Editor's Note: The NAT related issues if any in PIN is FFS.

* * * Start of Change 3 * * *

8.2.2.2	Dynamic profile information of a PIN

Dynamic profile information of a PIN contains the PIN dynamic data needed for management of the Personal IoT networks.

Table 8.2.2.2-1 describes the list of parameters that are classified as dynamic profile information and which are maintained at the PIN server, PEMC and PEGC. PIN dynamic profile information maintained at these entities are updated based on the events occurring in the PIN. Below are some of the events (not exhaustive):

-	PINE joins or leaves the PIN;

-	Role of PEMC or PEGC changes;

-	When a PINE updates the services it offers;

Table 8.2.2.2-1: Dynamic profile information of a PIN

| Parameter Name | Parameter Description | PIN Server | PEMC | PEGC |
| --- | --- | --- | --- | --- |
| PIN ID | The identifier of the PIN | Y | Y | Y |
| Services offered | List of services currently offered by the PIN | Y | Y | N |
| PIN state | Indicates the current state of the PIN (activated or de-activated). When the PIN is in deactivated state services offered by the PIN are inaccessible and no PIN elements can join the PIN. Also PEGC closes all the communication channel it has created for the flow of application traffic from PIN elements via 5GS to the application server | Y | Y | Y |
| PEMC list | The list of identifiers of the PIN elements which are currently serving as PEMC (e.g.: PIN client ID, UE GPSI etc.,) and whether the PEMC is authorized as primary PEMC or secondary PEMC. | Y | Y | Y |
|  | > PIN Element ID | Y | Y | Y |
|  | > PEMC role | Y | Y | Y |
|  | > Endpoint information of each PEMC (e.g. URI, FQDN, IP address, port number) used to communicate with the PEMC. | Y | Y | Y |
|  | > Duration or time period this PIN element allowed to being PEMC | Y | Y | Y |
|  | > Heartbeat timer value defining the periodicity of heartbeat requests this PEMC sends to the PIN server to indicate this PEMC is still available and serving as a PEMC. | Y | Y | Y |
| PEGC list | The list of identifiers of the PIN elements which are currently serving as PEGC (e.g.: PIN client ID, UE GPSI etc.,) | Y | Y | Y |
|  | > PIN Element ID | Y | Y | Y |
|  | > Endpoint information of each PEGC (e.g. URI, FQDN, IP address, port number) used to communicate with the PEGC. | Y | Y | Y |
|  | > Duration or time period this PIN element allowed to being PEGC | Y | Y | Y |
|  | > PEGC role: Primary, Backup | Y | Y | Y |
|  | > Backup PEGC list | Y | Y | Y |
|  | > List of PIN elements being served by PEGC and their connectivity information | Y | Y | Y |
|  | > Heartbeat timer value defining the periodicity of heartbeat requests this PEGC sends to the PEMC to indicate this PEGC is still available and serving as a PEGC. | Y | Y | Y |
| PIN Elements List | List of PIN elements currently registered/joined the PIN and their details |  |  |  |
|  | > PIN Element ID | Y | Y | Y |
|  | > Endpoint information of each PINE (e.g. URI, FQDN, IP address, port number) used to communicate with the PINE (NOTE 1). | Y | Y | Y |
|  | > Services offered by the PIN element | Y | Y | N |
|  | > Reachability information of the PIN element NOTE: Reachability information of the PIN element may include the IP address which is internally routable via PEGC (like LAN or Layer 2 gateway) or routable via 5GS. | Y | Y | Y |
|  | > List of application clients for this PIN element: | Y | Y | Y |
|  | >> Minimum KPIs required by each application client to operate effectively within the PIN (e.g., PIN bandwidth, PIN request rate, PIN response time) | Y | Y | Y |
|  | >> Operational schedules of each application client (e.g., time windows) | Y | Y | Y |
|  | > Identifier of the default PEGC authorized to service this PIN element. The PIN element will use this PEGC as the primary PEGC to relay PIN communications. Location and/or schedule information for the default PEGC may also be included such that the default PEGC may be selected by the PIN element based on its current location and proximity to the default PEGC and/or the availability schedule of the default PEGC. | Y | Y | Y |
|  | > Identifiers of backup PEGCs authorized to service this PIN element. The list is in prioritized order (the first PEGC listed will serve as the first backup PEGC). If the default PEGC is not available, the PIN element will use this prioritized list of PEGCs to relay PIN communications. Location and/or schedule information for each of the backup PEGCs may also be included such that a backup PEGC may be selected by the PIN element based on its current location and proximity to a backup PEGC and/or the availability schedule of the PEGC. | Y | Y | Y |
|  | > Heartbeat timer value defining the periodicity of heartbeat requests this PIN element sends to the PEMC to indicate this PIN element is still available within the PIN. | Y | Y | Y |
| NOTE 1: The port number is the port used by a PINE to expose a service within the PIN. |  |  |  |  |

* * * End of Change * * *