3GPP TSG-SA6 Meeting #58	S6-233463

Chicago, USA, November 13 – 17, 2023	(revision of S6-233245)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0031 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN-Notify-Correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [KPN N.V.], vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the PINE management procedures in clause 8.5.8.2, the addition or removal of a PIN element triggers a status notify to the PEGC and PIN elements acting as relays. However, in clause 8.5.8.2.3, the figure for the procedure for removal of a PIN element by the PEMC shows that the PIN status notify is sent to all PIN elements, which is not aligned with the other procedures. In Rev 2: Due to the relay capability in PIN element is not introduced in current specification, so only the PEGC keeps this relay capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating Figure 8.5.8.2.3-1 to align with the other procedures. - Align procedures in clauses 8.5.8.2.1, 8.5.8.2.2 and 8.5.8.2.3 to align with the figures in the corresponding clauses by including PIN elements with relay capabilities in the procedures. (This change was made during the meeting i.e.SA6#57) In Rev 2, only keep the relay capability in PEGC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is unclear and ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.8.2.1, 8.5.8.2.2, 8.5.8.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.5.8.2.1	PIN client requests to join into a PIN

Figure 8.5.8.2.1-1 illustrates procedure of PIN client requests to join into a PIN, based on request/response model.

Pre-conditions:

1.	The UE (PIN client) has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PEMC;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PIN client has been authorized to communicate with the PEMC and already has application layer connection with PEMC;

4.	The PIN client has already received the list of PIN ID, corresponding PEMC IP address and configuration information related to each PIN;

Figure 8.5.8.2.1-1: PIN client requests to join into a PIN

1.	The PIN client sends the PIN Management PINE join into PIN request to PEMC to join the PIN. The request includes the security credentials of the PIN client received during authorization procedure and may include the UE identifier such as GPSI, PIN client ID, UE location, PIN ID and PIN client profile(s) information as defined in clause 8.5.8.3.2.

The request also includes service that PINE can provide. In the request, both the service that PIN client in PINE can provide and the service that application client on PINE can provide.

Editor's note: The authorization procedure between PIN client and PEMC should be captured in SA3 scope.

2.	Upon receiving the request, the PEMC performs an authorization check to verify whether the PIN client has authorization to join the PIN.

3.	The PEMC sends a successful PIN Management PINE join into PIN response to PIN client. Also, the access control information for the PIN client is also included, for example, user name, account, SSID, BSSID. All the information is used by PIN elements in PIN to access 5G or access the network provided by PEGC. The PEMC also provides lifetime of the PIN, identity, address of PEGC and may also provide unique PIN client ID to identify the PIN element within a PIN.

4.	The PEMC sends PIN status notify to the PIN server containing the details of the new PIN client that joined the PIN, including PIN client ID, GPSI and etc.

5.	The PEMC sends PIN status notify to the PEGC (may with relay capabilities) and other PIN elements containing the details of the new PIN client that joined the PIN. And the PEGC decides to enables the PINE to access 5GS.

6-8.	The PEMC/PEGC/PINE updates PIN profile with the details of the new PIN client that joined the PIN and the service that the PINE can provide.

* * * Second Change * * * *

8.5.8.2.2	Procedure of PIN elements decides to leave the PIN

The following procedure defines the PIN elements decides to leave the PIN.

Pre-conditions:

1.	The PIN client has already been added into a PIN;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PIN client has been authorized to communicate with the PEMC and already has the application layer connection with the PEMC;

Figure 8.5.8.2.2-1: PIN client decides to leave a PIN

1.	The PINE decides to leave a PIN, and sends the PIN Management PINE leave from PIN request to PEMC to leave the PIN. The request includes the security credentials of the UE or PIN client received during authorization procedure and may include the UE identifier such as GPSI, PIN client ID, UE location and PIN ID.

Editor's note: The authorization procedure between PIN client and PEMC should be captured in SA3 scope.

2.	The PEMC authorizes the request, and decides to remove a PIN client from a PIN which indicated by PIN client ID or UE GPSI.

3.	The PEMC sends the PIN Management PINE leave from PIN response to PINE to notify that the PIN client is not the member of the PIN anymore.

4-5.	The PEMC sends the PIN status notify Request to the PEGC (may with relay capabilities), PIN server and PIN elements containing the details of the PIN client that requested to leave the PIN. The details of the PINE include PIN client ID, GPSI and etc.

6-8.	The PEMC/PEGC/PIN server updates the dynamic profile information of the PIN to remove the details of the PIN client that requested to leave the PIN. The PEGC disables the access control information for this PINE.

* * * Third Change * * * *

8.5.8.2.3	Procedure of PEMC removes the PIN elements from a PIN

The following procedure defines the PIN elements decides to leave the PIN.

Pre-conditions:

1.	The PIN client has already been added into a PIN;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PIN client has been authorized to communicate with the PEMC and already has the application layer connection with PEMC;

Figure 8.5.8.2.3-1: Remove a PIN element from a PIN by PEMC

1.	The PEMC decides to remove the PINE (identified by GPSI, PIN client ID).

2.	The PEMC sends the PIN status notify to the PINE that this PINE has been removed from the PIN.

3-4.	The PEMC sends the PIN status notify to the PEGC (may with relay capabilities), PIN elements and PIN server containing the details of the PIN client that is removed from the PIN. The details of the PINE include PIN client ID, GPSI and etc.

5-7.	The PEMC/PEGC/PIN server updates the dynamic profile information of the PIN to remove the details of the PIN client that is removed from the PIN. The PEGC disables the access control information for this PINE.

* * * END of Change * * * *