3GPP TSG-SA WG6 Meeting #58	S6-233577

Chicago, IL, USA 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0399 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on modifying the criteria during an ad hoc group call (MCPTT) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Last meeting changes were agreed for the multiple MC systems scenario. This CR aligns changes made for the multiple MC system scenario to the single MC system case: - replace alerting by authorized or requesting - update the pre-conditions - remove alerting from the figure and mention authorised user - clarify that the MC service server continuously checks |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rewording as listed in the reason for change field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not conistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.2.20, 10.19.2.21, 10.19.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 10.19.2.20	Modify ad hoc group call criteria request (MCPTT client – MCPTT server)

Table 10.19.2.20-1 describes the information flow Modify ad hoc group call criteria request from the MCPTT client to the MCPTT server.

Table 10.19.2.20-1: Modify ad hoc group call criteria request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the user authorized to modify the criteria |
| Functional alias | O | The functional alias of the authorized user |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call whose participants list needs to be modified |
| Criteria to determine the list of participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCPTT server for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area |

#### 10.19.2.21	Modify ad hoc group call criteria response (MCPTT server – MCPTT client)

Table 10.19.2.21-1 describes the information flow Modify ad hoc group call criteria response from the MCPTT server to the MCPTT client.

Table 10.19.2.21-1 Modify Ad hoc group call criteria response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the user authorized to modify the criteria |
| Functional alias | O | The functional alias of the authorized user |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group call participants request (success of failure) |

# * * * * 2nd change * * * *

##### 10.19.3.1.6	Modification of ad hoc group call criteria by an authorized user

Figure 10.19.3.1.6-1 below illustrates the modification of ad hoc group call criteria procedure by an authorized user.

Pre-conditions:

1.	The MCPTT user at the MCPTT client 1 is authorized to modify the criteria.

2.	The MCPTT server is aware of the criteria related to the ongoing ad hoc group call..

Figure 10.19.3.1.6-1: Modification of ad hoc group call criteria by the authorized user

1.	The MCPTT user 1 at the MCPTT client 1 is authorized and requests to modify the criteria for determining the list of participants. The MCPTT client 1 sends the modify ad hoc group call criteria request to the MCPTT server which contains an updated criteria to determine the list of participants.

2.	The MCPTT server verifies whether the MCPTT client 1 is authorized to modify the criteria which determines the list of participants during on-going ad hoc group voice communication. The MCPTT server determines the list of ad hoc group call participants based on the criteria provided and indentifies that MCPTT client 3 is to be removed from and MCPTT client 2 is to be added to the group.

3.	The MCPTT server sends modify ad hoc group call criteria response to MCPTT client 1.

4.	The MCPTT server sends the ad hoc group call leave request to the MCPTT client 3 to remove it from the on-going ad hoc group call.

5.	The MCPTT client 3 notifies the user of the ad hoc group call leave request.

6.	The MCPTT client 3 sends the ad hoc group call leave response to the MCPTT server. MCPTT client 3 is implicitly de-affiliated.

7.	The MCPTT server sends the ad hoc group call request towards MCPTT client 2.

NOTE 1:	Steps 7 to 9 can occur at any time following step 3.

8.	The receiving MCPTT client 2 notifies the user about the incoming ad hoc group call.

9.	The MCPTT client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCPTT server. The response may also contain a functional alias of the responding MCPTT user, which is verified (valid and activated for the user) by the MCPTT server. MCPTT client 2 is implicitly affiliated.

10.	The MCPTT server may notify the initiating MCPTT user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the ad hoc group call.

11.	The MCPTT server may notify the participants about the change in the participants list of on-going ad hoc group call.

The MCPTT server continuously checks whether other MCPTT clients meet or if participating MCPTT clients no longer meet the criteria for the ad hoc group emergency call.

NOTE 2:	If the ad hoc group call is associated with an ad hoc group emergency alert and the change of criteria caused the modification of ad hoc group call participant list then the ongoing ad hoc group emergency alert is modified accordingly.

# * * * * End of changes * * * *