3GPP TSG-SA WG6 Meeting #58	S6-233597

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0263 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complete description of consumers in GM-related information flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some clauses related to information flows in group management applies to both VAL servers or GM clients: - As stated in clause 10.3.5.2, group membership update request/response apply to both (Authorized) GM client or VAL server, - As stated in clause 10.3.6, store group configuration request/response apply to both (Authorized) GM client or VAL server, - As stated in clause 10.3.6.2, get group configuration request/response apply to both GM client or VAL server, - As stated in clause 10.3.7, Location-based group creation can be triggered by GM client or VAL server, Nevertheless, only reference to one requesting entity is present in the description of the above information flows or information elements. It is proposed to complete the description of the information flows and information elements to be compliant with the content that is already in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | GM client or VAL server as known consumers in clauses 10.3.2.6, 10.3.2.7, 10.3.2.8, 10.3.2.18, 10.3.2.19, 10.3.2.20, 10.3.2.21, 10.3.2.34, 10.3.6.2, and 10.3.7 (whenever it applies) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete stage 2 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2.6, 10.3.2.7, 10.3.2.8, 10.3.2.18, 10.3.2.19, 10.3.2.20, 10.3.2.21, 10.3.2.34, 10.3.6.2, 10.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 10.3.2.6	Group membership update request

Table 10.3.2.6-1 describes the information flow group membership update request from the group management client or VAL server to the group management server.

Table 10.3.2.6-1: Group membership update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| VAL group ID | M | Identity of the VAL group |
| Identity | M | List of identities of the VAL users and VAL UEs affected by this operation |
| Operations | M | Add to or delete from the group |
| VAL service specific information (see NOTE) | O | Placeholder for VAL service specific information |
| NOTE: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

* * * * Next Change * * * *

#### 10.3.2.7	Group membership update response

Table 10.3.2.7-1 describes the information flow group membership update response from the group management server to the group management client or VAL server.

Table 10.3.2.7-1: Group membership update response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | Identity of the VAL group |
| Result | M | Indicates the success or failure for the operation |

* * * * Next Change * * * *

#### 10.3.2.8	Group membership notification

Table 10.3.2.8-1 describes the information flow group membership notification from the group management server to the VAL server or the group management client.

Table 10.3.2.8-1: Group membership notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | Identity of the VAL group |
| Identity | M | List of identities of the VAL users and VAL UEs affected by this operation |
| Operations | M | Add to or delete from the group |
| VAL service specific information (see NOTE) | O | Placeholder for VAL service specific information |
| NOTE: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

Table 10.3.2.8-2 describes the information flow group membership notification from the group management server to the group management client.

Table 10.3.2.8-2: Group membership notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | Identity of the VAL group |
| Operations | M | Add to or delete from the group |

* * * * Next Change * * * *

#### 10.3.2.18	Store group configuration request

Table 10.3.2.18-1 describes the information flow store group configuration request from the group management client or VAL server to the group management server.

Table 10.3.2.18-1: Store group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL group configuration data | M | VAL group configuration data |

* * * * Next Change * * * *

#### 10.3.2.19	Store group configuration response

Table 10.3.2.19-1 describes the information flow store group configuration response from the group management server to the group management client or VAL server.

Table 10.3.2.19-1: Store group configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| Result | M | Indicates the success or failure for the result |

* * * * Next Change * * * *

#### 10.3.2.20	Get group configuration request

Table 10.3.2.20-1 describes the information flow get group configuration request from the group management client or VAL server to the group management server.

Table 10.3.2.20-1: Get group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL group information reference | M | Reference to configuration data for the VAL group |
| VAL services requested (see NOTE) | O | Service(s) for which group configuration is requested |
| NOTE: If 'VAL services requested' is not present, group configuration is requested for all services defined for the VAL group |  |  |

* * * * Next Change * * * *

#### 10.3.2.21	Get group configuration response

Table 10.3.2.21-1 describes the information flow get configuration response from the group management server to the group management client or VAL server.

Table 10.3.2.21: Get group configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| VAL group configuration data | M (see NOTE) | VAL group configuration data |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of the VAL group configuration data information element has no meaning. |  |  |

* * * * Next Change * * * *

#### 10.3.2.34	Location-based group creation request

Table 10.3.2.34-1 describes the information flow location-based group creation request from the group management client or VAL server to the group management server.

Table 10.3.2.34-1: location-based group creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| Location criteria | M | Criteria to combine Users or UEs in a location (e.g. geographical area or VAL service area ID). |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| VAL service specific information (see NOTE 2) | O | Placeholder for VAL service specific information |
| NOTE 1: This information element shall be included in the message for creating a group configured for multiple VAL services. NOTE 2: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

* * * * Next Change * * * *

#### 10.3.6.2	Retrieve group configurations

The procedure for retrieve group configurations at the group management client or the VAL server is described in figure 10.3.6.2-1. This procedure can be used following service authorisation when the configuration management client has received the list of groups and the group management client needs to obtain the group configurations, or following a notification from the group management server that new group configuration information is available.

Pre-conditions:

-	The group management server has received configuration data for groups, and has stored this configuration data;

-	The VAL UE has registered for service and the group management client or the VAL server needs to download group configuration data applicable to the user/UE.

Figure 10.3.6.2-1: Retrieve group configurations

1.	The group management client or the VAL server requests the group configuration data.

2.	The group management server provides the group configuration data to the client or the VAL server.

3.	The group management client or the VAL server stores the group configuration information.

* * * * Next Change * * * *

### 10.3.7	Location-based group creation

Figure 10.3.7-1 below illustrates the location-based group creation.

Pre-conditions:

1.	The group management client, group management server, VAL server, location management server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator or VAL server is not aware of the users' or UE identities which will be combined to form the VAL group.

Figure 10.3.7-1: Location-based group creation

1.	The group management client or the VAL server requests location-based group create operation to the group management server. The location criteria for determining the identities of the users or UEs to be combined shall be included in this message and the location criteria may be the VAL service area identified by the VAL service area ID.

2.	The group management server requests the location management server for obtaining the users or UEs corresponding to the location information or the VAL service area identified by the VAL service area ID.

3.	The location management server composes the list of users or UEs within the requested location.

4.	The group management server receives the composed list of users or UEs from the location management server. The location management server may include VAL service IDs associated with the list of users or UEs.

5.	During the group creation, the group management server creates and stores the information of the location-based group. The group management server performs the check on the policies e.g. maximum limit of the total number of VAL group members for the VAL group(s). If an external group identifier, identifying the member UEs of the VAL group at the 3GPP core network is available, then the external group ID is stored in the newly created VAL group's configuration information. If VAL service ID list was received in step 1 as part of group formation criteria and not in step 4 for the composed list of users or UEs, the group management server shall retrieve the VAL service data from the configuration management server as described in clause 11.3.2.13 and form the group with relevant users or UEs.

NOTE:	The exact policies are out of scope of the present document.

6.	The group management server provides a location-based group creation response to the group management client or the VAL server.

* * * * End of Changes * * * *