3GPP TSG-SA WG6 Meeting #58	S6-233620

Chicago, USA, 13th – 17th November 2023

Source:	Convida Wireless LLC

Title:	KI#3 solution F/AIML client selection

Spec:	3GPP TR 23.700-82 v0.1.0

Agenda item:	8.3

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

Quang Ly, ly.quang@convidawireless.com

1. Introduction

This contribution provides a solution on F/AIML client selection for KI #3 support for federated learning

2. Reason for Change

Proposes a solution for KI #3 support for federated learning. The F/AIML notation corresponds to a  new (SA6#58) architectural enhancement proposal.

After  a VAL server discovers F/AIML clients that are able to support the required federated learning operation and that have the corresponding dataset, then the F/AIML client selection can be made. The VAL server can request the F/AIML server to create a F/AIML group of the discovered F/AIML clients to enable future federated learning operations (e.g. training or inferencing). The forming of the F/AIML group is an indication that the F/AIML clients are available and to have required data for future federated learning operations and the F/AIML server uses the F/AIML group to manage federated learning operations.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82.

* * * First Change * * * *

# 8	Solutions

## 8.0	Mapping of solutions to key issues

Table 8.1-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #Z |  |  | X |  |

* * * Second Change * * * *

## 8.Z 	Solution #Z: F/AIML client selection

### 8.Z.1	Solution description

The following clauses specify procedures, information flows, and APIs for Key Issue #3 to support F/AIML client selection.

Assumptions:

1.	The proposed solution is based on client-server architecture for federated learning.

2.	The VAL server or F/AIML client has already discovered and received a list of F/AIML clients that are suitable and have available data for a particular ML application.

Figure 8.z.1-1: F/AIML client selection procedure

1.	A VAL server or F/AIML client sends a request to an F/AIML server to select a list of F/AIML clients that have been discovered to meet the requirements for federated learning operations. The F/AIML client selection request includes client/server identifier, security credentials, application identifier, F/AIML role, ML application type and models/algorithm, list of F/AIML clients for inclusion into an F/AIML group, a minimum number of F/AIML clients, training schedule, and QoS requirements for the federated learning operations.

Editor’s Note: Other IEs in the request are FFS.

2.	The F/AIML server validates the selection request and verifies the security credentials provided in the request. The F/AIML server further performs authentication and authorization checks to determine if the requestor is able to create an F/AIML group. If authorized, the F/AIML server creates the F/AIML group with the list of F/AIML clients as members and assigns an F/AIML group identifier for the group.

3.	The F/AIML server sends an F/AIML client selection response that includes the status of the selection request and the assigned F/AIML group identifier.