3GPP TSG-SA WG6 Meeting #58	S6-233662

Chicago, USA, 13th - 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0071 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Floor override for multi-talker interworking with GSM-R |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current 23.283, the multi-talker floor control feature is not supported. Such multi-talker is required for interworking with GSM-R. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the multi-talker floor control using floor revoked/floor reject message when the maximum number of simultaneous talkers is reached. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The multi-talker floor control for interworking with GSM-R is not fully supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 10.5.x, (new) 10.5.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

10.5.x	Multi-talker floor control using floor override, revoked, rejected on an interworking group homed in the LMR system

Figure 10.5.x-1 shows the procedure for floor control on an interworking group homed in the LMR system.

Pre-conditions:

1.	The interworking group is homed in the LMR system, and is configured to support multi-talker control and audio mixing by the network is applied.

2.	The Floor control server is configured to locally filter competing floor control requests before communicating with the IWF.

3.	Floor client A, Floor client B, and LMR users (represented by the IWF) are affiliated to that group.

4.	An interworking group call is ongoing involving MCPTT users and LMR users (represented by the IWF). The floor participant B and an LMR user are granted the floor and talking now.

IWF

Floor control server

Floor participant B

（talker）

Floor participant A

(new talker)

2. Floor request

7. IWF floor granted

11. Voice media (A)

3. Initial floor arbitration

4.  IWF floor request

Notify the user

Voice media (B)

Voice media (B and LMR user)

5. IWF floor revoked

6. Floor revoked

8. floor granted

9. IWF multi-talker floor taken

10. multi-talker floor taken

12. Voice media (A and LMR user)

13. User wants to talk

14. Floor request

15.  IWF floor request

16. IWF floor rejected

17. Floor rejected

Notify the user

1. User wants to talk

Notify the user

Figure 10.5.x-1: Multi-talker floor control using floor override, revoked, rejected on a group homed in the LMR system

1.	The floor participant A wants to send voice media over the session.

2.	Floor participant A sends a floor request message to the floor control server.

3.	If the floor control server is configured to perform initial floor filter, the floor control server performs the initial floor arbitration.

4.	Since the group is homed in the LMR system, the floor control server forwards the floor request to the IWF for floor control determination. The IWF performs floor arbitration in conjunction with the LMR system (not shown). The maximum number of simultaneous talkers in the MCPTT group has been reached, the IWF decides to apply the override mechanism towards floor participant B. The IWF determines that the floor can be granted to the floor participant A.

5.	The IWF sends an IWF floor revoked message to the floor control server.

6.	The floor control server sends the floor revoked message to floor participant B and the user is notified.

7.	The IWF sends an IWF floor granted message to the floor control server.

8.	The floor control server sends a floor granted message to floor participant client A. The user at floor participant A is notified about the floor is granted.

9.	The IWF also sends an IWF multi-talker floor taken message to the floor control server.

10.	The floor control server sends a multi-talker floor taken message to floor participant client B, and the user at floor participant B is notified.

11.	The floor participant A starts to transmit voice media.

12. The floor participant B receives the voice media of A and LMR user.

13. User at participant B wants to talk.

14.	The floor participant B sends floor request to floor control sever.

15.	The floor control sever sends the IWF floor request to the IWF.

16.	The IWF determines to reject the floor request from the floor participant B. The IWF sends the IWF floor reject to the floor control server.

17.	The floor control server sends the floor rejected to the floor participant B, and the user is notified.

10.5.y	Multi-talker floor control using floor override, revoked, rejected an interworking group homed in the MCPTT system

Figure 10.5.y-1 shows the procedure for floor control on an interworking group homed in the MCPTT system.

Pre-conditions:

1.	The interworking group is homed in the MCPTT system.

2.	The interworking group is previously defined on the group management server.

3.	MCPTT client A, MCPTT client B, and LMR users (represented by the IWF) are affiliated to that group.

4.	An interworking group call is ongoing involving MCPTT users and LMR users (represented by the IWF). The floor participant B and an LMR user are granted the floor and talking now.

IWF

Floor control server

Floor participant B

（talker）

Floor participant A

(new talker)

2. Floor request

8. Voice media (A)

3. Floor arbitration

Notify the user

Voice media (B)

Voice media (B and LMR user)

5. floor granted

6. IWF multi-talker floor taken

7. multi-talker floor taken

10.  IWF floor request

11. IWF floor rejected

1. User wants to talk

Notify the user

4. IWF floor revoked

Voice media (LMR user)

9.Voice media (A and B)

Figure 10.5.y-1: Multi-talker floor control using floor override, revoked, rejected on a group homed in the MCPTT system

1.	The floor participant A wants to send voice media over the session.

2.	Floor participant A sends a floor request message to the floor control server.

3.	Since the group is homed in the MCPTT system, the floor control server performs floor arbitration. The maximum number of simultaneous talkers in the MCPTT group has been reached, the floor control server decides to apply the override mechanism towards LMR user. The floor control server determines that the floor can be granted to the floor participant A.

4.	The floor control server sends an IWF floor revoked message to the IWF.

5.	The floor control server sends a floor granted message to floor participant client A. The user at floor participant A is notified about the floor is granted.

6.	The floor control server sends an IWF multi-talker floor taken message to the IWF.

7.	The floor control server sends a multi-talker floor taken message to floor participant client B, and the user at floor participant B is notified.

8.	The floor participant A starts to transmit voice media.

9. The floor control server sends the voice media of A and B to IWF.

10. LRM user at participant B wants to talk. The IWF sends an IWF floor request to the floor control server.

11.	The floor control server determines to reject the floor request from the LMR user. The floor control server sends the IWF floor reject to the IWF.