3GPP TSG-SA WG6 Meeting #58	S6-233687

Chicago, USA, 13th – 17th November 2023	(revision of S6-233387)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0020 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on media delivery procedure in SEALDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In last meeting, the agreed CR (S6-233387) clarifies the architecture relationship between SEALDD and SA4 in media communication. Based on the agreed functionality mapping and current specified SEALDD procedure, this paper focuses on the media delivery procedure mapping between SEALDD and SA4, and clarify how the specified SEALDD transmission procedure to support the general media streaming defined in SA4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper adds the related description to capture the procedure mapping between SEALDD and SA4, regarding of uplink media delivery and downlink media delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How the current specified SEALDD data transmission procedure support media delivery in SA4, is still unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new), 2, Annex X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision after SA6#57 based on the agreed CR S6-233387 hilighted in green. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

Annex X (Informative):
Media data delivery in SEALDD

X.1	General

The SEALDD layer provides a general functionality to support data storage and data delivery by introducing 5G interaction (N5/N33 in control plane, N6 in user plane), including application data storage and management, application signaling and data transmission, E2E URLCC transmission, IoT message transmission (i.e. MSGin5G), etc.

X.2	SEALDD deployment for media delivery

For media delivery functionality in SEALDD, as shown in Figure X.2-1, the functionality/protocol defined in SA4 can be reused and further enhanced by SEALDD for supporting media type traffic, 5GMS/RTC AF and 5GMS RTC AS in 3GPP TS 26.501 [y] and 3GPP TS 26.506 [z] is utilized in SEALDD server to support the general media type delivery. The 5G transmission adaptation component is added in SEALDD, to facilitate the specific optimization for XR application provided by 5G network.

NOTE: The functionality about 5G transmission adaptation component for media type will be further detailed based on the other progresses (e.g. application enabler for XR Services).

Figure x.2-1 SEALDD deployment for media delivery

X.3	Procedure mapping for media delivery

For media delivery procedure, the specified SEALDD regular data transmission procedure in clause 9.2.2.2 can support the uplink media transmission and the downlink media transmission defined in TS 26.501 [y], by performing the procedure mapping.

The uplink media delivery procedure in clause 6.1 of TS 26.501 [y] can be mapped with SEALDD regular data transmission procedure. The detailed mapping about procedure and parameter is shown in Figure x.3-1 and Table x.3-1.

Figure x.3-1 The procedure mapping between SEALDD and SA4 for uplink media delivery

Table x.3-1: The procedure and parameter mapping for uplink media delivery

| SA4 procedure in Figure 6.1-1 of TS 26.501 | Related SEALDD procedure in Figure 9.2.2.2-1 | Related parameter mapping |
| --- | --- | --- |
| Step 1: 5GMS provisioning in M1u | Steps 1-2: Regular transmission request/response in SEALDD-S | Negotiate the access information in M1u/M2u (SEALDD-S), and optionally, in M4u/M5u (SEALDD-Uu) |
| Step 2: internal interaction in M3u | Not involved | Not involved, no parameter mapping required |
| (Option#1) Step 3: service access information acquisition | Step 3: provisioning transmission session between VAL server and VAL client | Provide the M1u/M2u (SEALDD-S) for media egest and the M4u/M5u (SEALDD-Uu) to 5GMSu-aware Application (VAL client) |
| (Option#1) Step 4: UE APIs in M6u/M7u | Step 4: SEALDD service request in SEALDD-C | Provide the access information in M1u/M2u (SEALDD-S), and in M4u/M5u (SEALDD-Uu) |
| (Option#1) Step 5: uplink streaming initialization in M5u | Step 6: Regular transmission connection establishment request in SEALDD-Uu | Uplink transmission request in M5u (SEALDD-Uu) |
| (Option#2) Step 6: UE APIs in M6u/M7u | Step 4: SEALDD service request in SEALDD-C | Request the service and access information |
| (Option#2) Step 7: Service access information acquisition | Step 6-7: Regular transmission connection establishment request/response in SEALDD-Uu | Service access information in M1u/M2u (SEALDD-S), and in M4u/M5u (SEALDD-Uu) obtained from 5GMSu AF (SEALDD server) |
| Step 8: Media session handling | Step 9: SEALDD connection establishment | No parameter mapping required |
| Steps 9-11: uplink media streaming transmission in M4u and M2u | Below step 9: application traffic transmission in SEALDD-Uu and SEALDD-S | No parameter mapping required |

The downlink media delivery procedure in clause 5.1 of TS 26.501 [y] can be mapped with SEALDD regular data transmission procedure. The detailed mapping about procedure and parameter is shown in Figure x.3-2 and Table x.3-2.

Figure x.3-2 The procedure mapping between SEALDD and SA4 for downlink media delivery

Table x.3-2: The procedure and parameter mapping for downlink media delivery

| SA4 procedure in Figure 5.1-1 of TS 26.501 | Related SEALDD procedure in Figure 9.2.2.2-1 | Related parameter mapping |
| --- | --- | --- |
| Step 1: 5GMS provisioning in M1d | Steps 1-2: Regular transmission request/response in SEALDD-S | Negotiate the access information in M1d/M2d (SEALDD-S), and optionally, in M4d/M5d (SEALDD-Uu) |
| Step 2: Internal interaction in M3d | Not involved | Not involved, no parameter mapping required |
| Step 3: 5GMS ingest in M2d | Step 8: Connection establishment in SEALDD-S | No parameter mapping required |
| Step 4: Service access information Announcement | Step 3: Provisioning transmission session between VAL server and VAL client | Provide the M4d/M5d (SEALDD-Uu) to 5GMSd-aware Application (VAL client) |
| Step 5: UE APIs in M6d/M7d | Step 4: SEALDD service request in SEALDD-C | Provide the access information in M4d/M5d (SEALDD-Uu) |
| (Optional) Step 6: Service access information acquisition | Steps 6-7: Regular transmission connection establishment request/response in SEALDD-Uu | Service access information in M4d/M5d (SEALDD-Uu) obtained from 5GMSd AF (SEALDD server) |
| Step 7: Media session handling | Step 9: SEALDD connection establishment in SEALDD-Uu | No parameter mapping required |
| Step 8: Downlink media streaming transmission in M4d | Below step 9: Downlink application traffic transmission in SEALDD-Uu | No parameter mapping required |

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[7]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[8]	3GPP TS 23.548: "5G System Enhancements for Edge Computing ".

[9]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2".

[10]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[11]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[12]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[y]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[z]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".