3GPP TSG-SA WG6 Meeting #58	S6-233732

Chicago, USA 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0492 | rev | - | Current version: | 18.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identities behind the MC gateway UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR emphasises that the instantiated MC client in the MC gateway UE on behalf of the non-3GPP device which cannot host a client follows the same steps related to MC service authentication and MC service user authorisation procedures. Some other corrections to update the “MC service client” to be “MC client”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -adding a sentence indicating that the instantiated MC clients in the MC gateway UE follows the same steps related to MC service authentication and MC service user authorisation. - replacing “MC service client” by the “MC client”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion my occur about the way the MC service ID is provided to the instantiated MC clients in the MC gateway UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.1, 11.3.2 and 11.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 11.3	Using identities behind the MC gateway UE

### 11.3.1	General

The MC gateway UE enables access to the MC system for MC clients either hosted on the MC gateway UE or hosted on non-3GPP devices using non-3GPP connectivity with the MC gateway UE. Simultaneous sharing of an MC gateway UE by multiple MC clients requires a unique relationship between the MC service identities used by the MC service with corresponding media streams passing the MC gateway UE towards the MC system sharing 3GPP connectivity of the MC gateway UE.

An MC service user behind the MC gateway UE has no 3GPP transport access credentials (i.e., no UICC), whereas the MC system relies on private user identity associated with public user identities (i.e., IMPU/IMPI) needed for authentication purposes. Moreover, the one-to-one correspondence between MC service ID and IMPUs enables routing of signalling traffic between the MC service server and the MC service client.

The use of the various mission critical user identities is determined by clause 8 of the present document.

# * * * * Second change * * * *

### 11.3.2	When using separate IMS subscriptions

If the MC clients are hosted by non-3GPP devices or the MC clients are hosted by the MC gateway UE, an MC service user corresponding to an MC client uses a dedicated IMS/SIP subscription without any relation to the MC gateway UE subscription.

Figure 11.3.2-1 depicts the use of the IMS Credential (IMC) application in accordance with 3GPP TS 23.228 [9].

Figure 11.3.2-1: IMC per MC service client

The MC service user profiles are independent of the MC gateway MC service user profile.

# * * * * Third change * * * *

### 11.3.3	When sharing MC gateway UE's IMS subscription

For the case of non-3GPP devices which are unable to host an MC client, the MC gateway UE forwards the media from/ to the non-3GPP devices behind the MC gateway UE. As there are no MC clients hosted by the non-3GPP devices, they rely on credentials at the MC gateway UE to access MC services.

The MC clients, instantiated at the MC gateway UE on behalf of the non-3GPP devices which cannot host an MC client, rely on MC service IDs provided by the MC gateway UE, which are obtained during MC service authentication and user authorization procedures performed by the instantiated MC client in the MC gateway UE. In this case, the MC clients instantiated at the MC gateway UE share the credentials from the IMS subscription of the MC gateway UE, as shown in Figure 11.3.3-1. The number of MC service clients at the MC gateway UE, which are instantiated on behalf on the non-3GPP devices which cannot host an MC client, is determined by the number of subscribed MC service IDs that are associated with the IMS subscription of the MC gateway UE.

Figure 11.3.3-1: Sharing MC gateway UE's IMC for non-3GPP devices which cannot host a client

# * * * * End of changes * * * *