3GPP TSG-SA WG6 Meeting #58	S6-233858

Chicago, USA, 13th – 17th November 2023	(revision of S6-233636)

Source:	China Mobile

Title:	Pseudo-CR on key issue of support of virtual number

Spec:	3GPP TR 23.700-92 v0.2.0

Agenda item:	8.2

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR proposes text for key issue of support of virtual number. Virtual number is a widely used value added calling service. This feature is different from the existing Terminating Identification Restriction (TIR) service specified in 3GPP TS 22.173. The virtual number looks like a real phone number rather than an anonymous number. A virtual number may either pre-configured to the caller or assigned to the caller dynamically on demand of a user  and the there may be more than one virtual number for the same caller in different calls. In addition, the callee can still use the virtual number to call the caller back after their call is finished or to call the caller back when the call is not answered.

This service is used when:

1.	a user want to hide his/her number in a call, especially when the call is established between the end user and an application/enterprise, or an employee of an enterprise, e.g. a driver of a taxi company; and

2.	the other party can still use this virtual number to call the user after this call is terminated, i.e. the user’s number is not just hiden during the call,

the user may request the network to assign a virtual number for him/her.

The network may assign a virtual number to the user on demand, and maintains the mapping between the user’s real number and the virtual number. A user may be assigned many different virtual number in the call with different parties. therefore virtual number is a necessary value added service and needed to be supported by eMMTel Enabler layer.Study how to provide virtual number can be covered by the 4th objective of this study item.

2. Reason for Change

Contribution with new text.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.2.0.

* * * First Change * * * *

6.X	Key Issue #X: Support of virtual number

6.X.1	Description

Virtual number is a widely used value added calling service. This feature is different from the existing Terminating Identification Restriction (TIR) service specified in 3GPP TS 22.173. The virtual number looks like a real phone number rather than an anonymous number. A Virtual number may either pre-configured to the caller or assigned to the caller dynamically on demand of a user  and the there may be more than one virtual numbers for the same caller in different calls. In addition, the callee can still use the virtual number to call the caller back after their call is finished or to call the caller  back when the call is not answered.

This service is used when:

1.	a user want to hide his/her number in a call, especially when the call is established between the end user and an application/enterprise, or an employee of an enterprise, e.g. a driver of a taxi company; and

2.	the other party can still use this virtual number to call the user after this call is terminated, i.e. the user’s number is not just hiden during the call,

the user may request the network to assign a virtual number for him/her. The network may assign a virtual number to the user on demand, and maintains the mapping between the user’s real number and the virtual number. A user may be assigned many different virtual numbers in the calls with different parties. Considering the high flexibility requirement of the virtual number assignment and maintaining, this feature is not suitable to be handled in IMS. Therefore, the virtual number is expected to have no impact on IMS.

Considering most use cases of eMMTel listed in 3GPP TR 22.873 [10], e.g. AR call used in emergency call, AR call used in remote cooperation, and AR call used in consumer-to-business call, are applied between end user and an application/enterprise, when enable eMMTel service to application providers/Vertical service provider, providing virtual number service for the end user is needed to be considered as a necessary value added service, i.e. virtual number is needed to be supported by eMMTel Enabler layer.

Editor’s note 1:	whether virtual number is recommended to be provided by the enabler layer as a value added service or only provided by 5GC/IMS is FFS.

This key issue aims to study how to support the virtual number service in the eMMTel Enabler layer:

1.	How to manage the virtual number in eMMTel Enabler layer, e.g. how to assign a virtual number on demand of a user, and how to assign a virtual number for a user in different calls and manage these virtual numbers.

2.	The necessary procedure in an eMMTel call if virtual number is introduced.

Editor’s note 2:	if the virtual number related procedures impact the procedures in 5GC/IMS, collaboration with SA WG2 is needed.