3GPP TSG-SA WG6 Meeting #59	S6-240024

Athens, Greece 26th February – 1st March 2024

3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401579

Electronic Meeting, 22-29 January 2024

# Title:	Reply LS on QoS to Carrier Mapping for SL CA

# Response to:	LS to SA2 on QoS to Carrier Mapping for SL CA (R2-2313623/S2-2400053)

# Release:	Release 18

# Work Item:	NR_SL_enh2-Core

Source:	SA2

To:	RAN2

Cc:	CT1, SA6

Contact Person:

Name:	Hong Cheng

Tel. Number:

E-mail Address:	hongc AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 would like to thank RAN2 for the LS to SA2 on QoS to Carrier Mapping for SL CA (R2-2313623/S2-2400053).

Regarding the following assumption stated in the LS:

As a consequence of this agreement, it is RAN2’s understanding that upper layers and network ensure appropriate intersection of carriers for the QoS flows mapped to the same SLRB.

SA2 would like to clarify as follows:

- From the upper layer perspective, only PC5 QoS flows configurations are visible at the V2X layer and above. It is specified in TS 23.287 clause 5.4.1.1.3:

  - "In addition, the V2X layer also provides the communication mode (e.g. broadcast, groupcast, unicast) and radio frequencies for the PC5 QoS Flow to the AS layer for the PC5 operation. The radio frequencies are determined based on the V2X service type associated with the PC5 QoS Flow based on the configuration as described in clause 5.1.2.1." and

  - "The V2X layer ensures that V2X service types associated with different radio frequencies are classified into distinct PC5 QoS Flows."

- The mappings between the PC5 QoS flows and the SLRBs are part of the AS layer configuration, which is invisible to the V2X layer and above. As defined in TS 38.331 clause 6.3.5, the mapping is indicated by the sl-SDAP-Config nested inside the SL-RadioBearerConfig IE. Therefore, as illustrated in TS 23.287 Figure 5.4.1.1.3-1, the mapping of PC5 QoS flows to SLRBs is purely controlled and performed at the AS layer.

As described above, the V2X layer has no knowledge about PC5 QoS flows mapping to SLRB. The V2X layer can only ensure that the V2X service types associated with different radio frequencies are not classified into the same PC5 QoS flow based on the configuration of the V2X service types to radio frequencies.

SA2 would like to ask RAN2 to take the above into account in RAN2 work and handle the intersection of carriers at the AS layer.

2. Actions:

To RAN2:

ACTION: 	SA2 would like to ask RAN2 to take the above into account in RAN2 work.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #161		26 February - 1 March 2024	Athens, Greece

TSG-SA2 Meeting #162		15 - 19 April 2024	China (TBD)