3GPP TSG-SA WG6 Meeting #59	S6-240030

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0193 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | A Constrained UE cannot be triggered |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A Constrained UE is a UE that cannot communicate via the 3GPP network and can therefore not be triggered from the 3GPP network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a note that states that a Constrained UE should not provide Triggering Information to the MSGin5G Server as part of its MSGin5G Client Profile. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An MSGin5G Server may attempt to trigger the Contrained UE which will fail by definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.2.1	MSGin5G UE Registration

The signalling flow for MSGin5G UE registration is illustrated in figure 8.2.1-1. The procedure assumes that the MSGin5G UE is responsible for initiating registration to the MSGin5G Server in order to establish association with the MSGin5G Server to receive MSGin5G Services.

Pre-conditions:

1.	The MSGin5G UE has connected to the serving network successfully.

2.	The MSGin5G UE has successfully completed the Configuration procedure; alternatively, a UE Service ID and the MSGin5G Server address have been pre-configured on the MSGin5G UE.

3.	Both the MSGin5G UE and MSGin5G Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.2.1-1: MSGin5G Client registration

1.	The MSGin5G UE sends an MSGin5G UE registration request to the MSGin5G Server. The request includes the UE Service ID and may include the MSGin5G Client Profile and Requested expiration time as detailed in Table 8.2.1-1.

Table 8.2.1-1: MSGin5G UE registration request

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the requesting MSGin5G UE. |
| MSGin5G Client Profile | O | Set of parameters describing the MSGin5G Client |
| >MSGin5G Client Triggering Information | O | UE Identifier (i.e., MSISDN, external ID), port number(s) and associated protocol (e.g., SMS, NIDD, etc.) for device triggering. See Table 8.2.1-2. The MSGin5G Server uses the information in step 5 of clause 8.9.3.2. (NOTE) |
| >MSGin5G Client Communication Availability | O | Communication availability information for the MSGin5G Client to receive MSGin5G messages. This IE informs the MSGin5G Server if the client has a specific application-level schedule/periodicity to its MSGin5G communications. See Table 8.2.1-3. |
| > MSGin5G Client Supported Maximum MSGin5G segment size | O | The Maximum MSGin5G segment size can be used by the MSGin5G Server to deliver message to the client served by it in its MSGin5G service domain. The MSGin5G message sent to the MSGin5G Client should be segmented by the MSGin5G Server serves the receiver if the message size is bigger than the MSGin5G Client Supported Maximum MSGin5G segment size as specified in clause 8.5. The value of this IE is decided by the MSGin5G Client, and is depended on the MSGin5G Client capabilities, e.g. supported transport, computing capability or application processing time limitation. If this IE is not included, the MSGin5G Server shall use the pre-configured global value within the MSGin5G service domain. |
| Requested expiration time | O | Requested expiration time for the registration. |
| NOTE: The MSGin5G Client Triggering Information is not included if the MSGin5G Client resides on a Constrained device. |  |  |

Table 8.2.1-2: MSGin5G Client Triggering Information

| Information element | Status | Description |
| --- | --- | --- |
| MSGin5G UE ID | M | Identity of the UE hosting the MSGin5G Client (e.g., the External Identifier defined in TS 23.682 [8], or an MSISDN) |
| MSGin5G Client Ports | M | List of port numbers that the MSGin5G Client listens on for device triggers from the MSGin5G Server. Also included with each port number is an associated protocol (e.g., SMS, NIDD, etc.). |

Table 8.2.1-3: MSGin5G Client Communication Availability

| Information element | Status | Description |
| --- | --- | --- |
| Scheduled communication time | M | Time when the UE becomes available for communication. |
| Communication duration time | M | Duration time of periodic communication. |
| Periodic communication indicator | O | Identifies whether the client communicates periodically or not, e.g., on demand. |
| Periodic communication interval | O | Interval Time of periodic communication. This IE is mandatory if the Periodic communication indicator indicates periodic communications. |
| Data size indication | O | Indicates the expected data size to be exchanged during the communication duration. |
| Store and forward option | O | Indicates opting out of store and forward services for incoming MSGin5G requests. The MSGin5G Server uses the information to determine whether Store and Forward procedure applies as specified in clause 8.3.6. |

2.	Upon receiving the request, the MSGin5G Server initiates authentication procedures with the MSGin5G Client and authorizes the MSGin5G Client. If the registration is successful, the MSGin5G Server stores the UE Service ID and associated MSGin5G Client Profile. The UE Service ID and associated MSGin5G Client Profile should be maintained on the MSGin5G Server until one of the following cases applies:

a)	the MSGin5G UE de-registers from the MSGin5G Server as specified in clause 8.2.2;

b)	the MSGin5G UE re-registered successfully with a different MSGin5G Client Profile; In this case, the MSGin5G Server shall store the UE Service ID and associated new MSGin5G Client Profile;

c)	the MSGin5G UE registration is expired; or

d)	the MSGin5G Server deletes the MSGin5G UE registration as required by the service provider.

NOTE:	The authentication procedures in step 2 are built on top of the transport layer mechanism specified in Annex Y.2 of 3GPP TS 33.501 [16].

3.	The MSGin5G Server sends an MSGin5G UE registration response to the MSGin5G UE. The response includes the information elements as detailed in Table 8.2.1-4. The registration expiration time may be returned either as provided by the MSGin5G client in the registration request or determined by the MSGin5G Server based on local policy.

Table 8.2.1-4: MSGin5G UE registration response

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier of the requesting MSGin5G UE. |
| Registration result | M | Indication if the registration is success or failure |
| Registration expiration time (see NOTE 1) | O | Indicates the expiration time of the registration. |
| Failure Cause (see NOTE 2) | O | The reason for failure |
| NOTE 1: This IE shall only be present when the Registration is Success. NOTE 2: This IE shall only be present when the Registration result is Failure. |  |  |