3GPP TSG-SA WG6 Meeting #58	S6-240045

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

Source:	Vodafone

Title:	Discussion of terms "resource owner" and "resource owner client"

Agenda Item:	7.6

Contact:	peter.dawes@vodafone.com

# Introduction

TS 23.222 defined a "resource owner client" entity (clause 6.3.8) as part of supporting invocation scenarios where the API invoker needs an authorization from the resource owner. However,

- TS 23.222 uses both "resource owner" and "resource owner client" for the entity that grants access to a resource;

- The "API invoker" entity is also sometimes referred to as the client; and

- Related documents, in particular RFC 6749 The OAuth 2.0 Authorization Framework, use the term "resource owner" and not "resource owner client".

Renaming "resource entity owner client" as "resource owner" would be clearer and avoid using two terms where only one is needed.

# Discussion

3GPP TS 23.222 V19.0.0 uses both "resource owner" and "resource owner client". Resource owner is used for example in clause 3.1 Definitions: (Resource owner: An entity (either a UE user or an MNO subscriber) capable of granting access to a protected resource related to the invoked API.) and throughout clause 8.31 API invoker obtaining authorization from resource owner.

"Resource owner client" is used for example in clause 6.2.3 Functional model description to support RNAA.

3GPP TS 29.222 V18.4.0 has no occurrences of "resource owner client".

3GPP TS 33.122 V18.2.0 has 1 occurrence of "resource owner client" (27 occurrences of "resource owner").

IETF RFC 6749 The OAuth 2.0 Authorization Framework defines the roles below, which do not include a "resource owner client" role. However, RFC 6749 does define the term "client" as it relates to OAuth:

1.1.  Roles

OAuth defines four roles:

resource owner

An entity capable of granting access to a protected resource.

When the resource owner is a person, it is referred to as an

end-user.

resource server

The server hosting the protected resources, capable of accepting

and responding to protected resource requests using access tokens.

client

An application making protected resource requests on behalf of the

resource owner and with its authorization.  The term "client" does

not imply any particular implementation characteristics (e.g.,

whether the application executes on a server, a desktop, or other

devices).

authorization server

The server issuing access tokens to the client after successfully

authenticating the resource owner and obtaining authorization.

It seems understood that a resource owner needs some kind of user interface to grant access to resources without needing to define a new entity to represent such a user interface.

# Proposal

It is proposed to change all occurrences of "resource owner client" in TS 23.222 to "resource owner". CRs have been submitted to make the change in Rel-18, Rel-19.