3GPP TSG-SA WG6 Meeting #59	S6-240057

Athens, Greece 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0555 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EAS Information Provisioning request type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | EAS Information Provisioning request may be used by the EEC to request three possible operations at the EES: - "ACR scenario selection announcement" - "ACR scenario selection request" - "EAS selection request" The requested operation is indicated by the “Request type” IE. The “Request type” IE is currently optional, and no default behavior is specified. Therefore, the EES does not know how to interpret EAS Information Provisioning request when “Request type” is not provided. |  |  |  |  |  |  |  |  |
| to |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | “Request type” is changed to mandatory IE for EAS Information Provisioning request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EES cannot interpret the request from the EEC when “Request type” is not provided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.15.3.2	EAS information provisioning request

Table 8.15.3.2-1 describes the information elements for EAS information provisioning request from the EEC to the EES.

Table 8.15.3.2-1: EAS information provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | The identifier of the EEC. |
| ACID | M | The identifier of the AC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Selected EAS ID(s) | O | The identifier(s) of the selected EAS or the selected EAS(s) for EAS bundles, which is either instantiated or instantiable. |
| Application Group ID (NOTE 4) | O | Application group identifier as defined in 7.2.11. |
| List of EES(s) (NOTE 4) | O | List of EES information (e.g. address information) corresponding to the Application group ID which is used for the common EAS announcement between these EESs as provided by ECS during service provisioning. |
| Selected EAS Endpoint(s) | O | The endpoint(s) of the selected EAS or the selected EAS(s) for EAS bundles |
| DNAIs and service area of the selected EAS(s) | O | For each selected EAS ID, it includes the DNAIs and/or service area for EAS bundles, as described in "EAS Geographical Service Area" IE, "EAS Topological Service Area" IE and "List of EAS DNAI(s)" IE of Table 8.2.4-1. |
| Request type | M | Request types: - ACR scenario selection announcement - ACR scenario selection request - EAS selection |
| Selected ACR scenario list (NOTE 1) | O | The list of ACR scenarios (or the list of ACR scenarios for EAS bundles) selected by the EEC |
| AC Profile (NOTE 2, NOTE 3) | O | AC Profile as described in Table 8.2.2-1 |
| EEC Service Continuity Support (NOTE 2, NOTE 5) | O | Indicates if the EEC supports service continuity or not. The IE indicates which ACR scenarios are supported by the EEC, also indicates the EEC ability (e.g. EAS bundle information) of handling bundled EAS ACR. |
| Associated EES(s) endpoint | O | EES information of the other EES(s) which support the direct bundled EAS, within the same EDN, and associated with the EASID list. |
| CAS information | O | Target CAS information received from AC. |
| NOTE 1: The IE may be present only if Selected EASID(s) and Selected EAS Endpoint(s) are present and Request type is "ACR scenario selection announcement" NOTE 2: The IEs are present only if request type is “ACR scenario selection request” NOTE 3: The IE is present if AC Profile is not shared to EES previously NOTE 4: This IE may be present only if the request type is "EAS selection". NOTE 5: The EAS bundle information is not applicable for proxy type of EAS bundle. |  |  |

* * * End of Changes * * * *