3GPP TSG-SA WG6 Meeting #59	S6-240083

Athens, Greece 26th February – 1st March 2024	(revision of S6-24yyyy)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.289 | CR |  | 0113 |  |  |  |  |  | rev |  |  | - |  |  | Current version: |  |  |  |  |  |  | 18.7.0 |  |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | UICC apps |  |  |  |  |  |  | ME | X |  | Radio Access Network |  |  |  |  |  |  | Core Network | X |  |  |
| Title: |  |  | Alignments of 23.289 Rel-18 with other specs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  |  | AT&T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  | S6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  | MCOver5MBS |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  | 14 February, 2024 |  |  |  |  |  |  |
| Category: |  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-18 |  |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |  |  |  |  |  |
| Reason for change: |  |  |  |  |  | Some aspects of Rel-18 5G MBS for mission critical related functionality and architecture have been missed in 23.289, but they are necessary for completion of Stage 3 development and alignment with SA2.This CR fixes the omissions. As before, 4G eMBMS client interactions with lower layers of MC UEs, decribed and modeled in 23.479 are not explicitly updated for 5G, but they continue to be reused by reference, via 5G clarifications and adaptations in 23.289. One of the main goals and additions of 5G functionality by SA2 in Rel-18, WID 5MBS_Ph2 (SP-230099) was the enablement of 5MBS operation in RRC Inactive state, per specification TS 23.247. RAN 5G Rel-18 specs also support this functionality. One of the principal drivers for this enhancement has been the mission critical (MC) need for handling high levels of congestion, understood as large number of MC UEs concentrated in a small incident area (e.g., a cell). Familarity with Annex A “Public safety use cases of large number of UEs in a single cell” of TR 23.700-47 and basic RAN-level realization that in RRC Inactive state a UE can receive downlink MBS traffic but cannot perform uplink transmissions (except for random access reqs on RACH), are necessary for understanding the issue. In short, due to congestion, the cell is filled with, and cannot handle more RRC Connected UEs, and when an RRC Inactive UE in the same cell needs to transmit something (e.g. a location report), it will essentially need to “trade places” with an RRC Connected UE. This transaction is expensive from a latency and traffic congestion point of view, and good operations should significantly reduce the number (thus, the frequency) of those transactions (i.e,. of RRC Inactive UEs requesting reconnection in order to transmit something). When those transmissions are not explicitly user initiated (e.g., periodic location reports), the MC client on the UE needs to be kept up-to-date of the UE RRC state, such that it can adjust the periodicity of location reports and subscribed notifications accordingly (MC location triggers are defined and maintained in Stage 3 by CT1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  | - Clarify text referencing 23.479, which uses the term MBMS rather than eMBMS. - Ther term “bearer” is not used in 5MBS. - In 5G, cell changes are understood as changes in cell id or changes of RRC state within the same cell. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  | Missing functionality and lack of alignment with other specs and sometimes confusing terms. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  | 4.7.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  |  |  |  |  |  | Other core specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |  |
| affected: |  |  |  |  |  |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |  |
| (show related CRs) |  |  |  |  |  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * * * * FIRST CHANGE * * * * * *

### 4.7.6	General architecture showcasing use of MBS by UE for MC services

Figure 4.7.6-1 presents a high-level system architecture that shows how the MC service UEs support the delivery of mission critical services through MBS. Figure 4.7.6-2 shows the functional model used by the UE, highlighting the conceptual MC MBS API used for information transfer within the UE. The shown system architecture and functional model are analogous to the models described in 3GPP TS 23.479 [20] and consistent with 3GPP TS 23.501 [7] and 3GPP TS 23.247 [15].

Figure 4.7.6-1: System architecture for MC MBS systems

NOTE:	The shown architecture does not consider MBS User Services, i.e., signalling with MBSF/MBSTF, which is described in 3GPP TS 26.502 [21].

The conceptual MC MBS API resides between the MC service client and the conceptual MC MBS user agent.

Figure 4.7.6-2: Functional model highlighting the MC MBS API

The MC service client uses information received from the MC service server through MC signalling (e.g., announcements) and through application-level signalling (e.g., mappings of MBS sessions and MBS subchannels to specific MC service groups) to communicate with the conceptual MC MBS user agent via the conceptual MC MBS API, in order to establish and update the proper communication context between the entities. Multiple MC service clients can be supported by the MC MBS user agent. The conceptual MC MBS user agent presents data and information received from the UE's lower layers to each MC service client according to the most recently established communication context. The functionalities of the MC service client and of the MC MBS user agent are described in clauses 4.3.2 and 4.3.3 of 3GPP TS 23.479 [20]. The information flows and procedures described in 3GPP TS 23.479 [20] apply, with the following clarifications:

-	References to "MBMS" (meaning 4G "eMBMS") are understood to be references to 5G "MBS";

-	Unless used as in "multicast IP address", the stand-alone term "multicast" is understood as "broadcast or multicast";

-	References to "SAI" are understood to be references to "MBS service areas", e.g., cell id, tracking area id, MBS frequency selection area id, as specified in 3GPP TS 23.274 [15];

-	References to (e)MBMS 4G "bearer" are understood to be references to 5G "MBS session" and references to 4G "TMGI" are understood to be references to 5G "MBS session ID"; and

-	"Cell information", used in cell information responses and cell update notifications in 5G, contains not only the id of the cell the MC UE is being served by, but also the RRC state of the MC service UE in the cell (i.e., active, inactive, idle). 5G cell updates notify not only changes of the serving cell, but also changes of RRC state for the MC UE, within the same cell.

* * * * * END OF CHANGES * * * * *