SA WG6 Meeting #59	S6-240090

Athens, Greece, 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0053 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Additional PEMCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2024-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In PIN creation procedure, the PEMC can involve other several PEMCs as the individual PEMC, that are allowed to manage the PIN: In order to save the procedure of several PEMCs to be involved into the certain PIN as individual PEMC, the PEMC can have the additional PEMC GPSIs/PIN client ID in the PIN create request, to indicate additional PEMCs that are allowed to manage the PIN. This procedure doesn’t have conflict with that other PEMC requests to join the certain PIN and becomes PEMC separately. But after PIN creation, there is no method to add/remove other PEMCs into the PIN. So it is to limited that only during PIN creation procedure to add additional PEMC into the PIN. It is reasonable that during PIN modification procedure that can also add additional PEMC into the PIN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification of the details of adding additional PEMC into the PIN during PIN modification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is too limited that only during PIN creation procedure to add additional PEMC into PIN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.2.2, 8.5.10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1, all new text * * *

##### 8.5.4.2.2	PIN management with multiple PEMCs

Figure 8.5.4.2.2-1 illustrates a procedure for managing the PIN with multiple PEMCs. PIN elements when registering with the PIN indicates that they have the capability to act as PEMC and are assigned with primary or secondary PEMC role as described in the subclause 8.4.2.2.1.

Pre-conditions:

1.	A PIN owner or admin creates a PIN and configured multiple PEMCs.

2.	PEMC-S is assigned with the role of secondary PEMC and PEMC-P is assigned with the role of primary PEMC.

3.	PEMC-S has direct connection or PIN direct connection with the PEMC-P.

Figure 8.5.4.2.2-1: PIN management with multiple PEMCs

The PEMC may send PIN configuration request which includes PIN ID, UE ID or PIN client ID to PIN server to indicate several additional PEMCs to be the PEMCs in PIN which is identified by PIN ID. These PEMCs are also allowed to manage the PIN. The PIN server may arrange these PEMCs as primary or secondary PEMCs.

1. 	PEMC-S receives a request from PIN owner or PIN admin to perform any of the PIN management operations. Some of the PIN management operations (PIN profile update, PIN element removal) shall be performed by secondary PEMC (PEMC-S) itself and some operations (PIN deletion, PIN activation and PIN deactivation) can be sent to primary PEMC (PEMC-P) which shall be validated and executed by the PEMC-P.

2. 	PEMC-S prepares the corresponding request including the required details for the requested operation and sends it to PEMC-P. These requests shall carry the PIN client ID of the PEMC-S mandatorily.

3. 	PEMC-P on receiving the request checks whether the PEMC-S is authorized as secondary PEMC in-order to perform the operation.

4. 	If the authorization succeeds, PEMC-P proceeds with the requested operation.

5. 	PEMC-P sends the response to PEMC-S containing the status or result of the requested operation.

NOTE:	Only the operations that are required to be performed by the PIN owner or PIN admin can be performed through secondary PEMC and all other operations like PIN discovery, PIN service discovery, authorizing PIN elements to join PIN etc., cannot be handled by the secondary PEMC.

* * * Start of Change 2 * * *

8.5.10.3.2	PIN configuration request

Table 8.5.10.3.2-1 shows the informational elements of the PIN configuration request sent by a PIN Element to the PIN server to obtain authorization for the modification of a PIN.

Table 8.5.10.3.2-1: PIN configuration request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | The identifier of the PIN |
| Requester PINE ID | M | The identifier of the PIN Element making the request |
| Authorization type | M | Request for the authorization to modify the configuration of a PIN: PEMC role change, PEGC role change |
| PINE ID of predecessor in the role | M | PINE ID of the element serving in the indicated role and needs to be changed e.g. due to being unavailable. |
| PINE ID of proposed successor in the role | O | PINE ID of the element proposed to assume the role |
| Additional PEMCs | O | Indicate additional PEMCs that are allowed to manage the PIN. |

* * * End of Change * * *