3GPP TSG-SA WG6 Meeting #59	S6-240099

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0569 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fix for common EAS aspects for service provisioning procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Service provisioning procedure (clause ), following is specified “When Application group profile is provided in the request, which applies to the common EAS case: - if the ECS-ER is not available, then - the ECS identifies EES(s) based on the information contained in the request (e.g.AC profile, Application group profile, UE location);” However, ECS does not have application group profile or application group ID. So, ECS will not be able to identify EES(s) based on application group profile, and thus service provisioning procedure will fail or will not be able to return appropriate EES(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added Application group profile in EES registration towards ECS. - Added application group profile in EAS registration towards EES. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service provisioning procedure for common EAS will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.3.3.2, 8.4.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.4.3.3.2	EAS registration request

Table 8.4.3.3.1-2 describes information elements in the EAS registration request from the EAS to the EES.

Table 8.4.3.3.2-1: EAS registration request

| Information element | Status | Description |
| --- | --- | --- |
| EAS Profile | M | EAS Profile as described in Table 8.2.4-1 |
| Security credentials | M | Security credentials of the EAS. |
| Proposed expiration time | O | Proposed expiration time for the registration |
| List of Application Group profiles | O | List of Application Group profile supported by the EAS |

* * * Next Change * * * *

##### 8.4.4.3.2	EES registration request

Table 8.4.4.3.2-1 describes information elements for an EES registration request from the EES to the ECS.

Table 8.4.4.3.2-1: EES registration request

| Information element | Status | Description |
| --- | --- | --- |
| EES Profile | M | Profile information as specified in Table 8.2.6-1 |
| Security credentials | M | Security credentials of the EES. |
| Proposed expiration time | O | Proposed expiration time for the registration |
| List of Application Group profiles | O | List of Application Group profile supported by the EASs registered with this EES |

* * * End of Change * * * *