3GPP TSG-SA WG6 Meeting #58	S6-240103

Chicago, USA, 13th – 17th November 2023	(revision of S6-233517)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0528 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common EAS in partner ECSP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During Rel-18 work, some of the use cases, scenarios, and requirements from GSMA OPG/OPAG (e.g. as discussed in LS S6-231931 and S6-232127) were not addressed. One of such scenario is - discovery of common EAS from federated partners. Further, EDGEAPP_Ph3 WID has following objective: iii) Discovery and selection of a common EAS from federated partners For above objective, solution needs to be provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to support discovery of common EAS from federated partners, following changes are proposed: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for common EAS will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17.2.3.2, 8.17.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.17.2.3.2	Request-response model

When required to find a suitable partner ECS, the ECS queries the ECS-ER by providing information such as the location of the UE, applications required by the UE etc. In response the ECS-ER provides ECS profile of partner ECS(s) providing the required application at the location indicated by the ECS. Figure 8.17.2.3.2-1 illustrates the procedure.

Pre-conditions:

1.	The ECS has the address (e.g. URI) of the ECS-ER.

Figure 8.17.2.3.2-1: ECS querying ECS-ER

1.	The ECS sends a ECS discovery request to the ECS-ER. The request contains security information of the ECS, and optionally AC profile(s) and UE's current location to filter ECS. If the UE hosting the EEC is roaming in a V-PLMN (determined using the serving PLMN information in the received service provisioning request, or by interacting with the H-PLMN), then the ECS discovery request includes the information of the serving PLMN i.e. the V-PLMN. The request may include application group profiles (containing application group ID(s)) to identify EES(s) supporting specific application group ID(s).

2.	The ECS-ER authorizes the received request. If authorized, the ECS-ER processes the request and gathers the information of partner ECS(s) that can satisfy the query parameters received in step 1.

Editor's note: Whether and how ECS-ER uses a different procedure to collect information from another ECS-ER or ECS from a different domain is FFS. Related updates to EDGE-10 interface are FFS.

3.	The ECS-ER sends a ECS discovery response to the ECS. The response includes ECS profile of the Partner ECS(s) available in the H-PLMN and/or the V-PLMN, depending on the serving PLMN of the UE as indicated in step 1 and may include DNN and S-NSSAI information for roaming UEs to establish a PDU session to the Partner ECS as specified in 3GPP TS 23.548 [20]. The ECS caches the received information for further use.

NOTE:	The ECS sending the ECS discovery request can also be an ECS-ER.

* * * Next Change * * * *

#### 8.17.3.8	ECS discovery request

Table 8.17.3.8-1 describes the information elements for ECS discovery request; from the ECS to the ECS-ER.

Table 8.17.3.8-1: ECS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| ECS address | M | Endpoint information of ECS (e.g. URI, FQDN, IP address) |
| Security credentials | M | Security credentials of the ECS. |
| Federation information | O | List of information for different federation agreements related to the ECS |
| > ECSP identifiers | O | The list of ECSPs preferred by the ECS. This information is used by the ECS-ER to filter the discovered partner ECS information. |
| AC Profile(s) | O | Filter information about required services as described in Table 8.2.2-1. |
| > Application group profile | O | Application group profile associated with the AC Profile, as defined in Table 8.2.11-1. |
| Connectivity information | O | Connectivity information such as serving PLMN information where the services are required. |
| UE location | O | Location of the UE for which the services are required. |

* * * Next Change * * * *

###### 8.17.2.4.2.2	Request-response model

Figure 8.17.2.4.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-condition(s):

1.	ECS has received a service provisioning request from an EEC or T-EES discovery request from an EES.

2.	The ECS has the ECS profile of the partner ECS (e.g. through pre-configuration, OAM configuration or ECS discovery).

3.	Required federation agreement exists between the ECSPs.

Figure 8.17.2.4.2.2-1: Service provisioning information retrieval – Request/Response

1.	The ECS sends a service provisioning information retrieval request to the partner ECS. The request contains security information of the ECS, and optionally AC profile(s) and UE's current location. If the UE hosting the EEC is roaming in a V-PLMN (determined using the serving PLMN information in the received service provisioning request, or by interacting with the H-PLMN), then the service provisioning information retrieval request includes the information of the serving PLMN i.e. the V-PLMN. The request may include application group profiles (containing application group ID(s)) to identify EES(s) supporting specific application group ID(s).

2.	The partner ECS authorizes the received request. If authorized, the partner ECS processes the request and gathers the provisioning information of the EDNs that can satisfy the query parameters received in step 1.

3.	The partner ECS sends a service provisioning information retrieval response to the ECS. The response includes provisioning information of the EDNs that can satisfy the query parameters received in step 1 and may include lifetime information. The ECS caches the received information for further use. If the Lifetime IE is included, then the ECS may reuse the retrieved information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

NOTE:	Only the information allowed by the federation agreement is included in the service provisioning information response.

* * * Next Change * * * *

#### 8.17.3.17	Service provisioning information retrieval request

Table 8.17.3.17-1 describes the information elements for Service provisioning information retrieval request from the ECS to a partner ECS.

Table 8.17.3.17-1: Service provisioning information retrieval request

| Information element | Status | Description |
| --- | --- | --- |
| ECS address | M | Endpoint information of ECS (e.g. URI, FQDN, IP address) |
| Security credentials | M | Security credentials of the ECS. |
| Federation information | O | List of information for different federation agreements related to the ECS |
| > Federation identifier | O | Identifier of the federation |
| > ECSP identifiers | O | The list of ECSPs preferred by the ECS. This information is used by the ECS-ER to filter the discovered partner ECS information. |
| AC Profile(s) | O | Filter information about required services as described in Table 8.2.2-1. |
| > Application group profile | O | Application group profile associated with the AC Profile, as defined in Table 8.2.11-1. |
| Connectivity information | O | Connectivity information such as serving PLMN information where the services are required. |
| UE location | O | Location of the UE for which the services are required. |

* * * End of Change * * * *