3GPP TSG-SA WG6 Meeting #59	S6-240125

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0510 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MC GW UE features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2024-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is to add the general descriptions about different capabilities/features provided by the MC GW UE and the MC servers to enable the user to use a non-3GPP device with/without the MC client to communication with the MC systems. Different capabilities to satisfied the SA1 MC GW UE requirements: - Basic and transparently traffic routing - any MC users can use the MC gateway UE for all MC services - Per user, per user per service connection control – only specific user can use the gateway UE for all or specific MC services - Location mangement - MBMS receiving However, due to R18 timeline issue, only the Basic and transparently traffic routing can be completed and the others has to be postponed to R19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new description in clause 11.1 General, to clarify that different features of MC GW UE, and only the Basic and transparently traffic routing is supported in R18. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear which feature is fully supported in R18, may cause confusion to downstream groups. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1, 11.2.0, 11.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************** 1st changes ****************************/

11.1	General

An MC gateway UE enables MC service access for a MC service user residing on non-3GPP capable devices and for devices which are unable to host MC service clients. The following functionalities are defined to support the MC gateway UE feature:

- User identity and IMS subscriptions for the MC service user behind the MC gateway UE as defined in clause 11.3.

- Transparently traffic routing (i.e., signalling, media content and traffic attributes, e.g. priority and QoS, of communications between the MC system and MC service client remains unaltered) as defined in clause 11.4 and further illustrated in the functional model defined in clause 11.2.1.

NOTE 1:	In case of non-3GPP device cannot host the MC clients, the interactions (e.g., protocol conversion) between the non-3GPP device and the MC gateway UE is outside the scope of present document.

- Authorization of the usage of the MC gateway UE(s).

  - No authorization: any the MC service user and any MC services are allowed to use any MC gateway UEs.

  - user authorization: only specific users are authorized to use the MC gateway UE.

  - service authorization: only specific services are authorized to use the MC gateway UE.

  - Restriction of accessing multiple gateway MCX UEs simultaneously from a single device for a single MC service, i.e., restricting a MC service to one MC gateway UE.

- Location management which allows the MC service user behind the MC gateway UE to handle the location triggers and information related to 3GPP access network.

- MBMS support for MC clients residing on non-3GPP devices.

NOTE 2:	Only the a) no authorization is supported in this present document, b), c) and d) are not supported in the present document.

NOTE 3:	The Location management and MBMS support for MC clients residing on non-3GPP devices are not supported in the present document.

In order to be able to participate in a MC service, the MC gateway UE provides the following necessary functions to enable the MC service access for such MC clients:

-	MC gateway client to MC gateway UE association: Authorised binding and use of non-3GPP based access with an MC service client. It indicates the associated MC gateway clients when the MC gateway UE cannot serve the connected MC service clients (any longer).

-	3GPP transport resources: Sharing of MC gateway UE 3GPP transport resources among the associated MC service clients.

-	MC client identification: Independent MC service client identification, MC service client authorisation and MC service profile association.

-	Location Management: Providing location management support for the associated MC clients to handle the location information and triggers related to 3GPP access network.

-	MC client communication integrity: Ensure that signalling, media content and traffic attributes, e.g. priority and QoS, of communications between the MC system and MC service client remains unaltered.

The following functions are provided by the MC gateway client:

-	MC gateway UE association: Allowing an MC gateway client to simultaneously bind to multiple MC gateway UEs.

NOTE 1:	An MC gateway client and its associated MC clients can only have one active binding per MC service with the associated MC gateway UE.

-	MC service selection: Choose between multiple MC gateway UEs for the required MC service capability.

-	MC service continuation: Enables reselection of the MC gateway UE association while MC service continues.

NOTE 2:	Further information about MC gateway UE selection are summarized in Annex D.

## 11.2	Functional Model

### 11.2.0	General

The MC gateway UE offers access to the MC server for several MC clients as shown in Figure 11.2.0-1. The MC clients can be either located in the MC gateway UE or in non-3GPP devices connected to the MC gateway UE via non-3GPP access.

For non-3GPP devices which can host an MC client, the MC gateway UE enables connectivity to the MC server. For non-3GPP devices which cannot host the MC client, the MC gateway UE hosts the instantiation of the MC client for the non-3GPP device.

Figure 11.2.0-1: Functional architecture

The MC gateway UE provides MC service capabilities and 3GPP access capabilities using 3GPP network credentials for authorized access with an MC server.

For non-3GPP devices which cannot host MC clients, the MC gateway UE shall control the access and manage the communication between the non-3GPP devices and the MC server. Upon reception of connection authorization request from a non-3GPP device, the MC gateway UE instantiates an MC client, acting on behalf of the non-3GPP device, to provide the requested services (e.g. emergency call, group calls, short data messages services, etc.). The communication interworking and the definition of associated procedures between the MC client (initiated at the MC gateway UE) and the non-3GPP devices is out of scope of this document.

NOTE 1:	MC clients residing on a non-3GPP device cannot use UICC credentials to perform authorisation with the 3GPP transport system.

For MC clients getting MC service access via the MC gateway UE, the MC gateway UE forwards (unmodified) signalling and media from the individual MC clients to the MC server and vice versa.

If the MC service user on the non-3GPP device utilizes multiple MC services simultaneously, the MC service access may also be provided by one or multiple MC gateway UEs as shown in figure 11.2.0-2 while restricting each MC service to one MC gateway UE (e.g. MCPTT via MC gateway UE1, MCData via MC gateway UE2).

Figure 11.2.0-2: Simultaneous multiple MC gateway UE use by a single non-3GPP device

NOTE 2:	Even not shown in the above figure, the same principle of simultaneous use of multiple MC gateway UEs is applied for non-3GPP devices which cannot host an MC client.

11.2.1	Functional model

The transparently traffic routing without any gateway usage authorization and connection restriction is enabled by the user identity and IMS subscriptions mechanism as described in clause 11.3 and the routing capabilities as described in clause 11.4.

The authorization of using a MC gateway UE(s) per user or per user and per service level is required in order to efficiently use the MC gateway UEs which has limited resource. The MC gateway client, MC gateway UE server in the MC gateway UE, MC gateway UE function in the MC service server, and the GW-local, GW-core reference points are specified to support the authorization of usage of MC gateway UE(s).

In addition, the GW-local is specified to support the location management which allows the MC service user behind the MC gateway UE to handle the location triggers and information related to 3GPP access network, and the MBMS reception and forwarding for MC clients residing on non-3GPP devices.

NOTE 1:	For the transparently traffic routing without gateway usage authorization and connection restriction, the MC gateway client, MC gateway UE server, the MC gateway UE function, GW-local and GW-core are not required.

For the application-level signalling necessary for the association between the MC client on the non-3GPP device and the MC gateway UE reference point Gateway-local (GW-local) is used. The MC gateway UE uses the Gateway-Core (GW-Core) reference point towards the MC service server to announce the association between MC gateway client and MC gateway UE server. Once the use of the MC gateway UE has been authenticated and authorized by the corresponding MC service server, the MC service user is authorized MC gateway user.

The MC gateway UE sever enables the following functions:

-	Authentication of the MC gateway clients including the MC gateway UE function of the corresponding MC service server

-	The following functions are available when the connection authorization of the MC gateway client is successful:

-	Relay of signalling between MC client and MC service server/CSC server

-	Relay of signaling between the signaling user agent residing on the non-3GPP device and the SIP core

-	Relay of signaling between the HTTP client residing on the non-3GPP device and the HTTP proxy

-	Media plane forwarding between the MC service server and the MC clients

The MC gateway UE function as part of the corresponding MC service server enables the use of an MC gateway UE with a corresponding MC gateway UE function that authorizes the association between the MC gateway client/MC client and the corresponding MC gateway UE to enable forwarding of the media plane to MC clients via the MC gateway UE.

Figure 11.2.1-1: Functional model of MC gateway UE signalling plane