3GPP TSG-SA WG6 Meeting #59	S6-240135

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0212 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of GW MC service ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The GW MC service ID is not a correct identity used in the MC gateway UE case and the corresponding connection authorization procedures and information flows defined in TS 23.280. It is should be the permitted MC service ID of the MC gateway UE being configured only at the MCVideo server, so the MCVideo server can based on this item to authorize whether allows a requesting MC service user to use a MC gateway UE, i.e., the MC service ID of the MC gateway UE is in the list of permitted gateway UEs. - In current TS 23.281, such MC service ID should be MCVideo ID - It is possible that the premitted MC gateway UEs are different from different MCVideo users. So such configuration should be in the MCVideo user profile, rather than the MCVideo service configuration |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the List of permitted GW MC service ID(s) item from table A.5-2 - Add the List of permitted MC gateway UEs to use into table A.3-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without correct configuration, the connection authorization is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3, A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************** 1st changes ****************************/

A.3	MCVideo user profile configuration data

The general aspects of MC service user profile configuration data are specified in 3GPP TS 23.280 [6]. The MCVideo user profile configuration data is stored in the MCVideo user database. The MCVideo server obtains the MCVideo user profile configuration data from the MCVideo user database (MCVideo-2).

Tables A.3-1 and A.3-2 contain the MCVideo user profile configuration required to support the use of on-network MCVideo service. Tables A.3-1 and A.3-3 contain the MCVideo user profile configuration required to support the use of off-network MCVideo service. Data in table A.3-1 and table A.3-3 can be configured offline using the CSC-11 reference point.

Table A.3-1: MCVideo user profile configuration data (on and off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server | MCVideo user database |
| --- | --- | --- | --- | --- | --- |
| Subclause 5.2.11 of 3GPP TS 23.280 [6] | MCVideo identity (MCVideo ID) | Y | Y | Y | Y |
| 3GPP TS 33.180 [14] | KMSUri for security domain of MCVideo ID (see NOTE 1) | Y | Y | Y | Y |
| Subclause 5.2.11 of 3GPP TS 23.280 [6] | Pre-selected MCVideo user profile indication (see NOTE 2) | Y | Y | Y | Y |
| Subclause 5.2.11 of 3GPP TS 23.280 [6] | MCVideo user profile index | Y | Y | Y | Y |
| Subclause 5.2.11 of 3GPP TS 23.280 [6] | MCVideo user profile name | Y | Y | Y | Y |
| [R-5.17-007], [R-6.13.4-002] of 3GPP TS 22.280 [2] | User profile status (enabled/disabled) |  | Y | Y | Y |
| [R-5.7-001] [R-6.9-003] of 3GPP TS 22.280 [2] | Authorised to create and delete aliases of an MCVideo user and its associated user profiles. |  |  | Y | Y |
| [R-5.7-002], [R-6.9-003] of 3GPP TS 22.280 [2] | Alphanumeric aliases of user | Y | Y | Y | Y |
| [R-5.1.1-005], [R-5.9-001] of 3GPP TS 22.280 [2] | Participant type of the user | Y | Y | Y | Y |
| [R-5.1.8-006], [R-5.3-002], [R-5.9-001], [R-5.16.2-001], [R-5.16.2-002] of 3GPP TS 22.280 [2] | User's Mission Critical Organization (i.e. which organization a user belongs to) | Y | Y | Y | Y |
| [R-5.2.2-003] and [R-6.6.3-002] of 3GPP TS 22.280 [2] | Authorisation to create a group-broadcast group (see NOTE 8) |  |  | Y | Y |
| [R-5.2.2-003] and [R-6.6.3-002 of 3GPP TS 22.280 [2] | Authorisation to create a user-broadcast group (see NOTE 8) |  |  | Y | Y |
| [R-5.6.2.4.1-002] of 3GPP TS 22.280 [2] | Authorised to activate MCVideo emergency alert | Y | Y | Y | Y |
| [R-5.6.2.4.1-013] of 3GPP TS 22.280 [17] | Automatically trigger a MCVideo emergency communication after initiating the MCVideo emergency alert | Y | Y | Y | Y |
| [R-5.6.2.4.1-004] [R-5.6.2.4.1-008] [R-5.6.2.4.1-012] of 3GPP TS 22.280 [2] | Group used on initiation of an MCVideo emergency group call (see NOTE 5) | Y | Y | Y | Y |
| [R-5.6.2.4.1-004], [R-5.6.2.4.1-008], [R-5.6.2.4.1-012] of 3GPP TS 22.280 [17] | Recipient for an emergency private MCVideo call (see NOTE 5) |  |  |  |  |
|  | > MCVideo ID | Y | Y | Y | Y |
| 3GPP TS 33.180 [19] | > KMSUri for security domain of MCVideo ID (see NOTE 1) | Y | Y | Y | Y |
| [R-5.6.2.4.2-002] of 3GPP TS 22.280 [2] | Authorisation to cancel an MCVideo emergency alert | Y | Y | Y | Y |
| [R-6.15.6.2-002] of 3GPP TS 22.280 [2] | Authorised to activate an MCVideo ad hoc group emergency alert | Y | Y | Y | Y |
| [R-6.15.6.2-006] of 3GPP TS 22.280 [2] | Authorisation to cancel an MCVideo ad hoc group emergency alert | Y | Y | Y | Y |
| [R-6.15.6.2-007] of 3GPP TS 22.280 [2] | Authorised to set up an MCVideo group call using the ad hoc group used for the alert | Y | Y | Y | Y |
|  | Authorised to receive the participants information of an MCVideo ad hoc group emergency alert | N | Y | Y | Y |
|  | Authorised to modify the list of participants and criteria for an MCVideo ad hoc group emergency alert | Y | Y | Y | Y |
| [R-5.1.2.1.2-004] of 3GPP TS 22.281 [3] | Authorisation to modify the video settings of the transmitted video stream of another MCVideo User | Y | Y | Y | Y |
| [R-5.1.2.1.2-006] of 3GPP TS 22.281 [3] | Authorisation to renegotiate a codec during a video transmission. | Y | Y | Y | Y |
| [R-5.1.2.1.2-007] of 3GPP TS 22.281 [3] | Authorisation to remotely control the video capabilities or parameters for a camera on an MCVideo UE | Y | Y | Y | Y |
| [R-5.1.2.2.2-001] of 3GPP TS 22.281 [3] | Authorisation to remotely control the video capabilities or parameters of a remote MCVideo UE | Y | Y | Y | Y |
| [R-5.1.2.2.2-004] of 3GPP TS 22.281 [3] | Authorisation to receive and display the capabilities of a remote MCVideo UE | Y | Y | Y | Y |
| [R-5.1.3.1.2-004] of 3GPP TS 22.281 [3] | Authorisation to remotely activate another MCVideo User's camera | Y | Y | Y | Y |
| [R-5.1.9.2.2-002] of 3GPP TS 22.281 [3] | Authorisation to push a video to another MCVideo user . | Y | Y | Y | Y |
| [R-5.1.9.2.2-003] of 3GPP TS 22.281 [3] | Authorisation to enable and to disable the automatic sending of notification to a second MCVideo User that a video is being pushed to a third MCVideo User | Y | Y | Y | Y |
| [R-5.1.9.2.2-004] of 3GPP TS 22.281 [3] | List of MCVideo users for whom to receive notifications about video being pushed to them | Y | Y | Y | Y |
|  | > MCVideo IDs |  |  |  |  |
| [R-5.1.9.2.2-005] of 3GPP TS 22.281 [3] | List of specific video categories to receive (see NOTE 3) | Y | Y | Y | Y |
|  | > Video categories |  |  |  |  |
| [R-6.7.3-007] of 3GPP TS 22.280 [2] | List of user(s) who can be called in MCVideo private call |  |  |  |  |
|  | > MCVideo ID | Y | Y | Y | Y |
|  | > User info ID | Y | N | Y | Y |
|  | > ProSe discovery group ID | Y | N | Y | Y |
|  | > Presentation priority relative to other users and groups (see NOTE 4) | Y | Y | Y | Y |
| 3GPP TS 33.180 | > KMSUri for security domain of MCVideo ID (see NOTE 1) | Y | Y | Y | Y |
| [R-6.7.3-007] of 3GPP TS 22.280 [2] | Authorised to make a private video call towards users not included in "list of user(s) who can be called in MCVideo private call" | Y | Y | Y | Y |
| [R-5.1.10.2-002] of 3GPP TS 22.281 [3] | List of category tags |  |  |  |  |
|  | > Video category tag | Y | Y | Y | Y |
| [R-5.1.10.2-002] of 3GPP TS 22.281 [3] | Authorization to query MCVideo client | Y | Y | Y | Y |
| [R-5.1.3.2.2-004] [R-5.1.10.2-002] of 3GPP TS 22.281 [3] | List of category tags that authorized to query MCVideo client |  |  |  |  |
|  | > Video category tag for query | Y | Y | Y | Y |
| [R-5.1.3.2.2-004] [R-5.1.10.2-002] of 3GPP TS 22.281 [3] | List of geography areas that authorized to query MCVideo client |  |  |  |  |
|  | > Geography area to query | Y | Y | Y | Y |
| [R-5.1.1.1-015] of 3GPP TS 22.281 [3] | Authorization to perform video adaptation | Y | Y | Y | Y |
| [R-5.1.7-002] and [R-6.8.7.2-007] and [R-6.8.7.2-008] of 3GPP TS 22.280 [2] | Priority of the user (see NOTE 6) |  | Y | Y | Y |
| [R-6.12-003] of 3GPP TS 22.280 [17] | Authorised to restrict the dissemination of the location information | Y | Y | Y | Y |
| Subclause 10.9 of 3GPP TS 23.280 [6] | Authorised to request location information of another user in the primary MCVideo system (see NOTE 7) | Y | Y | Y | Y |
| Subclause 10.9 of 3GPP TS 23.280 [6] | List of partner MCVideo systems for which user is authorised to request location information for another user |  |  |  |  |
|  | > Identity of partner MCVideo system | Y | Y | Y | Y |
| NOTE 1: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. NOTE 2: As specified in 3GPP TS 23.280 [6], for each MCVideo user's set of MCVideo user profiles, only one MCVideo user profile shall be indicated as being the pre-selected MCVideo user profile. NOTE 3: If this list is blank then this implies that all video categories are acceptable for the MCVideo user. NOTE 4: The use of this parameter by the MCVideo UE is outside the scope of the present document. NOTE 5: This parameter is used for the emergency communication and also used as a target of the emergency alert request. At most one of them is configured; i.e. emergency communication will go to either a group or a user. If both are not configured the MCVideo user's currently selected group will be used. NOTE 6: The use of the parameter is left to implementation. NOTE 7: Further differentiation on authorisation for requesting location information based on detailed characteristics (e.g. MC organization, MC service ID, functional alias) is left to implementation. NOTE 8: This parameter applies to temporary broadcast groups built from regrouping mechanism. This authorisation automatically sets the originator of the temporary group as the only transmitting party. |  |  |  |  |  |

Table A.3-2: MCVideo user profile configuration data (on network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server | MCVideo user database |
| --- | --- | --- | --- | --- | --- |
| [R-5.1.5-001], [R-5.1.5-002], [R-5.10-001], [R-6.4.7-002], [R-6.8.1-008], [R-6.7.4-002] of 3GPP TS 22.280 [2] | List of on-network MCVideo groups for use by an MCVideo user |  |  |  |  |
|  | > MCVideo Group ID | Y | Y | Y | Y |
|  | > Application plane server identity information of group management server where group is defined |  |  |  |  |
|  | >> Server URI | Y | Y | Y | Y |
|  | > Application plane server identity information of identity management server which provides authorization for group (see NOTE 1) |  |  |  |  |
|  | >> Server URI | Y | Y | Y | Y |
| 3GPP TS 33.180 [14] | > KMSUri for security domain of group (see NOTE 4) | Y | Y | Y | Y |
|  | > Presentation priority of the group relative to other groups and users (see NOTE 2) | Y | Y | Y | Y |
| Subclause 5.2.5 of 3GPP TS 23.280 [6] | List of groups user implicitly affiliates to after MCVideo service authorization for the user |  |  |  |  |
|  | > MCVideo Group ID | Y | Y | Y | Y |
| [R-6.4.2-006] of 3GPP TS 22.280 [2] | Authorisation of an MCVideo user to request a list of which MCVideo groups a user has affiliated to |  | Y | Y | Y |
| [R-6.4.6.1-002], [R-6.4.6.1-003] of 3GPP TS 22.280 [2] | Authorisation to change affiliated groups of other specified user(s) |  | Y | Y | Y |
| [R-6.4.6.2-001], [R-6.4.6.2-002] of 3GPP TS 22.280 [2] | Authorisation to recommend to specified user(s) to affiliate to specific group(s) |  | Y | Y | Y |
| [R-6.6.1-004] of 3GPP TS 22.280 [2] | Authorisation to perform regrouping | Y | Y | Y | Y |
| [R-6.7.2-001] of 3GPP TS 22.280 [2] | Presence status is available/not available to other users | Y | Y | Y | Y |
| [R-6.7.1-002], [R-6.7.2-002] of 3GPP TS 22.280 [2] | List of MCVideo users that MCVideo user is authorised to obtain presence of |  |  |  |  |
|  | > MCVideo IDs | Y | Y | Y | Y |
| [R-6.8.7.4.2-001], [R-6.8.7.4.2-002] of 3GPP TS 22.280 [2] | Authorisation of a user to cancel an emergency alert on any MCVideo UE of any user |  | Y | Y | Y |
| [R-6.13.4-001] of 3GPP TS 22.280 [2] | Authorisation for an MCVideo user to enable/disable an MCVideo user |  | Y | Y | Y |
| [R-6.13.4-003], [R-6.13.4-005], [R-6.13.4-006], [R-6.13.4-007] of 3GPP TS 22.280 [2] | Authorisation for an MCVideo user to (permanently /temporarily) enable/disable a UE |  | Y | Y | Y |
| [R-7.14-002], [R-7.14-003] of 3GPP TS 22.280 [2] | Authorization for manual switch to off-network while in on-network | Y | Y | Y | Y |
| [R-5.1.5-004] of 3GPP TS 22.280 [2] | Limitation of number of affiliations per user (Nc2) | N | Y | Y | Y |
| [R-6.4.6.1-001], [R-6.4.6.1-004] of 3GPP TS 22.280 [2] | List of MCVideo users whose selected groups are authorized to be remotely changed |  |  |  |  |
|  | > MCVideo ID | Y | Y | Y | Y |
| [R-5.2.3.2-002], [R-5.2.3.2-003] of 3GPP TS 22.281 [3] | Period after which MCVideo data on a MCVideo UE is to be deleted if no action is taken by an authorized MCVideo user | Y | Y | Y | Y |
| [R-5.2.6.2.2-004] of 3GPP TS 22.281 [3] | Maximum number of simultaneous video streams that can be received (see NOTE 3) | Y | Y | Y | Y |
| [R-5.2.6.2.2-005] of 3GPP TS 22.281 [3] | Authorisation to automatically receive video communications | Y | Y | Y | Y |
| [R-5.2.6.2.2-006] of 3GPP TS 22.281 [3] | Authorisation to automatically receive emergency video streams | Y | Y | Y | Y |
| [R-5.2.6.2.2-007] of 3GPP TS 22.281 [3] | Authorisation to automatically receive imminent peril video streams | Y | Y | Y | Y |
| [R-5.2.6.2.2-008] of 3GPP TS 22.281 [3] | List of MCVideo groups for which video can be automatically/mandatorily received |  |  |  |  |
|  | > MCVideo group IDs | Y | Y | Y | Y |
| [R-5.2.7.2-001] of 3GPP TS 22.281 [3] | Authorisation to request to override an active MCVideo transmission | Y | Y | Y | Y |
| [R-5.2.7.2-002] of 3GPP TS 22.281 [3] | Authorisation to select MCVideo transmissions that can be overridden | Y | Y | Y | Y |
| [R-5.2.7.2-004] of 3GPP TS 22.281 [3] | Authorisation to allow MCVideo private communications to override or not to override active MCVideo group communications | Y | Y | Y | Y |
| [R-5.2.8-005], [R-5.2.8-006] of 3GPP TS 22.281 [3] | Maximum length of time of a single video transmission | Y | Y | Y | Y |
| [R-6.7.3-007a] of 3GPP TS 22.280 [2] | List of user(s) from which MCVideo private calls can be received |  |  |  |  |
|  | > MCVideo ID | Y | Y | Y | Y |
|  | > Presentation priority relative to other users and groups | Y | Y | Y | Y |
| 3GPP TS 33.180 [18] | > KMSUri for security domain of MCVideo ID | Y | Y | Y | Y |
| [R-5.9a-012] of 3GPP TS 22.280 [2] [R-5.9a-013] of 3GPP TS 22.280 [2] | Authorised to request association between active functional alias(es) and MCVideo ID(s) |  | Y | Y | Y |
| Subclause 5.2.9 of 3GPP TS 23.280 [16] | List of partner MCVideo systems in which this profile is valid for use during migration |  |  |  |  |
| Subclause 5.2.9 of 3GPP TS 23.280 [16] | > Identity of partner MCVideo system | Y | N | Y | Y |
| Subclause 10.1.1 of 3GPP TS 23.280 [16] | > Access information for partner MCVideo system (see NOTE 5) | Y | N | Y | Y |
| [R-6.6.4.2-002a] and [R-6.6.4.2-002b] of 3GPP TS 22.280 [2] | List of groups the client affiliates/de-affiliates when criteria is met |  |  |  |  |
|  | > MCVideo Group ID | Y | Y | Y | Y |
|  | >> Criteria for affiliation (see NOTE 7) | Y | Y | Y | Y |
|  | >> Criteria for de-affiliation (see NOTE 7) | Y | Y | Y | Y |
|  | >> Manual de-affiliation is not allowed if the criteria for affiliation are met | Y | Y | Y | Y |
| [R-6.6.4.2-002] of 3GPP TS 22.280 [2] | List of groups the client affiliates after receiving an emergency alert |  |  |  |  |
|  | > MCVideo Group ID | Y | Y | Y | Y |
|  | >> Manual de-affiliation is not allowed if the criteria for affiliation are met | Y | Y | Y | Y |
| [R-5.9a-012] of 3GPP TS 22.280 [2] | Authorised to take over a functional alias from another MCVideo user | N | Y | Y | Y |
|  | List of functional alias(es) of the MCVideo user |  |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | > Functional alias | Y | Y | Y | Y |
| [R-5.9a-018] of 3GPP TS 22.280 [2] | >> MCVideo server trigger criteria for activation (see NOTE 6) | N | Y | Y | Y |
| [R-5.9a-017], [R-5.9a-018] of 3GPP TS 22.280 [2] | >> MCVideo server trigger criteria for de-activation (see NOTE 6) | N | Y | Y | Y |
| [R-5.9a-019] of 3GPP TS 22.280 [16] | >> MCVideo client trigger criteria for activation (see NOTE 6) | Y | Y | Y | Y |
| [R-5.9a-019] of 3GPP TS 22.280 [16] | >> MCVideo client trigger criteria for de-activation (see NOTE 6) | Y | Y | Y | Y |
|  | >> Manual de-activation is not allowed if the criteria are met (see NOTE 6) | Y | Y | Y | Y |
| [R-5.10-001b] of 3GPP TS 22.280 [16] | Maximum number of successful simultaneous MCVideo service authorizations for this user (see NOTE 8) | N | Y | Y | Y |
| [R-5.9a-020] of 3GPP TS 22.280 [17] | List of functional aliases to which private calls are allowed when using a certain functional alias |  |  |  |  |
|  | > Used functional alias | Y | Y | Y | Y |
|  | >> List of functional aliases which can be called |  |  |  |  |
|  | >>> Functional alias | Y | Y | Y | Y |
| [R-5.9a-021] of 3GPP TS 22.280 [17] | List of functional aliases from which private calls can be received when using a certain functional alias |  |  |  |  |
|  | > Used functional alias | N | Y | Y | Y |
|  | >> List of functional aliases from which calls can be received |  |  |  |  |
|  | >>> Functional alias | N | Y | Y | Y |
|  | Ad hoc group call authorizations |  |  |  |  |
| [R-6.15.5.3-001] of 3GPP TS 22.280 [17] | > Authorised to initiate ad hoc group call | Y | Y | Y | Y |
| R-6.15.5.3-003] of 3GPP TS 22.280 [17] | > Authorised to participate in ad hoc group call | Y | Y | Y | Y |
|  | > Authorised to initiate emergency ad hoc group call | Y | Y | Y | Y |
|  | > Authorised to initiate imminent peril ad hoc group call | Y | Y | Y | Y |
|  | > Authorised to receive the participants information of ad hoc group call | N | Y | Y | Y |
|  | > Authorised to modify the list of participants and criteria for an ad hoc group call | Y | Y | Y | Y |
|  | > Authorised to release ongoing ad hoc group calls | Y | Y | Y | Y |
| Subclause 5.15 of 3GPP TS 22.280 [17] | List of permitted MC gateway UEs to use |  |  |  | Subclause 5.15 of 3GPP TS 22.280 [17] |
|  | > MCVideo ID | N | Y | Y | Y |
| NOTE 1: If this parameter is not configured, authorization to use the group shall be obtained from the identity management server identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. NOTE 2: The use of this parameter by the MCVideo UE is outside the scope of the present document. NOTE 3: The parameter can be set to an unlimited number of simultaneous streams received that can be received. NOTE 4: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. NOTE 5: Access information for each partner MCVideo system comprises the list of information required for initial UE configuration to access an MCVideo system, as defined in table A.6-1 of 3GPP TS 23.280 [16] NOTE 6: The criteria may consist of conditions like the location of the MCVideo user, local time etc. NOTE 7: The criteria may consist of conditions such as the location of the MCVideo user or the active functional alias of the MCVideo user. NOTE 8: If configured, this value has precedence over the system level parameter "maximum number of successful simultaneous service authorisations" in table A.5-2. If not configured, the corresponding parameter from table A.5-2 shall be used. |  |  |  |  |  |

Table A.3-3: MCVideo user profile configuration data (off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server | MCVideo user database |
| --- | --- | --- | --- | --- | --- |
| [R-7.2-003], [R-7.6-004] of 3GPP TS 22.280 [2] | List of off-network MCVideo groups for use by an MCVideo user |  |  |  |  |
|  | > MCVideo Group IDs | Y | N | Y | Y |
|  | > Application plane server identity information of group management server where group is defined |  |  |  |  |
|  | >> Server URI | Y | N | Y | Y |
|  | > Application plane server identity information of identity management server which provides authorization for group (see NOTE 1) |  |  |  |  |
|  | >> Server URI | Y | N | Y | Y |
| 3GPP TS 33.180 [14] | > KMSUri for security domain of group (see NOTE 3) | Y | N | Y | Y |
|  | > Presentation priority of the group relative to other groups and users (see NOTE 2) | Y | N | Y | Y |
| [R-7.12-002], [R-7.12-003] of 3GPP TS 22.280 [2] | Authorization for off-network services | Y | N | Y | Y |
| Subclause 7.18.1 | User info id (as specified in 3GPP TS 23.303 [7]) | Y | N | Y | Y |
| NOTE 1: If this parameter is not configured, authorization to use the group shall be obtained from the identity management server identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. NOTE 2: The use of this parameter by the MCVideo UE is outside the scope of the present document. NOTE 3: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. |  |  |  |  |  |

# /************************** 2nd changes ****************************/

A.5	MCVideo service configuration data

The general aspects of MC service configuration are specified in 3GPP TS 23.280 [6]. The MCVideo service configuration data is stored in the MCVideo server.

Tables A.5-1 and A.5-2 describe the configuration data required to support the use of on-network MCVideo service. Tables A.5-1 and A.5-3 describe the configuration data required to support the use of off-network MCVideo service. Data in table A.5-1 and table A.5-3 can be configured offline using the CSC-11 reference point.

Table A.5-1: MCVideo service configuration data (on and off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.2.2-001] of 3GPP TS 22.280 [2] | Levels of group hierarchy for group-broadcast groups (Bc1) | Y | Y | Y |
| [R-5.2.3-001] of 3GPP TS 22.280 [2] | Levels of user hierarchy for user-broadcast groups (Bc2) | Y | Y | Y |
| [R-5.7-002] of 3GPP TS 22.280 [2] | Minimum length (Nc3) of an alphanumeric identifier (i.e. alias) assigned by an MCVideo administrator. | Y | N | Y |

Table A.5-2: MCVideo service configuration data (on network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.12-001] of 3GPP TS 22.280 [2] | Protect confidentiality of signalling (see NOTE 1) | Y | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | Protect integrity of signalling (see NOTE 1) | Y | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | Use signalling protection between MCVideo servers (see NOTE 1) | N | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | Use transmission control protection between MCVideo servers (see NOTE 1) | N | Y | Y |
| [R-5.10-001a] of 3GPP TS 22.280 [2] | Maximum number of successful simultaneous service authorizations of clients from a user | N | Y | Y |
|  | List of functional alias identities |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | > Functional alias | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | >> Limit number of simultaneous activations | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | >> This functional alias can be taken over | N | Y | Y |
|  | >> List of users who can activate this functional alias |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | >>> MCVideo ID | N | Y | Y |
| [R-5.9a-016] of 3GPP TS 22.280 [2] | >> Communication priority (see NOTE 2) | N | Y | Y |
|  | Ad hoc group call configurations |  |  |  |
| [R-6.15.5.3-005] of 3GPP TS 22.280 [17] | > Support of ad hoc group call (enabled/disabled) (see NOTE 3) | Y | Y | Y |
| [R-6.15.5.3-002] of 3GPP TS 22.280 [17] | > Maximum number of participants allowed to participate in an ad hoc group call | Y | Y | Y |
| [R-6.15.5.3-004] of 3GPP TS 22.280 [17] | > Hang timer for ad hoc group call | N | Y | Y |
|  | > Maximum duration for ad hoc group call | Y | Y | Y |
|  | > List of preferred video codecs for ad hoc group call | Y | Y | Y |
| NOTE 1: Security mechanisms are specified in 3GPP TS 33.180 [14]. NOTE 2: The usage of this parameter by the MCVideo server is up to implementation. NOTE 3: If the support for ad hoc group call is disabled by the MC system then all other configurations related to ad hoc group call are not applicable. |  |  |  |  |

Table A.5-3: MCVideo service configuration data (off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-7.6-001], [R-7.6-002], [R-7.6-003] of 3GPP TS 22.280 [2] | Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values |  |  |  |
|  | > MCVideo private call signalling | Y | N | Y |
|  | > MCVideo private call media | Y | N | Y |
|  | > MCVideo Emergency private call signalling | Y | N | Y |
|  | > MCVideo Emergency private call media | Y | N | Y |
| Subclause 7.5.3.3 | Periodicity interval (in seconds) of Capability announcement (see NOTE) | Y | N | Y |
| NOTE: If the value of the periodicity interval is set to zero, the capability announcement is not sent. |  |  |  |  |