3GPP TSG-SA WG6 Meeting #59	S6-240139

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0213 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support both user list and criteria in a ad hoc group call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Ad hoc can be formed by a list of participants or by criteria provided by an authorized user. The list of participants created by the criteria may not be complete or some necessary users may not be in the group. The authorized user should be able to which users shall stay in the call until the call is released and such user will not be removed by the server based on the criteria if exist. |  |  |  |  |  |  |  |  |
| Summary of change: |  | add a new parameter along with the user list in the ad hoc group call request/modify message from the authorized user, which is used to indicate users shall stay in the call until the call is released and such user will not be removed by the server based on the criteria if exist. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A necessary user may be removed by the MC service server by applying the call criteria. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.19.2.1, 7.19.2.11, 7.19.3.1.1, 7.19.3.1.4, 7.19.3.2.4, 7.19.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************** 1st changes ****************************/

7.19.2.1	Ad hoc group call request (MCVideo client – MCVideo server)

Table 7.19.2.1-1 describes the information flow ad hoc group call request from the MCVideo client to the MCVideo server.

Table 7.19.2.1-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ad hoc group ID (see NOTE 1) | O | The MCVideo group ID to be associated with the ad hoc group communication |
| Encryption supported (see NOTE 2) | O | Indicates whether this ad hoc group call supports end-to-end encryption |
| MCVideo ID list (see NOTE 3, NOTE 6, NOTE 7) | O | MCVideo IDs of the participants being invited for the ad hoc group call |
| > Stay-in indicator | O | Indicate the MCVideo user shall stay in the call until the call is released and such user will not be removed by the MCVideo server based on the call criteria if exist. |
| SDP offer | M | Media parameters of MCVideo clients |
| Implicit transmit media request (see NOTE 4) | O | When originating client requests the transmission of media, this element shall be included |
| Broadcast indicator (see NOTE 5) | O | Indicates that the group call request is for a broadcast group call |
| Imminent peril indicator (see NOTE 5) | O | Indicates that the ad hoc group call request is an MCVideo imminent peril call |
| Emergency Indicator (see NOTE 5) | O | Indicates that the ad hoc group call request is an MCVideo emergency call |
| MCVideo ID list (see NOTE 3) | O | List of participants required to acknowledge the ad hoc group call before start of the audio transmission |
| Location information | O | Location of the calling party. |
| Criteria for determining the participants (see NOTE 6, NOTE 7) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCVideo server for determining the participants e.g., it can be a location based criteria to invite particpants in a particular area |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: If this information element is not included, the MCVideo server assigns an MCVideo ad hoc group ID to be used for the ad hoc group communication. This information element is returned to the calling party to use in the ad hoc group communication. If the request follows an ad hoc group for emergency alert, then this element must be present. NOTE 2: This information element is present and set to true only if this ad hoc group call is encrypted. When the ad hoc group call is initiated with participants provided by the initiator this acts as an indicator that subsequent requests follow targeting the individual participants and carrying the relevant key material. If this information element is set to false or not present, then this ad hoc group call is unencrypted. NOTE 3: This element is included only when the originating client sends the list of participants. NOTE 4: This element is included only when the originating client requests the transmission of media. NOTE 5: If used, only one of these information elements is present. NOTE 6: Only one of these information elements is present. NOTE 7: If the request follows an ad hoc group for emergency alert, then this element is not present. |  |  |

# /************************** 2nd changes ****************************/

7.19.2.11	Modify ad hoc group call participants request (MCVideo client – MCVideo server)

Table 7.19.2.11-1 describes the information flow Modify ad hoc group call participants request from the MCVideo client to the MCVideo server.

Table 7.19.2.11-1: Modify ad hoc group call participants request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the requesting MCVideo user who is authorized to modify the ad hoc group call participants |
| Functional alias | O | The functional alias of the requesting MCVideo user who is authorized to modify the ad hoc group call participants |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call whose participants needs to be modified |
| MCVideo ID list (see NOTE) | O | List of additional MCVideo users to be added to the on-going ad hoc group call |
| > Stay-in indicator | O | Indicate the MCVideo user shall stay in the call until the call is released and such user will not be removed by the MCVideo server based on the call criteria if exist. |
| MCVideo ID list (see NOTE) | O | List of MCVideo users to be removed from the on-going ad hoc group call |
| > Manually remove indicator | O | Indicate the MCVideo user shall be removed and the MCVideo server shall not add it back even if it meets the call criteria. |
| NOTE: Either one or both of these information elements is present |  |  |

# /************************** Next changes ****************************/

7.19.3.1.1	Ad hoc group call setup

Figure 7.19.3.1.1-1 below illustrates the ad hoc group call setup procedure initiated by an authorized user.

Pre-conditions:

1.	The authorized user at MCVideo client 1 wants to invite MCVideo users at MCVideo client 2, MCVideo client 3 and MCVideo client 4 for the ad hoc group call and is aware of the MCVideo IDs of the participants or to invite the ad hoc group member if the ad hoc group call follows an ad hoc group emergency alert, the MCVideo client 1 is aware of the ad hoc group ID.

2.	Number of participants being invited for the ad hoc group call is within the limit.

3.	End-to-End encryption is supported for this ad hoc group call.

4.	The pre-configured group identity and pre-configured group configuration to be used for an ad hoc group have been preconfigured in MCVideo client and other participants of ad hoc group have also received the relevant security related information to allow them to communicate in an ad hoc group communication.

NOTE 1:	Selection of MCVideo IDs of the participants can be manual or from the user profile configuration data or by any other means. This is left for the implementation.

Figure 7.19.3.1.1-1: Ad hoc group call setup

1.	User at MCVideo client 1 would like to initiate an ad hoc group call. The MCVideo client 1 initiates the ad hoc group call by sending the ad hoc group call request containing the list of participants or an ad hoc group ID from an ad hoc group emergency alert to the MCVideo server. Encryption supported information element shall be set to true since end-to-end encryption is supported. An SDP offer containing the MCVideo client media parameters is included. If there is a transmit media request to transmit media, then the ad hoc group call request contains an indication of an implicit transmit media request. If the MCVideo user of MCVideo client 1 has selected a functional alias, then the ad hoc group call request contains that functional alias. If the ad hoc group call request contains an implicit transmit media request it may also include location information.

If the MCVideo user at MCVideo client 1 initiates an MCVideo emergency ad hoc group call or the MCVideo emergency state is already set for the MCVideo client 1 (due to a previously triggered MCVideo emergency alert):

i.	the MCVideo ad hoc group call request shall contain an emergency indicator;

ii.	if the MCVideo emergency state is not set already, MCVideo client 1 sets its MCVideo emergency state. The MCVideo emergency state of MCVideo client 1 is retained until explicitly cancelled by the user of MCVideo client 1.

2.	The MCVideo server accepts the ad hoc group call request if the ad hoc group call is supported and authorized. Otherwise reject the ad hoc group call request and do not continue with the rest of the steps. If authorised, it validates whether the number of invited participants is within the configured limit before proceeding with the call setup.

If functional alias is present, the MCVideo server checks whether the provided functional alias allowed to be used and has been activated for the user.

If location information was included in the ad hoc group call request, the MCVideo server checks the privacy policy of the MCVideo user to decide if the location information of MCVideo client 1 can be provided to other users on the call (refer to Annex A.3 "Authorisation to provide location information to other MCVideo users on a call when talking").

If an emergency indicator is present in the received MCVideo ad hoc group call request, the MCVideo ad hoc group is considered to be in the in-progress emergency state until this ad hoc group call is terminated; and

If an imminent peril indicator is present in the received MCVideo ad hoc group call request, the MCVideo ad hoc group is considered to be in the in-progress imminent peril state until this ad hoc group call is terminated.

If the MC service users' information received in step 1 does not contain an ad hoc group ID from an ad hoc group emergency alert, the MCVideo server forms the ad hoc group by using MCVideo users' information received in step 1, determines the preconfigured group to be used for the configuration of the ad hoc group and assigns a MCVideo group ID for the newly formed ad hoc group.

The MCVideo server considers the ad hoc group call participants as implicitly affiliated to the ad hoc group.

3.	The MCVideo server shall send the ad hoc group call request return message to MCVideo client 1 containing the below:

i.	The MCVideo ad hoc group ID either generated by the MCVideo server or provided by the MCVideo client 1 if the ad hoc group ID is from an ad hoc group emergency alert (only included when the ad hoc group call is authorized);

ii.	The group ID of the pre-configured group to be used for the ad hoc group call (only included when the ad hoc group call is authorized); and

iii.	Result of whether the ad hoc group call is authorized or not.

If the ad hoc group call request is not authorized, MCVideo client 1 shall not proceed with the rest of the steps.

4a-4c.	The MCVideo server sends the ad hoc group call requests towards the MCVideo clients of the invited users based on step 1. While sending the ad hoc group call requests, the MCVideo server shall remove the information elements that are not required to be conveyed to the target MCVideo clients (e.g. MCVideo ID list of the users who are required to acknowledge)

5a-5c.	The receiving MCVideo clients are notified about the incoming ad hoc group call.

6a-6c.	The receiving MCVideo clients accept the ad hoc group call requests and send ad hoc group call responses to the MCVideo server. The response may also contain a functional alias of the responding MCVideo user, which is verified (valid and activated for the user) by the MCVideo server.

7.	The MCVideo server sends the ad hoc group call response to MCVideo client 1 through the signalling path to inform about successful call establishment. If the stay-in indicator is provided along with the MCVideo IDs of the participants to be invited in step 1, the MCVideo server stores such stay-in indicator for the corresponding participant. If criteria are added to the call later, the participant with stay-in indicator shall be kept in the call and not be removed by the MCVideo server even if it does not meet the criteria.

NOTE 2:	The participant with stay-in indicator can be removed by the authorized user via the modify ad hoc group call participants request.

NOTE 3:	Steps 5 to step 7 can start to occur before all of step 4 is completed since the MCVideo server do not require to wait for the previous ad hoc group call request to complete before sending the ad hoc group request to another participant.

8.	If the initiating MCVideo user requires the acknowledgement from the invited MCVideo users, and the required MCVideo users do not acknowledge the call setup within a configured time (the "acknowledged call setup timeout"), then the MCVideo server may proceed with or abandon the call and then notify the initiating MCVideo user that the acknowledgements did not include all required members according to ad hoc group call policy from the user profile configuration. The MCVideo server may notify the initiating MCVideo user of all MCVideo users who did not acknowledge the ad hoc group call request within the configured time. This notification may be sent to the initiating MCVideo user by the MCVideo server more than once during the call when MCVideo users join or leave the MCVideo ad hoc group call.

9.	MCVideo client 1, MCVideo client 2, MCVideo client 3 and MCVideo client 4 establish media plane and transmission control resources.

NOTE 4:	Step 9 can occur any time following step 7 if the conditions to proceed with the call are met.

# /************************** Next changes ****************************/

7.19.3.1.4	Modification of ad hoc group call participants by an authorized user

Figure 7.19.3.1.4-1 below illustrates the modification of ad hoc group call participants procedure by an authorized user.

Pre-conditions:

1.	An ad hoc group call is already in progress and the participants list is provided by the originating MCVideo user while initiating the MCVideo ad hoc group call.

2.	The participants of the MCVideo ad hoc group call belong to the single MCVideo system.

3.	The MCVideo users on MCVideo client 1, MCVideo client 3 to MCVideo client n are on an ongoing ad hoc group call.

4.	The MCVideo user at MCVideo client 1 determines to remove the user of MCVideo client 3 from the ad hoc group call and add user of MCVideo client 2 into the on-going ad hoc group call.

Figure 7.19.3.1.4-1: Modification of ad hoc group call participants by an authorized user

1.	The MCVideo user at the MCVideo client 1 is authorized and requests to modify ad hoc group call participants. The MCVideo client 1 sends the modify ad hoc group call participants request to the MCVideo server in order to remove MCVideo client 3 from the ongoing ad hoc group call and add MCVideo client 2 into it.

2.	The MCVideo server verifies whether the MCVideo client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call.

3.	The MCVideo server sends modify ad hoc group call participants response to the MCVideo client 1.

4.	The MCVideo server sends the ad hoc group call leave request to the MCVideo client 3 in order to remove it from the on-going ad hoc group call. If the manually remove indicator is provided along with the MCVideo ID of the MCVideo client 3 and there are criteria used in the call, the MCVideo server removes MCVideo client 3 and marks it as manually removed so that the MCVideo server will not add it back to the call based on the criteria.

NOTE 1:	The participant with manually remove indicator being removed can be added back by the authorized user via the modify ad hoc group call participants request.

5.	The MCVideo client 3 notifies the user of the ad hoc group call leave request.

6.	The MCVideo client 3 sends the ad hoc group call leave response to the MCVideo server.

7.	The MCVideo server sends the ad hoc group call request towards MCVideo client 2. If the stay-in indicator is provided along with the MCVideo IDs of the participants to be invited in step 1, the MCVideo server stores such stay-in indicator for the corresponding participant. The participant with stay-in indicator shall be kept in the call and not be removed by the MCVideo server even if it does not meet the call criteria if exists.

NOTE 2:	The participant with stay-in indicator can be removed by the authorized user via the modify ad hoc group call participants request.

NOTE 3:	Steps 7 to 9 can occur at any time following step 3.

8.	The receiving MCVideo client 2 notifies the user about the incoming ad hoc group call.

9.	The MCVideo client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCVideo server. The response may also contain a functional alias of the responding MCVideo user, which is verified (valid and activated for the user) by the MCVideo server. The MCVideo server considers the MCVideo user as implicitly affiliated to the ad hoc group.

10.	The MCVideo server may notify the initiating MCVideo user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCVideo user by the MCVideo server more than once during the call when MCVideo users join or leave the ad hoc group call.

11.	The MCVideo server may notify the participants about the change in the participants of on-going ad hoc group call.

# /************************** Next changes ****************************/

7.19.3.2.4	Procedure for ad hoc group call setup – Participants list provided by the Initiator

Figure 7.19.3.2.4-1 illustrates the procedure for ad hoc group call setup procedure initiated by an authorized user wherein either a list of participants or an ad hoc group ID from an ad hoc group emergency alert is provided by the authorised user and the determined MCVideo users are from multiple MCVideo systems.

Pre-conditions:

1.	The security aspects of sharing the user information between primary and partner MC systems shall be governed as per the service provider agreement between them. In this case, it is considered that the partner MC system shares their users' information to the primary MC system.

2.	The authorized MCVideo user/dispatcher belongs to the primary MC system.

3.	The MCVideo server of the primary MC system is where the authorized MCVideo user/dispatcher controls the ad hoc group.

4.	Some users of the ad hoc group belong to partner MC systems.

5.	The preconfigured group identity and preconfigured group configuration to be used for an ad hoc group have been preconfigured in the MCVideo client and other participants of ad hoc group have also received the relevant security related information to allow them to communicate in an ad hoc group communication.

6.	If an ad hoc group ID from an ad hoc group emergency alert is used to invite the ad hoc group members, MCVideo client is aware of the ad hoc group ID.

Figure 7.19.3.2.4-1: Ad hoc group call setup

1.	The MCVideo client of the authorized user initiates an ad hoc group call with multiple users from primary and partner MC systems. An ad hoc group call request message with the information of the MCVideo IDs or an ad hoc group ID if the ad hoc group call follows an ad hoc group emergency alert is routed to the MCVideo server of the primary MC system.

2.	If the ad hoc group call is supported, the MCVideo server of the primary MC system verifies whether the user at MCVideo client is authorized to initiate an ad hoc group call. If not authorized, the MCVideo server of the primary MC system rejects the ad hoc group call request as specified in the step 3.

If the information received in the request in step 1 does not contain an ad hoc group ID from an ad hoc group emergency alert, the MCVideo server of the primary MC system forms the ad hoc group by using received information. The MCData server further determines the preconfigured group to be used for the configuration and assigns an MCVideo group ID for the newly formed ad hoc group.

If no MCVideo ad hoc group ID was included in the ad hoc group call request of step 1, or if the provided MCVideo ad hoc group ID is not accepted by the MCVideo server, the MCVideo server assigns a MCVideo group ID for the newly formed ad hoc group

If the MC service users' information received in step 1 does not contain an MCVideo ad hoc group ID from an ad hoc group emergency alert, the MCVideo server of the primary MC system forms the ad hoc group by using MCVideo users' information received in step 1, determines the preconfigured group to be used for the configuration of the ad hoc and assigns a MCVideo group ID for the newly formed ad hoc group.

The MCVideo server identifies the appropriate MCVideo server responsible for the MCVideo users of the ad hoc group. The MCVideo server considers the ad hoc group call participants as implicitly affiliated to the ad hoc group.

NOTE 1:	The ad hoc group information including the MCVideo group ID and the list of users is held in dynamic data in the MCVideo server.

3.	The MCVideo server shall send the ad hoc group call request return message to the MCVideo client containing the below:

i.	The MCVideo ad hoc group ID either generated by the MCVideo server or provided by the MCVideo client if the ad hoc group ID is from an ad hoc group emergency alert (only included when the ad hoc group call is authorized);

ii.	The group ID of the pre-configured group to be used for the ad hoc group call (only included when the ad hoc group call is authorized); and

iii.	Result of whether the ad hoc group call is authorized or not.

If the ad hoc group call request is not authorized, the MCVideo client shall not proceed with the rest of the steps.

4.	The MCVideo server of the primary MC system sends the ad hoc group call request to the affiliated group members of the ad hoc group belonging to the primary MC system.

5.	The MCVideo clients receive in the ad hoc group call request the information of the MCVideo group ID for the ad hoc group and further notify their corresponding MCVideo user. The affiliated group members of the ad hoc group of the primary MC system may accept or reject the call and respond with the group call response.

6.	The primary MCVideo server further initiates an ad hoc group call request message to the MCVideo users of the partner MC system. The ad hoc group call request message is routed to the MCVideo users via the MCVideo server of the partner MC system.

7.	The MCVideo clients receive in the ad hoc group call request the information of the MCVideo group ID for the ad hoc group and further notify their corresponding MCVideo user. The MCVideo users upon receipt of the invitation may accept or reject the call, and respond with the ad hoc group call response. The ad hoc group call response message is routed to the MCVideo server of the primary MC system via the MCVideo server of the partner MC system.

8.	The MCVideo server of the primary MC system provides an ad hoc group call response message to the MCVideo client of the authorized MCVideo user upon receiving response to the corresponding ad hoc group call request in step 1. The ad hoc group call response will consist of the success or failure result and/or detailed reason information in case of failure. If the stay-in indicator is provided along with the MCVideo IDs of the participants to be invited in step 1, the MCVideo server stores such stay-in indicator for the corresponding participant. If criteria are added to the call later, the participant with stay-in indicator shall be kept in the call and not be removed by the MCVideo server even if it does not meet the criteria.

NOTE 2:	The participant with stay-in indicator can be removed by the authorized user via the modify ad hoc group call participants request.

NOTE 3:	The ad hoc group call response message is triggered depending on the conditions to proceed with the call.

9.	Upon successful ad hoc group call setup, a group call is established amongst the multiple group members from primary and partner MC systems. The media plane and transmission control resources are established.

# /************************** Next changes ****************************/

7.19.3.2.6	Modification of ad hoc group call participants by an authorized user

Figure 7.19.3.2.6-1 below illustrates the MCVideo user modifying the ad hoc group call participants procedure involving multiple MCVideo systems.

Pre-conditions:

1.	An MCVideo ad hoc group call is already in progress and the participants list provided by the originating MCVideo user while initiating the MCVideo ad hoc group call.

2.	The participants of the MCVideo ad hoc group call belongs to the multiple MCVideo system.

3.	The MCVideo user at MCVideo client 1 determines that the MCVideo client 3 needs to be invited to on-going ad hoc group call and MCVideo client 2 needs to be removed from the on-going ad hoc group call from the primary MCVideo system.

4.	The MCVideo user at MCVideo client 1 determines that the MCVideo client 5 needs to be invited to on-going ad hoc group call and MCVideo client 4 needs to be removed from the on-going ad hoc group call from the partner MCVideo system.

Figure 7.19.3.2.6-1: Modification of ad hoc group call participants by an authorized user

1.	The MCVideo client 1 sends the modify ad hoc group call participants request to the MCVideo server 1 to remove both MCVideo client 2 and MCVideo client 4 from the on-going ad hoc group call and to add both MCVideo client 3 and MCVideo client 5 into on-going ad hoc group call.

2.	The MCVideo server 1 verifies whether the MCVideo client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call.

3.	The MCVideo server 1 sends modify ad hoc group call participants response to the MCVideo client 1.

4.	The MCVideo server 1 determines that MCVideo client 1 has requested that MCVideo client 3 from the primary MC system be invited to an on-going ad hoc group call and successfully added to the call according to the procedure defined in clause 7.19.3.1.4.

5.	The MCVideo server 1 determines that MCVideo client 1 has requested that MCVideo client 2 from the primary MC system be removed from the on-going ad hoc group call and successfully removed from call according to the procedure defined in clause 7.19.3.1.4.

6.	The MCVideo server 1 determines that MCVideo client 1 has requested that MCVideo client 5 from the partner MC system be invited to an on-going ad hoc group call. The MCVideo server 1 sends the ad hoc group call request to the MCVideo client 5. If the stay-in indicator is provided along with the MCVideo IDs of the participants to be invited in step 1, the MCVideo server stores such stay-in indicator for the corresponding participant. The participant with stay-in indicator shall be kept in the call and not be removed by the MCVideo server even if it does not meet the call criteria if exists.

NOTE 2:	The participant with stay-in indicator can be removed by the authorized user via the modify ad hoc group call participants request.

7.	The MCVideo client 5 notifies the user about the incoming ad hoc group call.

8.	The MCVideo client 5 accepts the ad hoc group call request and sends the ad hoc group call response to the MCVideo server 1.

9.	The on-going ad hoc group call is updated by adding MCVideo client 5 which is based on the modify participant list provided by the MCVideo client 1.

10.	The MCVideo server 1 determines that MCVideo client 1 has requested that MCVideo client 4 from the partner MC system be removed from the on-going ad hoc group call. The MCVideo server 1 sends the ad hoc group call leave request to the MCVideo client 4.

11.	The MCVideo client 4 notifies the user of the ad hoc group call leave request.

12.	The MCVideo client 4 sends the ad hoc group call leave response to the MCVideo server 1.

13.	The on-going ad hoc group call is updated by removing MCVideo client 4, which is based on the modify participant list provided by the MCVideo client 1.

NOTE 3:	The ad hoc group call request and response exchanged between MCVideo server 1 of primary MC system and MCVideo client 4/MCVideo client 5 will always traversal through the MCVideo server 2.

14.	The MCVideo server 1 may notify the initiating MCVideo user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCVideo user by the MCVideo server more than once during the call when MCVideo users join or leave the ad hoc group call.

15.	The MCVideo server 1 may notify the authorized participants about the change in the participants list of on-going ad hoc group call.