3GPP TSG-SA WG6 Meeting #59	S6-240152

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

Source:	China Mobile

Title:	New KI on Multi-modal Service ID negotiation

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

New KI on Multi-modal Service ID negotiation.

2. Reason for Change

As defined in TS 23.501, The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, for each data flow that belongs to the multi-modal service, a Multi-modal Service ID, the service requirements and the QoS monitoring requirements:

-	The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data flows that are part of a specific multi-modal application.

-	The AF may provide QoS monitoring requirements for data flows associated to a multi-modal service to the PCF . The PCF generates the authorized QoS Monitoring policy for each data flow.

The Multi-modal Service ID is supposed to be provided by AF to associate the data flows that belong to the multi-modal service. In the case that data flows come from multiple ASs(e.g., video from a camera, and motion data from GamePad), a mechanism for AF/ASs to negotiate the Multi-modal Service ID is needed. It would be more efficient if SA6 provides Multi-modal Service ID negotiation service, rather than having the AFs negotiate the Multi-modal Service ID among themselves one by one.

This paper proposes to study whether and how to provide that service from Application enabler layer.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change * * * *

4.x	Key issue #x: Multi-modal Service ID negotiation

The Multi-modal Service ID is supposed to be provided by AF to associate the data flows that belong to the multi-modal service. In the case that data flows come from multiple ASs (e.g., video from a camera, and motion data from GamePad), a mechanism for AF/ASs to negotiate the Multi-modal Service ID is needed. It would be more efficient if SA6 provides Multi-modal Service ID negotiation service, rather than having the AFs negotiate the Multi-modal Service ID among themselves one by one.

The Key Issue studies the following aspect:

-	Whether and how the application enabler layer provides Multi-modal Service ID negotiation service.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>