3GPP TSG-SA WG6 Meeting #59	S6-240154

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

Source:	China Mobile

Title:	Sol for KI#1 & KI#2: multi-modal flows alignment and monitoring

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

- This paper proposes solution for KI#1 & KI#2 by proposing multi-modal flows alignment and monitoring.

2. Reason for Change

- As discussed in KI#1, according to clause 6.43 of TS 22.261 [2], synchronization between different media components is crucial for immersive multi-modal VR applications. In other words, the delay difference among the associated flows should be small. Otherwise, the arrived flows need to wait for other associated flows that do not arrived yet.

- Also in KI #2, “Whether and how to support the E2E multi-modal communication flows between application clients and application servers within the application enablement layer?” are proposed to be studied.

- This solution aims to monitor and alleviate the delay difference to support the E2E multi-modal communication flows by proposing multi-modal flows alignment and monitoring.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change * * * *

5.X	Multi-modal flows alignment and monitoring requirements

5.X.1	Description

This subclause specifies the requirements for multi-modal flows alignment and monitoring service.

5.X.2	Requirements

[AR-5.x.2-a] The XRApp shall provide a mechanism to enable multi-modal flows alignment by the authorized VAL server.

[AR-5.x.2-b] The XRApp shall provide a mechanism to enable multi-modal flows alignment monitoring by the authorized users or VAL server.

* * * Next Change * * * *

7.1	Mapping of solutions to key issues

Table 7.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #X | X | X |  |  |
| Sol #2 |  |  |  |  |

7.x	Solution #x: Multi-modal flows alignment and monitoring

7.x.1	Architecture Impacts

- This solution requires a mechanism for data storage to cache the traffic flows.

- x.2	Solution description

7.x.2.1	General

- Synchronization between different multi-modal XR application components is crucial. In other words, the delay difference among the associated flows should be small. Otherwise, the arrived flows need to wait for other associated flows that do not arrived yet.

- However, due to the insufficient resource or some unpredictable reasons, (R)AN and Internet are the most likely places to introduce delay difference.

- To avoid the downlink traffic flow delay difference introduced from Internet, the XRApp server located at edge which is last hop of the DN and close to UPF could do the multi-modal traffic flow alignment, by steering associated multi-modal traffic flows to be transferred at specified time.

- As shown in figure 7.x.2.1-1, flow#1 and flow#2 are associated flows. They are sent out from VAL server at the same time, while flow#1 arrives at XRApp server earlier than flow#2 due to different routing path. To align those associated flows, the XRApp server holds and buffers the flow#1 for a while. Until all the associated flows arrived(i.e., flow#2), the XRApp server sends out those associated flows at the same time. In addition, the XRApp server could request the AF session with required QoS procedures as defined in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502, to alleviate the delay difference in 5GC and (R)AN by trigger the PDU Set integrated handling.

Figure 7.x.2.1-1: DL traffic flow alignment

7.x.2.2	Procedure of flow alignment

- Figure 7.x.2.2-1illustrates the procedures of multi-modal flows alignment.

- Precondition:

-	The VAL server can discover and select the XRApp server by CAPIF functions.

XRApp client

5GC

XRApp server

1. multi-modal flows coordination request

VAL server

2. flows coordination

3. AF session with required QoS

4. multi-modal flows coordination response

connection established

-	The connection has been established among the UE, XRApp server and VAL server.

Figure 7.x.2.2-1: multi-modal flows alignment

- VAL server sends the multi-modal flows alignment request with VAL ID, flows description, flows transmission requirement, flows alignment requirement. Flows transmission requirement including the delay requirement. Flows alignment requirement including the maximum acceptable duration for traffic flow alignment. Maximum acceptable time duration for traffic flow alignment is the maximum acceptable delay introduced by the operation of traffic flow alignment.

And optionally Multi-modal Service ID, PDU Set related assistance information, sequence number, end of burst, timestamp, number of associated flows are provided in the request. Multi-modal Service ID is used to associate multiple data flows to one multi-modal (XRM) application.

- Upon receiving the request, the XRApp server performs an authorization check. If authorization is successful, the XRApp server executes the flow alignment. The flows needs to be aligned is detected by the VAL ID, flows description, and optionally Multi-modal Service ID.

If number of associated flows, sequence number and end of burst are provided, the number of associated flows, sequence number and end of burst are used to determine whether all the associated flows arrived. If the timestamp is provided, the flow that are sent at the specific time will be aligned.

If the maximum acceptable duration for traffic flow alignment is not provided, then the XRApp server would determine the maximum acceptable duration for traffic flow alignment based on flows transmission requirement.

- The XRApp server may trigger the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502, based on the provided PDU Set related assistance information.

- The XRApp server sends the multi-modal flows alignment response to the VAL server.

7.x.2.3	Procedure of flow alignment monitoring

- Figure 7.x.2.3-1illustrates the procedure of multi-modal flows alignment monitoring.

- Precondition:

-	The multi-modal flows alignment has been requested and triggered.

Figure 7.x.2.3-1: traffic flow alignment monitoring

XRApp client

5GC

XRApp server

1. multi-modal flows coordination monitoring request

VAL server

4. flows coordination monitoring

6. multi-modal flows coordination monitoring notification

multi-modal flows alignment

2. multi-modal flows coordination monitoring request

3. multi-modal flows coordination monitoring response

5. multi-modal flows coordination monitoring notification

- VAL server sends the multi-modal flows alignment monitoring request. VAL ID, flows description, monitoring period, and Protocol Description are provided in the request.

- Upon receiving the request, the XRApp server performs an authorization check. If authorization is successful, the XRApp server sends the multi-modal flows alignment monitoring request to the XRApp client. For the flows without timestamp in the header, XRApp server may reject the request.

- The XRApp server sends the multi-modal flows alignment monitoring response to the VAL server, indicating whether the multi-modal flows alignment monitoring request is successful.

- According to the Protocol Description, the XRApp client calculates the end to end delay difference based on the timestamp in header.

5-6.XRApp client sends the multi-modal flows alignment monitoring result to the XRApp server, then to the VAL server.

* * * Next Change * * * *

<Proposed change in revision marks>