3GPP TSG-SA WG6 Meeting #59	S6-240159

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0114 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS service area handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the LS S6-231606, SA2 has agreed two CRs related to local MBS services and MBS service area update (S2-2311901, S2-2311276, S2-2311705, S2-2311285). - the MC service server will request a sub-MBS service area each to the supported MB-SMF to cover the required MBS service area (at TMGI allocation/ MBS session creation without TMGI). For that the location dependent MBS services should be followed according to the agreed CR mentioned earlier. - if at a session update, the MC service server receives a reduced MBS service area instead of the requested one, the MC service server needs to create a new MBS session towards the remaining MBS service area, or may use unicast PDU session. The NEF/MBSF discovers the corresponding/ supporting MB-SMF using NRF or based on local configuration based on the MBS service area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - add a note in clause 4.7.1 referring to the local services according to clause 6.2 of 3GPP TS 23.247. - refer to location dependent aspects in clause 7.3.3.1.1 related to MBS session creation. - add text in clause 7.3.3.2.1 related to the case when the requested MBS session spreads over multiple MB-SMF service areas. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The behaviour of MC service server is not clear when it comes to local services and MBS service area is not supported by the MB-SMF service area of the requested MB-SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1, 7.3.3.1.1, 7.3.3.2.1, and 3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.7.1	General

The main purpose of 5G Multicast-Broadcast Service (MBS) use by mission critical services is to provide efficient downlink delivery of user traffic in group calls and communications. The architectural figures in this clause are aligned  with the 5GS architecture for MBS shown in Figure 5.1-2 of 3GPP TS 23.247 [15], which identifies both mandatory and optional functional entities and interfaces, in reference point representation, available for use by the MC services.

Multicast and broadcast communication services in 5G for MC group communications rely on the creation and establishment of MBS sessions to deliver user data in downlink. Shared and individual delivery from the MC service server to multiple MC users (i.e., users affiliated to a certain MC group) is supported either as point-to-point or point-to-multipoint over the radio. The MBS sessions are either broadcast or multicast type and consist of one or multiple QoS flows for different service requirements. For the broadcast MBS session or local MBS session, the MBS service area is configured with the MBS session.

NOTE 1:	Support of MBS and specific session types is an implementation choice.

NOTE 2:	Aspects related to MC services over local MBS sessions are considered according to clause 6.2 of 3GPP TS 23.247 [15].

Within this arrangement, the MC service server decides whether to create broadcast or multicast MBS sessions to be associated with certain MC groups. The 5GC adaptively decides whether to deliver the MBS traffic from the MB-UPF in the form of shared delivery or individual delivery, where the latter is applicable to multicast MBS sessions only. The NG-RAN decides to utilize point-to-point or point-to-multipoint delivery methods applicable for shared delivery only. MBS provides reliability enhancements and minimizes loss of information, e.g., due to mobility and handover.

MBS group scheduling mechanism enables simultaneous reception of MBS and unicast user traffic by the MC service UEs. The UEs can receive broadcast MBS sessions irrespective of their RRC state (i.e., connected, inactive or idle) and multicast sessions only in RRC-CONNECTED and RRC-INACTIVE state.

The following capabilities (non-exhaustive list) provided by MBS could be used by MC service servers as described in 3GPP TS 23.247 [15]:

-	MBS session creation;

-	MBS session update;

-	MBS session release;

-	MBS session ID allocation;

-	Transparent MBS Data forwarding;

-	Dynamic PCC control for MBS session;

-	UE's MBS assistance information provision.

# * * * * Second change * * * *

##### 7.3.3.1.1	General

The procedures in this clause describe how MBS session creation and MBS session announcement can be used for the transmission of MC service group communication data over either broadcast or multicast MBS sessions. The MBS session can either be created with or without dynamic PCC rule, where the latter requires less interaction done by the MC service server towards the 5GC (either directly or via NEF).

For local MBS sessions, the mechanism of location dependent services is assumed to be applied according to clause 6.2 and clause 7.3.4 of 3GPP TS 23.247 [15].

# * * * * Third change * * * *

##### 7.3.3.2.1	General

The MC service server can create one or several MBS sessions based on certain service requirements (e.g., QoS profile), a certain service area, or the activity status of multicast MBS sessions. However, during the life cycle of the MBS sessions, the MC service server may need to update the sessions to meet emerging needs, including the service requirements, service area related parameters, etc…, as defined in 3GPP TS 23.247 [15].

In case of dynamic PCC rule, the MC service server needs to determine what aspects are needed to be updated (either service area related aspects/ multicast activation status, service requirements aspects, or both as described in 3GPP TS 23.247 [15]) in order to interact with the required entity in the 5GC either directly, (in case the MC service server is in trusted domain), or indirectly via NEF/MBSF.

If the MC service server requests to update an MBS service area such that it is not supported by the MB-SMF service area of the requested MB-SMF, the MC service server receives a reduced MBS service area, which is supported by the requested MB-SMF, and it can create a new MBS session towards the corresponding MB-SMF (either directly or via NEF/MBSF) which supports the remaining MB service area. If the latter is not possible, it can use unicast PDU sessions.

# * * * * Fourth change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the following terms given in 3GPP TS 23.280 [3] apply:

MC service

MC service user

MC service UE

MC system

MC user

For the purposes of the present document, the following terms given in 3GPP TS 23.247 [15] apply:

MBS session

Broadcast MBS session

Multicast communication service

Multicast MBS session

Broadcast communication service

MBS service area

MB-SMF service area

# * * * * End of changes * * * *